| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0693 |
| --- | --- |

| Title: | AHG12: Summary of HLS proposals on subpictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution provides a topic-wise summary of contributions related to:

- General aspects of subpictures

- Subpicture layout and ID signalling

- Subpicture boundary filtering

The purpose of the summary is to help structure and improve efficiency of the discussion of these contributions.

This version of the contribution does not cover contributions related to:

- subpicture-level random access and merging

- subpicture wraparound, padding, and cropping

- subpicture conformance definition and signalling

# General design decisions

- A subpicture consists of one or more complete rectangular slices within a picture. Subpictures cannot be used with raster-scan-order slices. (several contributions)

- Tile rows and columns at the picture level apply across the entire picture. (P0245)

- SPS/PPS parameters must be the same for all the subpictures in a picture. (P0245)

  - Supposedly this means that the same slice_pic_parameter_set_id value is referenced by all the VCL NAL units of the same picture

- APS parameters must be the same for all the subpictures in a picture. (P0245)

- All of the subpictures in the aggregate picture must use a common DPB with common DPB management (P0245)

- Properties shared across each subpicture (P0245):

  - Same frame rate

  - Same reference picture lists

  - DPB can only be flushed at the same picture

  - Same temporal layer

  - Same reference picture resampling (RPR) scale factor

  - Same collocated reference picture

  - Delta-POCs to all reference pictures must be the same

- Can subpictures, if extractable, have a different random access properties from the whole picture?

  - Yes (P0139)

  - Remark: "random access properties" is not clear. This question probably relates to allowing subpictures from IRAP and non-IRAP pictures to be merged to a single picture.

# On subpicture layout

## Container structure for signalling subpicture layout

- SPS

  - When signalled in the SPS, the signalling cannot use slices, tiles, or bricks for indicating subpicture boundaries

- PPS

  - When signalled in the PPS, the signalling can use slices, tiles, or bricks for indicating subpicture boundaries

  - Note: There can be semantic constraints to keep the subpicture layout unchanged within a C(L)VS. This is discussed below.

## Subpicture layout signalling in SPS

- Using a subpicture grid approach

  - Modifications of the subpicture grid design on VVC Draft 6

    - Indicating the grid in units of 8 luma samples (P0142)

    - Indicating the grid in units of CTBs (P0142, P0377, P0432)

    - replace max_subpics_minus1 with max_subpics_minus2 (P0129, P0432)

    - single_subpic_per_grid_flag to avoid signaling grid index for each subpicture when the grid is at the subpicture granularity (P0129)

    - Asserted bug fix: P0464 proposes modifications to the existing semantic derivations of the subpicture height and width dimensions i.e., SubPicHeight and SubPicWidth. In the current working draft (WD 6) of the Versatile Video Coding (VVC), the semantic derivations for SubPicHeight and SubPicWidth result in incorrect values when the subpicture grid index (subpic_grid_idx) present in the last row (i.e., subpic_grid_idx[row][col])  is different from the grid index present directly above (i.e., subpic_grid_idx[row-1][col]) it and alternatively, when the subpicture grid index present in the last column (i.e., subpic_grid_idx[row][col]) is different from the subpicture grid index present in front of it (i.e., subpic_grid_idx[row][col-1]).

  - Using the grid in units of CTBs with the signalling of the bottom-right delta grid index (P0139, P0143)

  - Having uniform and non-uniform subpicture grid signalling modes (similar to the signalling modes for tiles) (P0471)

- Using units of CTBs, indicating top-left and bottom-right coordinates (P0171)

Constraints:

  - It is required that sub-pictures cannot be overlapped with each other, and all the sub-pictures must cover the whole picture. (P0377)

  - Add a requirement of bitsteam conformance that the following constraints apply: NumSubPicGridCols and NumSubPicGridRows cannot be concurenttly equal to 1 if subpics_present_flag is equal to 1. NumSubPics shall be equal to 0 if subpics_present_flag is equal to 0. (P0432)

## Subpicture layout signalling in PPS

- Slice based:

- Loop over subpictures, each loop entry listing slice IDs belonging to the subpicture (P0130)

- Loop over slices, each loop entry providing the subpicture ID to which the slice belongs (P0144, P0480)

- Loop over slices, each loop entry providing the subpicture index to which the slice belongs (P0171)

- Loop over subpictures, each loop entry providing subpicture boundaries using the bottom-right slice index (P0126)

- Associate slices to subpictures in one of the loops across the slices of a picture in the PPS (P0242) – details missing, this might belong to one or more of the bullets above

- Tile-based: Loop over tiles, each loop entry providing the subpicture ID to which the tile belongs (P0171).

  - This has the consequence that a subpicture contains one or more complete slices and one or more complete tiles.

- Brick based:

- Loop over subpictures, each loop entry providing subpicture boundaries using the bottom-right brick index (P0126)

- Loop over bricks, each loop entry providing the subpicture ID to which the brick belongs (P0171)

## Constraints on subpicture layout

Note: These constraints are beyond those imposed by the container structure. E.g. a syntax element in an SPS cannot change within a CLVS.

- Subpicture sizes and positions are required to be unchanged within a CLVS (P0126).

- Subpictures sizes are static in the CVS (P0480)

- P0480: It is a requirement of bitstream conformance that when sps_num_sub_pics_minus1 is greater than 0, the values of the tiling syntax elements [Ed. RS: list according to meeting decision] are the same in all PPS that are active within a CVS.

  - Note: In this contribution, the number of subpictures is signalled in the SPS, all other layout information in the PPS.

## single_slice_per_subpic_flag

- In SPS as an indication only (not affecting decoding) (P0579).

- A constraint flag general_constraint_info( ) of the PTL structure: single_slice_subpic_only_constraint_flag (P0579)

- In PPS, used for inferring the derivation of subpicture_id when they are not explicitly present (P0579)

- In PPS, used for skipping subpicture boundary signalling (based on bottom-right brick indices) or slice boundary signalling (based on bottom-right slice indices) (P0126)

# Slice header design

## Subpicture ID

- Subpicture ID is included in the slice header (P0126, P0431, …)

  - Present, when:

    - subpics_present_flag equal to 1 (P0126, P0579, P0609)

    - subpics_present_flag equal to 1 and NumSubPics greater than 0 (P0431)

    - rect_slice_flag equal to 1 (P0480)

- subpic ID is proposed in NAL unit header or slice header. First, a flag use_nuh_layer_id_as_subpic_id_flag is signaled in SPS. When use_nuh_layer_id_as_subpic_id_flag is equal to 1, subpicID is set equal to nuh_layer_id signaled in NAL unit header. When use_nuh_layer_id_as_subpic_id_flag is equal to 0, subpicID is set equal to subpic_id, which may be signaled in slice header (P0224)

- Define a subpicture index for each slice.  This could be a fixed length code (FLC) at the top of the slice header for easy bit stream manipulation. (P0242)

  - Details missing from the contribution.

## Semantics of slice address (when subpictures are in use)

- Slice_address is defined as the slice index within the subpicture. (P0126, P0609)

- The slice address is the subPic brick ID (P0480).

  - subPic brick ID is the brick index within the subpicture, where bricks are indexed in their decoding order – to be confirmed with the proponent

## Other syntax elements

- first_slice_in_sub_pic_flag (P0480)

# Mapping of subpicture IDs to the subpicture layout

## May subpicture IDs change within a CVS?

- Yes (e.g. P0139)

- Yes, but it can be signalled if they stay unchanged (P0126)

  - It is indicated with a flag (in PPS) whether the mapping of subpicture IDs to subpicture indices remains unchanged within a CLVS or may change within a CLVS.

- No (those contributions proposing to signal the mapping in SPS, e.g. P0579, P0609)

## Container structure for the mapping of subpicture IDs

- A mapping indicated in the picture header NAL unit (P0095)

- A mapping of subpicture IDs to subpicture indices is signalled in the PPS

  - Loop over the number of subpictures in a picture, each loop entry indicating the subpicture ID (P0126, P0141, P0431, P0579)

  - Loop over slices, each loop entry listing the subpicture ID to which the slice belongs (P0144)

- When subpicture IDs do not change in the CVS, a mapping of subpicture IDs to subpicture indices is signalled in the SPS (P0141)

- A mapping of subpicture IDs to subpicture indices is signalled in the SPS, loop over subpictures, each loop entry indicating the subpicture ID (P0579, P0609)

## On explicitly signalled slice IDs

- The explicitly signalled slice IDs are removed from the PPS and the slice. (P0126, P0609)

# On flags controlling the behaviour of subpicture boundaries

- subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] moved to PPS (P0126, P0480, …)

- P0145 and P0246: Disallow loop_filter_across_subpic_enabled_flag[ i ] equal to 1 when subpic_treated_as_pic_flag[ i ] is equal to 1, by either of the following:

  - Make the presence loop_filter_across_subpic_enabled_flag[ i ] conditional on subpic_treated_as_pic_flag[ i ] equal to 0. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 0.

| if( !subpic_treated_as_pic_flag[ i ] ) |  |
| --- | --- |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |

  - Semantic constraint.

- May the value of the flags change within a CL(V)S?

  - For each subpicture index i, subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] are required to be unchanged within a CLVS (P0126) / CVS (P0480).

  - Related to subpicture-based adaptive resolution change: The constraints on having the same subpicture layout and the same values of subpic_treated_as_pic_flag[ i ] and loop_filter_across_subpic_enabled_flag[ i ] throughout a CLVS are not imposed. Instead, when a reference picture is used for prediction, it is required that it has a subpicture with the same ID as the subpicture ID of the current slice.

# Miscellaneous

- max_subpics_minus1 is changed to max_subpics_minus2 and added in the VPS. Adding num_subpics_minus2 in the SPS. (P0140)

- A bug fix: When single_tile_in_pic_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to 0. (P0144)

- A bug fix (P0579):

| for( i = 0; i <= NumSubPics; i++ ) { |  |
| --- | --- |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |

# Summarized contributions

## General aspects

| JVET-P0139 | AHG17/AHG12: General questions on subpicture designs for discussion | L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) |
| --- | --- | --- |
| JVET-P0242 | AHG12/AHG16/AHG17: Subpicture Support | W. Wan, T. Hellman, M. Zhou, B. Heng, P. Chen (Broadcom) |
| JVET-P0245 | AHG12/AHG17: Subpicture Properties | W. Wan, T. Hellman, B. Heng (Broadcom) |

## Subpicture layout and ID signalling

| JVET-P0095 | AHG12/AHG17: On signalling of picture-specific syntax elements in access unit delimiter | M. M. Hannuksela (Nokia) |
| --- | --- | --- |
| JVET-P0126 | AHG12: Signalling of subpicture IDs and layout | M. M. Hannuksela (Nokia), Y.-K. Wang, Hendry (Futurewei) |
| JVET-P0129 | AHG12: On subpicture grid syntax | Y. He, A. Hamza (InterDigital) |
| JVET-P0130 | AHG12: On associating slices to subpictures | Y. He, A. Hamza (InterDigital) |
| JVET-P0140 | AHG17/AHG12: Some comments on the VVC text | L. Chen, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) |
| JVET-P0141 | AHG17/AHG12: On signalling the subpicture IDs | L. Chen, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) |
| JVET-P0142 | AHG17/AHG12: Comments on the subpicture grid in the SPS | L. Chen, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) |
| JVET-P0143 | AHG17/AHG12: On signalling of subpicture structure in the SPS | L. Chen, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) |
| JVET-P0144 | AHG17/AHG12: On associating slices with a subpicture | L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) |
| JVET-P0171 | AHG12: Modification for subpicture | J. Li, K. Abe, V. Drugeon (Panasonic) |
| JVET-P0224 | AHG8/AHG12: Signaling subpicture identifier | B. Choi, S. Wenger, S. Liu (Tencent) |
| JVET-P0377 | AHG12: Cleanups on syntax design of sub-pictures | K. Zhang, L. Zhang, H. Liu, Z. Deng, J. Xu, N. Zhang (Bytedance) |
| JVET-P0403 | AHG8/AHG12: Subpicture-specific reference picture resampling | M. M. Hannuksela, A. Aminlou, K. Kammachi-Sreedhar (Nokia) |
| JVET-P0431 | AHG12: On subpicture ID | Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm) |
| JVET-P0432 | AHG12: On signaling and grid size of subpictures | Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz (Qualcomm) |
| JVET-P0464 | AHG12 Comments on Subpicture Semantic Derivations | S. Paluri, J. Zhao, S. Kim (LGE) |
| JVET-P0471 | AHG12 Explicit Signalling of Uniform and Non-Uniform Subpicture Grid in the SPS | S. Paluri, J. Zhao, S. Kim (LGE) |
| JVET-P0480 | AHG17: On simplification of subpicture design | R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI) |
| JVET-P0579 | AHG12: On Subpicture Information Signalling | S. Deshpande, J. Samuelsson (Sharp) |
| JVET-P0609 | AHG12: On slice address signaling | R. SjÃ¶berg, M. Damghanian, M. Pettersson (Ericsson) |

Notes:

- Only item 4 of P0095 is covered in this summary.

- Only items 1 and 2 of P0140 are covered in this summary.

- Only item 2a of P0403 is covered in this summary.

- P0480 is covered in this summary excluding item 4 "Relax the requirement on AUD".

## Subpicture boundary filtering

| JVET-P0145 | AHG17/AHG12: On the flags for processing with subpictures | L. Chen, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek) |
| --- | --- | --- |
| JVET-P0246 | AHG17/Non-CE5: on loop filter processing for subpicture treated as a picture | H. Jang, S. Paluri, J. Nam, S. Kim, J. Lim (LGE), |

Note: P0246 is covered in this summary excluding the proposed decoding process changes.