| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019; Document: JVET-P0703-v1. | Document: JVET-P0698 |
| --- | --- |

| Title: | Cross-check of JVET-P0209: Non-CE4: On the affine AMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Haiwei Sun Jingya Li | Tel: Email: | +65-6550-5423 haiwei.sun@sg.panasonic.com jingya.li@sg.panasonic.com |  |
| Source: |  |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-P0209 test 1, test 2 and test 3 on simplifying the affine AMVR flag coding and on extending affine AMVR to 4 precisions. It is reported that the results matched those provided in JVET-P0209. Software implementation matches the proposal of the contribution.

# Introduction

In JVET-P0209, below three tests are performed.

Test 1: to only check the MVDs of the first CPMV during amvr_flag signaling in affine AMVR mode.

Test 2: to extend affine AMVR to include ½ luma-sample precision.

Test 3: to modify AmvrShift on top of Test 2 to limit the worst-case code length for abs_mvd_minus2.

# Simulation results

BD-Rate results of the tests on top of the VTM-6.0 compared to anchor with RA configuration on Class B, C, D and F are shown in following tables.

Table 1: Results of Test 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.07% | 0.02% | 99% | 98% |
| Class C | 0.00% | 0.11% | 0.05% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.01% | -0.02% | 0.05% | 100% | 98% |
| Class F | 0.00% | 0.02% | -0.04% | 101% | 98% |

Table 2: Results of Test 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.05% | -0.09% | 101% | 98% |
| Class C | -0.05% | 0.02% | 0.04% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.05% | 0.12% | 0.01% | 102% | 98% |
| Class F | -0.04% | -0.07% | -0.08% | 101% | 98% |

Table 3: Results of Test 3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | 0.00% | -0.03% | 101% | 99% |
| Class C | -0.04% | 0.00% | -0.11% | 103% | 101% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.04% | 0.00% | 0.03% | 100% | 97% |
| Class F | -0.03% | -0.05% | -0.01% | 101% | 99% |