| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0702-r1 |
| --- | --- |

| Title: | Non-CE6 / Non-CE3: Combined Test of JVET-P0352 and JVET-P0354 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam NASER Tangi POIRIER Fabrice LE LEANNEC Franck GALPIN | Tel: Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution provides the results of combining JVET-P0352 and JVET-P0352. When the maximum transform size is set to 32 (sps_max_luma_transform_size_64_flag  is zero), JVET-P0352 proposes to allow MIP for CU’s up to 64x64 by TU tiling and then performing MIP. Similarly, JVET-P0352 proposes allowing LFNST for CU’s up to 64x64 by TU tiling and performing LFNST on each of the tile. The combination of the two tools is reported to provide 0.06% luma gain in AI condition when sps_max_luma_transform_size_64_flag  is zero in both test and anchor. It is also reported that the coding gain in luma of class A1 is 0.20% and 0.42% in AI and RA conditions.

# Introduction

In VVC draft 6 specification, both MIP and LFNST are coded when both CU width and height are less than or equal to the maximum transform size. The maximum transform size (MaxTbSizeY) depends on the SPS flag sps_max_luma_transform_size_64_flag  as followes:

MaxTbLog2SizeY = sps_max_luma_transform_size_64_flag  ?  6  :  5	(7-28)

MaxTbSizeY = 1  <<  MaxTbLog2SizeY		(7-30)

JVET-P0352 proposes enabling MIP up to 64x64 even if the maximum transform size is 32x32, where TU tiling is used if CU size is larger than TU size. Similarly, JVET-P0352 proposes enabling LFNST up to 64x64 even if the maximum transform size is 32x32, where TU tiling is used if CU size is larger than TU size. This is to allow more flexibility to the encoder and to improve the coding efficiency when the maximum transform size is set to 32x32.

# Proposal

The proposal allows MIP and LFNST for CU sizes up to 64x64. When any of the CU’s dimensions exceeds 32, TU tiling is performed. In other word, MIP and LFSNT are performed on smaller CU’s after TU tiling.

The corresponding specification change is:

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA? 1 : 0 |  |
| if( slice_type != I ¦ ¦ sps_ibc_enabled_flag ¦ ¦ sps_palette_enabled_flag) { |  |
| if( treeType != DUAL_TREE_CHROMA && !( ( ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ modeType = = MODE_TYPE_INTRA ) && !sps_ibc_enabled_flag ) ) |  |
| cu_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( cu_skip_flag[ x0 ][ y0 ] = = 0 && slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && modeType = = MODE_TYPE_ALL ) |  |
| pred_mode_flag | ae(v) |
| if( ( ( slice_type = = I && cu_skip_flag[ x0 ][ y0 ] = =0 ) ¦ ¦ ( slice_type != I && ( CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 && cu_skip_flag[ x0 ][ y0 ] = = 0 ) ) ) ) && cbWidth <= 64 && cbHeight <= 64 && modeType != MODE_TYPE_INTER && sps_ibc_enabled_flag && treeType != DUAL_TREE_CHROMA ) |  |
| pred_mode_ibc_flag | ae(v) |
| if( ( ( ( slice_type = = I ¦ ¦ ( cbWidth = = 4 && cbHeight = = 4 ) ¦ ¦ sps_ibc_enabled_flag ) && CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) ¦ ¦ ( slice_type != I && !( cbWidth = = 4 && cbHeight = = 4 ) && !sps_ibc_enabled_flag && CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) ) && sps_palette_enabled_flag && cbWidth <= 64 && cbHeight <= 64 && && cu_skip_flag[ x0 ][ y0 ] = = 0 && modeType != MODE_INTER ) |  |
| pred_mode_plt_flag | ae(v) |
| } |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_PLT ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( pred_mode_plt_flag ) { |  |
| if( treeType = = DUAL_TREE_LUMA ) |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 1 ) |  |
| else /* SINGLE_TREE */ |  |
| palette_coding( x0, y0, cbWidth, cbHeight, 0, 3 ) |  |
| } else { |  |
| if( sps_bdpcm_enabled_flag && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize ) |  |
| intra_bdpcm_flag | ae(v) |
| if( intra_bdpcm_flag ) |  |
| intra_bdpcm_dir_flag | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag && ( Abs( Log2( cbWidth ) − Log2( cbHeight ) ) <= 2 ) && cbWidth <= MaxTbSizeY 64 && cbHeight <= MaxTbSizeY 64) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) |  |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| else { |  |
| if( sps_mrl_enabled_flag && ( ( y0 % CtbSizeY ) > 0 ) ) |  |
| intra_luma_ref_idx[ x0 ][ y0 ] | ae(v) |
| if ( sps_isp_enabled_flag && intra_luma_ref_idx[ x0 ][ y0 ] = = 0 && ( cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) && ( cbWidth * cbHeight > MinTbSizeY * MinTbSizeY ) ) |  |
| intra_subpartitions_mode_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_subpartitions_mode_flag[ x0 ][ y0 ] = = 1 ) |  |
| intra_subpartitions_split_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_mpm_flag[ x0 ][ y0 ] ) { |  |
| if( intra_luma_ref_idx[ x0 ][ y0 ] = = 0 ) |  |
| intra_luma_not_planar_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_luma_not_planar_flag[ x0 ][ y0 ] ) |  |
| intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } else |  |
| intra_luma_mpm_remainder[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| MTS_notDCT2 = 0 |  |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : cbWidth |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : cbHeight |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && IntraSubPartitionsSplitType = = ISP_NO_SPLIT && ( !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && tu_mts_idx[ x0 ][ y0 ] = = 0 && Max( cbWidth, cbHeight ) <= MaxTbSizeY 64) { |  |
| if( LfnstDcOnly = = 0 && MTS_notDCT2 == 0 && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx[ x0 ][ y0 ] | ae(v) |
| … |  |

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| if( sps_joint_cbcr_enabled_flag && ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( tu_cbf_cb[ x0 ][ y0 ] ¦ ¦ tu_cbf_cr[ x0 ][ y0 ] ) ) ¦ ¦ ( tu_cbf_cb[ x0 ][ y0 ] && tu_cbf_cr[ x0 ][ y0 ] ) ) ) |  |
| tu_joint_cbcr_residual_flag[ x0 ][ y0 ] | ae(v) |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| if ( tu_mts_idx[x0][y] > 0) |  |
| MTS_notDCT2 = 1 |  |
| } |  |
| } |  |

# Experimental Results

The proposed method was implemented on top of VTM6.0. The tests were conducted under the CTC. Both anchor and test use maximum transform size of 32x32 (--Log2MaxTbSize=5). The results are shown in the table below:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 --Log2MaxTrSize=5 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.18% | -0.16% | 103% | 99% |
| Class A2 | -0.04% | -0.03% | -0.04% | 102% | 101% |
| Class B | -0.03% | 0.02% | 0.03% | 99% | 95% |
| Class C | 0.00% | -0.01% | -0.06% | 100% | 103% |
| Class E | -0.09% | -0.03% | -0.14% | 100% | 101% |
| Overall | -0.06% | -0.04% | -0.06% | 101% | 99% |
| Class D | -0.01% | -0.05% | 0.02% | 102% | 106% |
| Class F | -0.01% | -0.08% | -0.01% | 99% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 --Log2MaxTrSize=5 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.63% | -0.86% | 103% | 98% |
| Class A2 | -0.07% | 0.08% | -0.11% | 101% | 100% |
| Class B | -0.05% | -0.04% | -0.08% | 100% | 99% |
| Class C | 0.01% | 0.10% | 0.03% | 101% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.10% | -0.21% | 101% | 99% |
| Class D | 0.01% | 0.06% | 0.12% | 100% | 100% |
| Class F | -0.03% | 0.11% | -0.03% | 101% | 98% |

# Conclusion

This contribution proposes enabling MIP and LFNST up to 64x64 regardless of the maximum transform size, where TU tiling is performed for larger CU’s. It is asserted that this approach does not require any additional process to the current VTM design, and it improves the coding performance.

# References

- X. Zhao, X. Li, S. Liu, “Non-CE6: Configurable maximum transform size in VVC”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET- O0545, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).