| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P730-v2 |
| --- | --- |

| Title: | AHG15: default quantization matrix |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe de Lagrange, Edouard François | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

It is proposed to replace the all-16 default quantization matrix with an 8..64 linear ramp along the diagonal. Combined with JVET-P0110, this is said to reduce the coding cost of custom QM by allowing first-order approximation through scale and offset, and provide a reasonable default as-is.

# Introduction

Quantization matrices (QMs) were introduced in VVC during the 14th JVET meeting, based on contribution JVET-N0847, which proposed a direct adaptation of the support of QMs available in HEVC. However, the default quantization matrices have been reduced to a single, all-16 one., which is equivalent to QM bypass.

More relevant default QMs may be desirable, e.g. optimized for an average use case, which could be better than all-flat and good enough for an average user. However, it may be difficult to agree on the appropriate values. Different standards have defined different default QMs with different shapes, e.g. concave for MPEG-2, mostly plane for AVC, convex for HEVC. Interestingly, the standard that defined QMs which found the widest acceptance seems to be JPEG, though not as default QMs but recommended in an annex: the user is still required to signal them explicitly.

Figure 1. Example standard quantization matrices. Left to right, top to bottom solid/grid:
jpeg luma/chroma, mpeg2 intra/inter, avc intra/inter, hevc intra/inter,

Another approach could be to propose default QMs that improve coding efficiency of custom QMs, or give more freedom than an all-flat one.

JVET-P0110 enables derivation of scaled and offset versions of default QMs: it proposes scaled prediction and a custom number of dpcm-coded residual coefficients; the first residual coefficient is indeed a global offset. However, scaling an all-flat matrix is redundant with offset.

A non-flat default QM would then add a degree of freedom to prediction from default QM.

As QM values generally increase along the diagonal, and are mostly symmetrical, it is suggested to define a default QM which is a linear ramp along this diagonal. This would enable a first-order approximation of custom QMs, which has a better chance to reduce the residual error than a zero-order one (the case of all-flat default QM).

# Proposal description

Since the scale factor defined by JVET-P0110 is limited to 0..2 range by 1/16 steps, the ramp used as a default QM should span enough values while preserving scale granularity. Commonly found extremes are in the 6-100 range. A default QM range of 8-64 seems reasonable, as it can reach 1-112 range when scaled by 2 (and clipped).

It is proposed to define a default QM of size 8x8 with the following formula:

defaultQM = 8 + 4*(x+y), with x and y in the 0..7 range	(3)

This is illustrated in the following figure, with the solid shape representing the proposed default QM, and the grids representing the extreme scaled versions in the JVET-P0110 context.

Figure 2: proposed default QM (solid) and scaled extremes (grids)

# Example application

The following figures show example prediction with proposed default matrix, if a scale factor and offset is available as part of prediction process. The example intra and inter QMs are taken from a field capture of an UHD HEVC broadcast.

Figure 3. Example prediction with default matrix for intra QM.
Left: default matrix (solid), target intra matrix (grid)
Right (solid): prediction with scale = 8/16 and offset = -2

Figure 4. Example prediction with default matrix for intra QM.
Left: default matrix (solid), target intra matrix (grid)
Right (solid): prediction with scale = 8/16 and offset = -2

# Excel sheets and graph pictures are attached to show bit count difference over a default flat matrix, using the testset of JVET-P0110 or additional two sets taken from field captures that use mostly plane QMs.

# Draft text (based on VVC draft 6)

Semantics of scaling_list_data are changed as follows:

Table 7-7 – Specification of default values of ScalingList[ 1 ][ matrixId ][ i ] with i = 0..3

| i | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| ScalingList[ 1 ][ 1,2,4,5 ][ i ] | 8 | 24 | 24 | 40 |

Table 7-8 – Specification of default values of ScalingList[ 2 ][ matrixId ][ i ] with i = 0..15

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ScalingList[ 2 ][ 0..5 ][ i ] | 8 | 16 | 16 | 24 | 24 | 24 | 32 | 32 | 32 | 32 | 40 | 40 | 40 | 48 | 48 | 56 |

Table 7-9 – Specification of default values of ScalingList[ 3..5 ][ matrixId ][ i ] with i = 0..63

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ScalingList[ 3..5 ][ 0..5 ][ i ] | 8 | 12 | 12 | 16 | 16 | 16 | 20 | 20 | 20 | 20 | 24 | 24 | 24 | 24 | 24 | 28 |
| i − 16 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[ 3..5 ][ 0..5 ][ i ] | 28 | 28 | 28 | 28 | 28 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 36 | 36 | 36 | 36 |
| i − 32 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[ 3..5 ][ 0..5 ][ i ] | 36 | 36 | 36 | 36 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 44 | 44 | 44 | 44 | 44 |
| i − 48 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[ 3..5 ][ 0..5 ][ i ] | 44 | 48 | 48 | 48 | 48 | 48 | 52 | 52 | 52 | 52 | 56 | 56 | 56 | 60 | 60 | 64 |

Table 7-10 – Specification of default values of ScalingList[ 6 ][ matrixId ][ i ] with i = 0..63

| i | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ScalingList[ 6 ][ 0, 3 ][ i ] | 8 | 12 | 12 | 16 | 16 | 16 | 20 | 20 | 20 | 20 | 24 | 24 | 24 | 24 | 24 | 28 |
| i − 16 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[ 6 ][ 0, 3 ][ i ] | 28 | 28 | 28 | 28 | 28 | 32 | 32 | 32 | 32 | 32 | 32 | 32 | 36 | 36 | 36 | 36 |
| i − 32 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[ 6 ][ 0, 3 ][ i ] | 36 | 36 | 36 | 36 | 40 | 40 | 40 | 40 | 40 | 40 | 40 | 44 | 44 | 44 | 44 | 44 |
| i − 48 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ScalingList[ 6 ][ 0, 3 ][ i ] | 44 | 48 | 48 | 48 | 48 | 48 | 52 | 52 | 52 | 52 | 56 | 56 | 56 | 60 | 60 | 64 |

# Draft text (based on JVET-P0110)

Semantics of scaling_list_data are changed as follows:

  - If scaling_list_pred_matrix_id_delta[ matrixId ] is equal to zero, the following applies to set default values:

    - refMatrixSize is set equal to 8,

    - all values of the 8 x 8 array refScalingMatrix are set to 16

refScalingMatrix =		(x-x)
	{
	{  8, 12, 16, 20, 24, 28, 32, 36 }
	{ 12, 16, 20, 24, 28, 32, 36, 40 }
	{ 16, 20, 24, 28, 32, 36, 40, 44 }
	{ 20, 24, 28, 32, 36, 40, 44, 48 }
	{ 24, 28, 32, 36, 40, 44, 48, 52 }
	{ 28, 32, 36, 40, 44, 48, 52, 56 }
	{ 32, 36, 40, 44, 48, 52, 56, 60 }
	{ 36, 40, 44, 48, 52, 56, 60, 64 }
	},

# Conclusion

Tests in the context of JVET-P0110 indicate moderate bit savings (a few percent in average) over the test set of JVET-P0110, with as much as 15% on the AVC-based example. In a test with QMs extracted from two field captures of UHD HEVC streams, replacing the all-16 default QM with the proposed one results in 30% gain, because these example use QMs that are mostly planes.

Incidentally, though certainly not optimal, the proposed values probably make some sense if used untouched as a quantization matrix; they are close to the Intra AVC default QM with 1.5x scale.

It is recommended to adopt the proposed default matrix in the next VVC draft.

# Patent rights declaration

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.