| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0739-v2 |
| --- | --- |

| Title: | CE5-related: Combination of JVET-P0165, P0556, and P0557 on simplifying CCALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Chen-Yen Lai, Ching-Yeh Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei Nan Hu, Jie Dong, Vadim Seregin, Marta Karczewicz | Email: | {jenny.lai, olena.chubach, chingyeh.chen, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com nanh@qti.qualcomm.com |
| Source: | MediaTek Inc., Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In CE5-2, the filtered result of cross-component adaptive loop filter (CCALF) is added on top of the chroma adaptive loop filter (ALF) output. This contribution provided two combinations of CCALF simplifications as follows.

- JVET-P0165 Method 1+2a + JVET-P0556 + JVET-P0557 Method 2

- JVET-P0165 Method 1+2a + JVET-P0556 + JVET-P0557 Method 3

Simulation results with chroma gains shifted to luma are summarized as follows.

The YCbCr BD-rate (Y-BD-rate*0.8 + Cb-BD-rate*0.1 + Cb-BD-rate*0.1, according to AHG13) of the CE5-2.1 CCALF is -0.85% for RA, while significant complexity increase is observed (46.7% increase of multiplications compared to VTM6.0 ALF).

Combination 1 preserves 75.7% of CE5-2.1 CCALF coding gain. (YCbCr: -0.64%)

Combination 2 preserves 75.4% of CE5-2.1 CCALF coding gain. (YCbCr: -0.64%)

It is asserted that both proposed combinations keep the worst case number of ALF operations exactly the same as that of VTM6.0 ALF, while any individual methods in JVET-P0165, JVET-P0556, and JVET-P0557 increase the worst case.

# Introduction

The concept of CCALF [1] is to use luma samples to improve chroma samples, and the corresponding filter coefficients are derived and signalled. The filter shape of CCALF is shown in Fig. 1, where one square is one luma sample, the red square is a luma position of to-be-processed chroma sample, each number represents one filter coefficient, and different squares with the same number share one filter coefficient. In total, there are 18 filter taps with 14 filter coefficients, so we need 14 multiplications per chroma sample in CCALF.

Figure 1.  The filter shape in CCALF.

In addition, CCALF coefficients used for each chroma component of a slice are stored in a buffer corresponding to a temporal sublayer for future reuse. When CCALF is enabled, a reuse flag is signalled at slice to indicate whether the coefficients from the corresponding temporal buffer are used or not for each chroma component. If the reuse flag is not enabled, CCALF coefficients from adaptation parameter set (APS) will be used.

# Proposed methods

To reduce the number of multiplications per pixel when applying CCALF and avoid the data dependence of CCALF coefficients signalling, it is proposed to combine JVET-P0165 [2], JVET-P0556 [3], and JVET-P0557 [4].

Two combinations are proposed in this contribution.

- JVET-P0165 Method 1+2a + JVET-P0556 + JVET-P0557 Method 2

- JVET-P0165 Method 1+2a + JVET-P0556 + JVET-P0557 Method 3

In JVET-P0165, it is proposed to avoid application of both CCALF and chroma ALF in Method 1 and to further align filter shape between CCALF and chroma ALF in which all coefficients of the filter can have different values in Method 2a shown in Figure 2.

Figure 2.  The filter shape and filter coefficients in CCALF proposed from JVET-P0165.

In JVET-P0556, the temporal buffers for CCALF coefficients signalling are removed. A slice can only use CCALF coefficients from APS.

In JVET-P0557, the restrictions for CCALF filter coefficients are proposed. There are four methods proposed in JVET-P0557, but only Method 2 and Method 3 are tested in this contribution since Method 4 caused larger coding efficiency drop reported in JVET-P0557. In the Method 2 of JVET-P0557, only the following values: {-128, -64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64, 128} can be used, and they are signalled as {-8, -7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7, 8} respectively before binarization in order to reduce the signalling overhead. First order exponential-Golomb (EG1) codes are used to signal the absolute values of the coefficients. Sign bits are signalled for nonzero coefficients. In the Method 3 of JVET-P0557, only the following values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64} can be used, which are signalled as {-7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7} respectively before binarization in order to reduce the signalling overhead. Fixed length codes of 3 bits are used to signal the absolute values of the coefficients. Sign bits are signalled for nonzero coefficients.

It is asserted that both proposed combinations keep the worst case number of ALF operations exactly the same as that of VTM6.0 ALF

# Simulation results

The combined tests have been implemented on VTM6.0 [5]. The simulations were performed following the JVET common test conditions (CTC) [6]. Table 1 shows simulation results of CCALF in CE5-2. Table 2 shows simulation results of JVET-P0165 Method 1+2a + JVET-P0556 + JVET-P0557 Method 2. Table 3 shows simulation results of JVET-P0165 Method 1+2a + JVET-P0556 + JVET-P0557 Method 3. The lambda and chroma QP changes (shifting chroma coding gains to luma) in CE5-2 are applied in the test. The run time data are inaccurate.

Table 1.  Results of CE5-2 CCALF

|  |  |  | AI Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.19% | 5.37% | 3.75% | 102% | 102% |
| Class A2 | -2.04% | -0.29% | -0.74% | 100% | 101% |
| Class B | -0.74% | -1.74% | 0.92% | 101% | 101% |
| Class C | -0.76% | 3.98% | 4.51% | 99% | 101% |
| Class E | -0.46% | -2.81% | 4.71% | 101% | 99% |
| Overall | -0.99% | 0.78% | 2.54% | 101% | 101% |
| Class D | -0.75% | 5.36% | 4.85% | 100% | 102% |
| Class F | -0.73% | -1.03% | -1.29% | 100% | 98% |
|  |  |  | RA Main10 |  |  |
|  |  |  | Over VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.82% | 5.25% | 4.13% | 99% | 101% |
| Class A2 | -2.05% | -1.14% | -6.18% | 97% | 102% |
| Class B | -0.54% | -6.34% | -2.25% | 100% | 99% |
| Class C | -0.78% | 5.75% | 6.57% | 101% | 97% |
| Class E |  |  |  |  |  |
| Overall | -1.16% | 0.24% | 0.59% | 99% | 100% |
| Class D | -0.64% | 6.08% | 6.42% | 98% | 97% |
| Class F | -0.92% | 1.86% | 2.34% | 100% | 97% |
|  |  |  | LB Main10 |  |  |
|  |  |  | Over VTM6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.54% | -9.26% | -6.60% | 102% | 101% |
| Class C | -0.73% | 3.86% | 5.44% | 100% | 99% |
| Class E | -0.24% | -5.62% | 7.60% | 102% | 98% |
| Overall | -0.53% | -3.98% | 0.96% | 101% | 100% |
| Class D | -0.56% | 4.90% | 7.37% | 105% | 104% |
| Class F | -0.84% | -2.82% | -0.28% | 104% | 101% |

Table 2.  Results of JVET-P0165 Method 1+2a + JVET-P0556 + JVET-P0557 Method 2 (results to be updated)

|  |  |  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Over VTM-6.0 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  | -1.27% |  | 6.65% |  | 5.78% |  | 115% |  | 101% |  |
| Class A2 |  | -2.16% |  | 2.17% |  | 0.58% |  | 117% |  | 101% |  |
| Class B |  | -0.81% |  | -0.23% |  | 2.42% |  | 120% |  | 102% |  |
| Class C |  | -0.83% |  | 5.57% |  | 6.30% |  | 122% |  | 95% |  |
| Class E |  | -0.50% |  | -0.99% |  | 5.66% |  | 120% |  | 95% |  |
| Overall |  | -1.06% |  | 2.48% |  | 4.08% |  | 119% |  | 99% |  |
| Class D |  | -0.84% |  | 5.78% |  | 6.02% |  | 124% |  | 81% |  |
| Class F |  | -0.86% |  | 0.70% |  | 0.15% |  | 122% |  | 97% |  |
|  |  |  |  |  |  | Random access Main10 |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM-6.0 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  | -1.86% |  | 6.59% |  | 5.61% |  | 108% |  | 103% |  |
| Class A2 |  | -2.12% |  | 0.55% |  | -5.28% |  | 107% |  | 105% |  |
| Class B |  | -0.54% |  | -5.09% |  | -1.48% |  | 111% |  | 93% |  |
| Class C |  | -0.81% |  | 6.75% |  | 7.56% |  | 115% |  | 109% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -1.19% |  | 1.53% |  | 1.59% |  | 111% |  | 101% |  |
| Class D |  | -0.69% |  | 5.87% |  | 6.94% |  | 113% |  | 107% |  |
| Class F |  | -1.03% |  | 2.44% |  | 2.93% |  | 113% |  | 103% |  |
|  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM-6.0 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | 0.72% |  | -34.29% |  | -11.44% |  | 8% |  | 43% |  |
| Class C |  | -0.77% |  | 6.31% |  | 8.25% |  | 111% |  | 105% |  |
| Class E |  | -0.20% |  | -5.67% |  | 6.89% |  | 104% |  | 89% |  |
| Overall |  | -0.01% |  | -13.60% |  | -0.29% |  | 37% |  | 69% |  |
| Class D |  | -0.57% |  | 6.86% |  | 8.84% |  | 114% |  | 107% |  |
| Class F |  | -0.88% |  | -1.45% |  | 0.94% |  | 111% |  | 96% |  |

Table 3.  Results of JVET-P0165 Method 1+2a + JVET-P0556 + JVET-P0557 Method 3 (results to be updated)

|  |  |  |  |  |  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Over VTM-6.0 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  | -1.27% |  | 6.63% |  | 5.78% |  | 116% |  | 103% |  |
| Class A2 |  | -2.16% |  | 2.17% |  | 0.56% |  | 118% |  | 102% |  |
| Class B |  | -0.81% |  | -0.24% |  | 2.41% |  | 120% |  | 101% |  |
| Class C |  | -0.82% |  | 5.57% |  | 6.30% |  | 122% |  | 94% |  |
| Class E |  | -0.51% |  | -1.00% |  | 5.63% |  | 120% |  | 94% |  |
| Overall |  | -1.06% |  | 2.47% |  | 4.07% |  | 120% |  | 99% |  |
| Class D |  | -0.84% |  | 5.81% |  | 6.06% |  | 126% |  | 81% |  |
| Class F |  | -0.87% |  | 0.70% |  | 0.15% |  | 121% |  | 95% |  |
|  |  |  |  |  |  | Random access Main10 |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM-6.0 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  | -1.83% |  | 6.37% |  | 5.58% |  | 110% |  | 111% |  |
| Class A2 |  | -2.12% |  | 0.56% |  | -5.30% |  | 108% |  | 113% |  |
| Class B |  | -0.54% |  | -5.16% |  | -1.52% |  | 111% |  | 107% |  |
| Class C |  | -0.80% |  | 6.87% |  | 7.58% |  | 115% |  | 109% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -1.18% |  | 1.50% |  | 1.57% |  | 111% |  | 109% |  |
| Class D |  | -0.70% |  | 6.13% |  | 7.17% |  | 113% |  | 107% |  |
| Class F |  | -1.02% |  | 2.45% |  | 2.97% |  | 113% |  | 102% |  |
|  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |
|  |  |  |  |  |  | Over VTM-6.0 |  |  |  |  |  |
|  |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | 0.27% |  | -25.24% |  | -10.65% |  | 15% |  | 52% |  |
| Class C |  | -0.78% |  | 6.39% |  | 8.41% |  | 113% |  | 106% |  |
| Class E |  | -0.20% |  | -6.36% |  | 6.78% |  | 104% |  | 91% |  |
| Overall |  | -0.20% |  | -9.98% |  | 0.06% |  | 48% |  | 76% |  |
| Class D |  | -0.58% |  | 6.98% |  | 9.41% |  | 117% |  | 111% |  |
| Class F |  | -0.76% |  | -1.37% |  | 0.26% |  | 112% |  | 95% |  |

# Conclusion

In this contribution, two combined tests are proposed. To simplify CCALF, it is proposed to reduce the number of multiplications per pixel, remove of temporal layer of CCALF filter coefficient, and further restrict the filter coefficients to some pre-defined values. It is important to keep the worst case number of VTM6.0 ALF operations unchanged, which is achieved by the two proposed combinations.

# References

- K. Misra, F. Bossen, and A. Segall, “CE5-2.1, CE5-2.2: Cross component adaptive loop filter,” Document of Joint Video Experts Team, JVET-P0080

- O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, and S.-M. Lei, “CE5-related: Simplified CCALF,” Document of Joint Video Experts Team, JVET-P0165

- N. Hu, J. Dong, V. Seregin, and M. Karczewicz, “CE5-related: Temporal buffer removal for cross component adaptive loop filter,” Document of Joint Video Experts Team, JVET-P0556

- N. Hu, J. Dong, V. Seregin, and M. Karczewicz, “CE5-related: Multiplication removal for cross component adaptive loop filter,” Document of Joint Video Experts Team, JVET-P0557

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).