| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0740-v1 |
| --- | --- |

| Title: | CE5-related: Combination of JVET-P0086 and JVET-P0161 on deblocking boundary strength fix for TPM and affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei Anand Meher Kotra, Semih Esenlik Biao Wang, Han Gao, Elena Alshina | Email: | {chia-ming.tsai, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com Anand.meher.kotra @huawei.com |
| Source: | MediaTek Inc., Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution provides a combination of JVET-P0086 and JVET-P0161 on deblocking boundary strength (bS) fix for TPM and affine mode. Two methods are proposed. In Method 1, when calculating the luma bS for a coding block (CB) coded with triangular prediction mode (TPM), only coefficient-related checks are performed for the internal transform block (TB) boundaries, and the bS is set to 1 for the CB boundaries. Method 2 is built on top of Method 1, and the motion-related checks are performed only for the edges coincided with 8x8 grids inside a CB coded with affine mode while the coefficient-related checks are kept unchanged. Objective results of the proposed methods under adaptive loop filter (ALF) on/off cases are reported as follows.

[Method 1] When calculating luma bS for a CB coded with TPM, only coefficient-related checks are performed for the internal TB boundaries, and set bS to 1 for the CB boundaries
Under ALF on:
VTM6.0-RA: {Y-BD-rate = -0.01%, EncT = 100%, DecT = 101%}
VTM6.0-LB: {Y-BD-rate = 0.0X%, EncT = 10X%, DecT = 10X%}
Under ALF off:
VTM6.0-RA: {Y-BD-rate = 0.01%, EncT = 100%, DecT = 100%}
VTM6.0-LB: {Y-BD-rate = 0.0X%, EncT = 10X%, DecT = 10X%}

[Method 2] Method 1 + motion-related checks are performed only for the edges coincided with 8x8 grids inside a CB coded with affine mode
Under ALF on:
VTM6.0-RA: {Y-BD-rate = 0.0X%, EncT = 10X%, DecT = 10X%}
VTM6.0-LB: {Y-BD-rate = 0.0X%, EncT = 10X%, DecT = 10X%}
Under ALF off:
VTM6.0-RA: {Y-BD-rate = 0.0X%, EncT = 10X%, DecT = 10X%}
VTM6.0-LB: {Y-BD-rate = 0.0X%, EncT = 10X%, DecT = 10X%}

# Introduction

JVET-P0086 [1] is proposed regarding the inconsistency issue caused by different deblocking grid basis for TB and prediction block (PB) as mentioned in JVET-P0043 [2]. JVET-P0161 [3] proposed a bS calculation fix for TPM regarding the issue mentioned in JVET-P0602 [4].

# Proposed methods

In this contribution, two methods are proposed to combine JVET-P0086 and JVET-P0161 on deblocking bS fix for TPM and affine mode.

Method 1:
When calculating the luma bS for a CB coded with TPM, only coefficient-related checks are performed for the internal TB boundaries, and the bS is set to 1 for the CB boundaries, as illustrated in Fig. 1.

Figure 1. Given a 128x128 CB coded with TPM, for the internal TB boundaries (i.e., edges marked by dash lines) only coefficient-related checks are performed, and for the CB boundaries, the bS value is set to 1.

Method 2:
Method 2 is built on top of Method 1, and the motion-related checks are performed only for the edges coincided with 8x8 grids inside a CB coded with affine mode while the coefficient-related checks are kept unchanged.

# Experimental results

The proposed methods are implemented on top of VTM6.0 [5], and the simulation is performed under common test condition (CTC) [6]. Table 1 and Table 2 show the results of Method 1 and Method 2, respectively, under ALF on/off cases [7].

Table 1.  Objective results of Method 1 (to be updated)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  | Over VTM6.0 with ALF switch off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 100% | 100% | 0.02% | 0.01% | 0.01% | 100% | 100% |
| Class A2 | 0.01% | 0.04% | -0.02% | 100% | 100% | 0.02% | -0.05% | 0.02% | 100% | 100% |
| Class B | -0.02% | -0.03% | 0.02% | 100% | 100% | 0.00% | -0.01% | -0.02% | 100% | 100% |
| Class C | 0.00% | 0.06% | 0.02% | 101% | 102% | 0.02% | 0.00% | -0.06% | 100% | 100% |
| Overall | -0.01% | 0.01% | 0.01% | 100% | 101% | 0.01% | -0.01% | -0.02% | 100% | 100% |
| Class D | 0.01% | -0.03% | -0.06% | 101% | 103% | -0.01% | 0.00% | 0.05% | 100% | 100% |
| Class F | 0.01% | 0.00% | -0.02% | 101% | 102% | 0.00% | -0.04% | 0.02% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  | Over VTM6.0 with ALF switch off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -0.01% | -0.16% | 0.03% | 100% | 100% | 0.01% | 0.04% | 0.00% | 100% | 101% |
| Class E | 0.02% | 0.32% | 0.25% | 100% | 102% | 0.13% | 0.04% | -0.02% | 100% | 101% |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.02% | 0.30% | 0.05% | 100% | 100% | 0.00% | 0.09% | -0.18% | 100% | 102% |
| Class F | 0.06% | -0.06% | 0.54% | 100% | 100% | 0.01% | 0.20% | -0.31% | 100% | 100% |

Table 2.  Objective results of Method 2 (to be updated)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM6.0 |  |  |  |  | Over VTM6.0 with ALF switch off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM6.0 |  |  |  |  | Over VTM6.0 with ALF switch off |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

# Conclusion

This contribution proposed two methods to combine JVET-P0086 and JVET-P0161 on deblocking bS fix for TPM and affine mode. It is suggested to adopt the proposed bS fix into VVC.

# References

- A. M. Kotra, S. Esenlik, H. Gao, B. Wang, and E. Alshina, “AHG16/Non-CE5: Deblocking boundary strength fix for Affine and TPM,” Document of Joint Video Experts Team, JVET-P0086.

- B. Heng and M. Zhou, “AHG16/Non-CE5: A cleanup for de-blocking in the affine and TPM mode,” Document of Joint Video Experts Team, JVET-P0043.

- C.-M. Tsai, C.-W. Hsu, Y.-W. Huang, and S.-M. Lei, “CE5-related: Deblocking considering prediction weights in BCW and TPM,” Document of Joint Video Experts Team, JVET-P0161.

- S. Iwamura, S. Nemoto, and A. Ichigaya, “[AHG18][non-CE5] Boundary strength derivation for CUs with TPM,” Document of Joint Video Experts Team, JVET-P0602.

- VTM6.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

- C.-Y. Chen and A. Norkin, “Description of Core Experiment 5 (CE5): Loop filtering,” Document of Joint Video Experts Team, JVET-O2025.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).