| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0774 |
| --- | --- |

| Title: | Crosscheck of JVET-P467, TEST2, TEST4 (AHG18: Rice parameter extension) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma 8910 University Center Dr., Suite 400, San Diego, CA 92122, USA | Email: | tsung-chuanma@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This document presents a cross-check report on “Crosscheck of JVET-P467, TEST2, TEST4 (AHG18: Rice parameter extension)” as proposed in JVET-P0467. It is confirmed that the test results are partially matched with those provided by the proponent.

# Introduction

In JVET-P0467 [1], it simplifies intra reference sample filter; one reference sample filter and two interpolation filters (Gaussian and DCT-IF) selection process.

# Experiment results

The cross-check tests were performed based on VTM-6.0 software [2] with VTM configuration files under JVET common test condition [3].

The cross-check results of JVET-P0467 test2 and test 4 are summarized in Table I and Table II. Tthe test results are partially matched with those provided by the proponent. And, the detailed results can be found in the accompanying spreadsheet.

Table I. Cross-check results of JVET-P0467 Test 2

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | TEST |  | AHG18 | TEST |  | AHG18 | TEST |  |
| Class A1 | 2.2 | 2.2 | -0.45% | 2.3 | 2.5 | XXX% |  |  |  |
| Class A2 | 1.7 | 1.8 | -0.76% | 1.9 | 2.2 | XXX% |  |  |  |
| Class B | 2.1 | 2.1 | -2.00% | 2.3 | 2.3 | -0.41% | 2.3 | 0.0 | ###### |
| Class C | 2.1 | 2.1 | -1.99% | 2.6 | 2.6 | -0.44% | 2.6 | 0.0 | ###### |
| Class D | 1.9 | 2.0 | -3.78% | 2.8 | 2.8 | -0.74% | 2.7 | 0.0 | ###### |
| Class E | 2.9 | 3.0 | -2.56% |  |  |  | 3.3 | 0.0 | ###### |
| Class F | 5.5 | 5.8 | -4.84% | 39.4 | 40.9 | -2.25% | 60.6 | 0.0 | ###### |
| TGM | 12.3 | 12.9 | -5.09% | 112.6 | 115.9 | -2.71% | 142.4 | 0.0 | ###### |
| Overall | 2.2 | 2.2 | -1.63% | 2.3 | 2.4 | XXX% | 2.6 | 0.0 | ###### |
| Enc Time[%] | 94% |  |  | 97% |  |  | #NUM! |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  | #NUM! |  |  |

Table II. Cross-check results of JVET-P0467 Test 4

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | TEST |  | AHG18 | TEST |  | AHG18 | TEST |  |
| Class A1 | 2.2 | 2.2 | -0.74% | 2.3 | 2.3 | XXX% |  |  |  |
| Class A2 | 1.7 | 1.8 | -1.10% | 1.9 | 1.9 | XXX% |  |  |  |
| Class B | 2.1 | 2.1 | -2.03% | 2.3 | 2.3 | -0.41% | 2.3 | 0.0 | ###### |
| Class C | 2.1 | 2.1 | -2.10% | 2.6 | 2.6 | -0.45% | 2.6 | 0.0 | ###### |
| Class D | 1.9 | 2.0 | -3.86% | 2.8 | 2.8 | -0.75% | 2.7 | 0.0 | ###### |
| Class E | 2.9 | 3.0 | -2.59% |  |  |  | 3.3 | 0.0 | ###### |
| Class F | 5.5 | 5.8 | -4.77% | 39.4 | 40.8 | -2.19% | 60.6 | 0.0 | ###### |
| TGM | 12.3 | 12.9 | -4.83% | 112.6 | 115.6 | -2.56% | 142.4 | 0.0 | ###### |
| Overall | 2.2 | 2.2 | -1.77% | 2.3 | 2.3 | XXX% | 2.6 | 0.0 | ###### |
| Enc Time[%] | 95% |  |  | 99% |  |  | #NUM! |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | #NUM! |  |  |

# References

- J. Choi, H. Jang, J. Heo, S. Yoo, J. Lim, S. Kim, “AHG18: Rice parameter extension”, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P0467, Oct. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-O2010, July. 2019