| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0791-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-P0535 (Non-CE3: Removal of block size restriction in MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck report |  |  |
| Author(s) or Contact(s): | Jonathan Pfaff | Email: | jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Introduction

In the current version of VVC, matrix based intra prediction (MIP) can be performed on intra blocks of shape M×N with M and N integral powers of two if the following conditions apply:

- 4≤min⁡(M,N),

- 64≥max⁡(M,N)

- M≤4*N

- N≤4*M.

The proposal crosschecked here proposes to remove conditions 3 and 4. This means that, in contrast to the current VVC, in the proposed method crosschecked, MIP can also be applied on intra blocks of shape 64×4, 4×64, 64×8, 8×64, 32×4, 4×32. In order to implement MIP on these block shapes, the part of the spec that describes the MIP prediction does not need to be changed.

# Experimental results

The crosschecker inspected the software and the text and can confirm that they incorporate the method that is described above. Also, the reported BD rates match under CTC conditions [1]. The higher encoder runtime by the crosschecker is to be attributed to some cluster inaccuracies. Some local tests by the crosschecker showed that the runtimes match. Results are reported in the two tables below. Here, according to the conditions in the core experiments on intra prediction [2], only the AI and the RA configuration were tested.

Table 1. Result of proposed method for the AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,03% | 0,00% | 0,04% | 99% | 101% |
| Class A2 | 0,00% | 0,06% | -0,04% | 101% | 100% |
| Class B | 0,00% | 0,00% | -0,03% | 104% | 100% |
| Class C | 0,00% | -0,09% | -0,01% | 102% | 100% |
| Class E | -0,01% | -0,02% | 0,03% | 100% | 103% |
| Overall | -0,01% | -0,01% | 0,00% | 101% | 101% |
| Class D | 0,01% | -0,05% | -0,11% | 101% | 100% |
| Class F | -0,01% | 0,00% | -0,05% | 101% | 102% |

Table 2. Result of proposed method for the RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class A2 | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class B | -0,02% | 0,07% | 0,04% | 104% | 101% |
| Class C | -0,01% | -0,07% | -0,17% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | #WERT! | #WERT! | #WERT! | #ZAHL! | #ZAHL! |
| Class D | -0,03% | -0,30% | -0,14% | 102% | 103% |
| Class F | 0,01% | -0,01% | 0,05% | 103% | 104% |

# References

- F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, Switzerland, March 2019

- G. Van der Auwera et al.,“Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-O2023, Gothenburg, Sweden, July 2019

# Patent rights declaration(s)

Fraunhofer HHI does not have any current or pending patent rights relating to the technology described in this contribution.