| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0797-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-P0375: Non-CE8: Palette mode CU size restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Weijia Zhu 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-360-525-0535 weijia.zhu@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This document reports crosscheck results of JVET-P0375 (Non-CE8: Palette mode CU size restriction). The results generated by us match those provided by the proponent.

# Introduction

In JVET-P0375, It is proposed to disable palette mode for inter slice CUs with sizes less than or equal to 32 samples.

# Simulation Results

Simulations were performed by using the code provided by the proponent. VTM6.1 is used as a benchmark. Simulation results are shown below. Please note that the complexity may be not reliable due to the heterogeneous clusters.

Table 1. The performance of JVET-P0375 under AI with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 99% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Animation | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 2. The performance of JVET-P0375 under RA with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.11% | 0.07% | -0.03% | 101% | 100% |
| TGM 720p | -0.03% | 0.06% | -0.03% | 101% | 100% |
| Animation | -0.51% | -0.50% | -0.45% | 101% | 99% |
| Mixed content | -0.03% | 0.05% | 0.13% | 100% | 101% |
| Camera-Captured content | 0.03% | 0.09% | 0.01% | 100% | 100% |
| Overall | -0.13% | -0.04% | -0.07% | 101% | 100% |

Table 3. The performance of JVET-P0375 under LDB with dual tree on

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.00% | 0.19% | 0.11% | 101% | 100% |
| TGM 720p | -0.08% | -0.26% | -0.17% | 100% | 100% |
| Animation | -0.51% | -0.41% | -0.40% | 101% | 100% |
| Mixed content | -0.03% | -0.02% | -0.05% | 101% | 101% |
| Camera-Captured content | 0.02% | 0.00% | -0.13% | 101% | 100% |
| Overall | -0.12% | -0.10% | -0.11% | 101% | 100% |

Table 4. The performance of JVET-P0375 under AI with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.00% | 0.00% | 0.00% | 100% | 99% |
| TGM 720p | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Animation | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Mixed content | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Camera-Captured content | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 5. The performance of JVET-P0375 under RA with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | -0.05% | 0.11% | 0.08% | 101% | 100% |
| TGM 720p | -0.04% | 0.00% | 0.00% | 101% | 100% |
| Animation | -0.52% | -0.48% | -0.48% | 101% | 100% |
| Mixed content | -0.03% | 0.07% | -0.02% | 100% | 100% |
| Camera-Captured content | -0.08% | -0.01% | -0.02% | 100% | 99% |
| Overall | -0.13% | -0.05% | -0.08% | 101% | 100% |

Table 6. The performance of JVET-P0375 under LDB with dual tree off

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| TGM 1080p | 0.13% | 0.32% | 0.23% | 101% | 100% |
| TGM 720p | 0.30% | 0.23% | 0.28% | 100% | 100% |
| Animation | -0.55% | -0.44% | -0.35% | 100% | 99% |
| Mixed content | 0.03% | 0.19% | 0.21% | 100% | 101% |
| Camera-Captured content | -0.04% | 0.02% | -0.02% | 100% | 100% |
| Overall | 0.01% | 0.09% | 0.10% | 100% | 100% |