| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0909-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-P0256 (AHG15: 16x16 base scaling matrix for 64x64 TU) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Olena Chubach, Chen-Yen Lai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 olena.chubach@mediatek.com jenny.lai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This document reports results of crosschecking a proposal to use a 16x16 base scaling matrix instead of 8x8 for 64xN and Nx64 TUs in JVET-P0256. The verification task was done, and the BD-rate results match exactly with those in JVET-P0256.

# Verification

Panasonic provided the source code of their proposed method described in JVET-P0256 [1]. This test is implemented on top of VTM6.0 with custom base quantization matrices used and is matching the method described in JVET-P0256. The anchor is VTM6.0 with scaling matrices disabled.

The results were provided only for one of the symmetric sets of user-defined scaling matrices. The purpose of testing was to prove that implementation for 16x16 base scaling matrix works for 64xN/Nx64 TU. Since the main purpose of this contribution is to update granularity of controlling QP for 64xN/Nx64 TU, such that it is kept other matrices were not checked. The overhead that is needed to code 16x16 matrix will depend on what types of scaling matrices are used.

The verification was carried out accordingly, only for one set of scaling matrices. The base scaling matrices used are based on those in JCTVC-F362 [2]. The 16x16 base scaling matrix in the contribution was created by up-sampling 8x8 matrix by duplication of its elements in horizontal and vertical directions.

The tests are conducted under the common test conditions (CTC) defined in [3] and are carried out using the VTM6.0 software [4], but scaling matrices were further enabled. Table 1 shows the results using VTM6.0 with scaling matrices disabled as the anchor and enabling scaling matrices proposed in JVET-P0256 in the test. The BD-rates of AI, RA and partial BD-rates of LB from our experimental results match exactly those in JVET-P0256. Please note that the runtime comparison may be not accurate.

Table 1. The results of test using VTM6.0 with scaling matrices disabled as anchor

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
| o | Y | U | V | EncT | DecT |
| Class A1 | 9.27% | 18.65% | 8.77% | 105% | 105% |
| Class A2 | 8.60% | 13.37% | 12.25% | 99% | 109% |
| Class B | 11.31% | 8.33% | 8.09% | 102% | 111% |
| Class C | 13.57% | 11.39% | 11.45% | 90% | 114% |
| Class E | 13.63% | 9.37% | 10.59% | 99% | 108% |
| Overall | 11.41% | 11.74% | 10.06% | 99% | 110% |
| Class D | 20.40% | 16.41% | 17.34% | 89% | 131% |
| Class F | 8.44% | 11.85% | 12.45% | 94% | 104% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 5.57% | 14.70% | 7.95% | 103% | 107% |
| Class A2 | 4.93% | 12.04% | 11.07% | 98% | 99% |
| Class B | 8.52% | 8.69% | 7.99% | 100% | 102% |
| Class C | 9.05% | 10.83% | 10.41% | 95% | 122% |
| Class E |  |  |  |  |  |
| Overall | 7.35% | 11.13% | 9.24% | 99% | 107% |
| Class D | 10.70% | 10.63% | 11.16% | 94% | 168% |
| Class F | 6.03% | 12.26% | 13.49% | 99% | 119% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 8.08% | 6.26% | 3.37% | 7% | 36% |
| Class C | 9.39% | 9.60% | 9.31% | 97% | 123% |
| Class E | 6.63% | -2.17% | -1.48% | 100% | 122% |
| Overall | 8.15% | 5.27% | 4.14% | 33% | 73% |
| Class D | 9.48% | 7.35% | 7.71% | 94% | 176% |
| Class F | 5.43% | 11.45% | 12.23% | 43% | 90% |

# References

[1] T. Toma and K. Abe, “AHG15: 16x16 base scaling matrix for 64x64 TU,” Document of Joint Video Experts Team, JVET-P0256.

[2] T. Suzuki, A. Tabatabai, M. Zhou, and V. Sze, “Proposal to support quantization matrix in HEVC,” Document of Joint Collaborative Team on Video Coding, JCTVC-F362.

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

[4] VTM6.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-6.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).