| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0968-v7 |
| --- | --- |

| Title: | Report of BoG on high-level syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | BoG report |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Futurewei, San Diego, CA, USA | Email: | +1-908-803-3888 ye-kui.wang@futurewei.com |
| Source: | BoG chair |  |  |

_____________________________

# Abstract

The HLS BoG met on the following days and time slots:

- 3 Oct. Thursday 14:00 to 21:40

- 4 Oct. Friday 09:00 to 21:35

- 5 Oct. Saturday 09:00 to 21:30

- 7 Oct. Monday 14:00 to 21:20

- 9 Oct. Wednesday 9:00 to 17:30

- Altogether, 155 HLS proposals, in the following three areas, have been reviewed by the JVET plenary meeting on Tuesday 10/1 and Wednesday 10/2, JVET Track B Thursday 10/3 morning, and the above listed HLS BoG meetings:

- 6.18 AHG17: General high-level syntax (65, excluding cross checks)

- 6.19	AHG12: high-level parallelism and coded picture regions (57)

- 6.20	AHG8: layered coding and resolution adaptivity (33)

In the list of recommendations below, the name in each instance of "SW/name" is suggested to be the person who would be responsible for coordinating the software implementation (if needed) of the recommendation if confirmed by the JVET. Currently, for each item (except one), the name is that of the first author of the first listed contribution.

Note that there is no need for identifying a person for spec text coordination for HLS adoptions.

Note that decisions (e.g., on subpicture topics) that were made earlier at JVET plenary or track meetings are not listed below. There were also some purely editorial suggestions that were noted in the HLS BoG meeting minutes to be delegated to the editors, and those are not listed here, either.

The BoG recommends the following adoptions:

- Reserve nuh_layer_id values 56 to 63, inclusive, in VVCv1 [JVET-P0362][SW/R. Sjöberg]

- Rearrange/cleanup the NAL unit types as follows [JVET-P0363][SW/M. Pettersson]:

  - Switch position of RADL and RASL in the table to align with the order in HEVC.

  - Add VPS_NUT, which is missing, and order the parameter set NUTs in hierarchical order as in HEVC, in the order of DPS_NUT, VPS_NUT, SPS_NUT, PPS_NUT, APS_NUT.

  - Group reserved non-IRAP VCL NUTs within one code range instead of two.

  - Allocate 5 codes instead of 6 for reserved non-IRAP VCL NUTs and 3 codes instead of 2 for reserved IRAP VCL NUTs.

- Change the PTL syntax and semantics as follows [JVET-P0217][SW/J. Samuelsson]:

  - Move general constraints and general_level_idc before sub-profiles.

  - Allow signalling of zero or more sub-profiles.

  - Add inference of sub-layer level.

- Allow more than one PTL syntax structure in the DPS, with the following changes: reserve 5 bits instead of 1 bit, and use u(4) for the number of PTL syntax structures. [JVET-P0478, SW/R. Skupin]

- Add the following RPL-related constraints to ensure that the inter prediction relationship among IRAP, RASL, RADL, trailing, and GDR pictures etc. function as expected [JVET-P0978][SW/M. Pettersson]:

- When the current picture is a CRA picture, there shall be no entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes, in output order or decoding order, any preceding IRAP picture in decoding order (when present).

- When the current picture is a trailing picture, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the IRAP picture associated with the current picture.

- When the current picture is a trailing picture that follows in both decoding order and output order one or more leading pictures associated with the same IRAP picture, if any, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that was generated by the decoding process for generating unavailable reference pictures for the IRAP picture associated with the current picture.

- When the current picture is a recovery point picture or a picture that follows the recovery point picture in output order, there shall be no entry in RefPicList[ 0 ] or RefPicList[ 1 ] that contains a picture that was generated by the decoding process for generating unavailable reference pictures for the GDR picture of the recovery point picture.

- When the current picture is a trailing picture, there shall be no picture referred to by an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the associated IRAP picture in output order or decoding order.

- When the current picture is a trailing picture that follows in both decoding order and output order one or more leading pictures associated with the same IRAP picture, if any, there shall be no picture referred to by an entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the associated IRAP picture in output order or decoding order

- When the current picture is a RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that is any of the following:

  - A RASL picture

  - A picture that was generated by the decoding process for generating unavailable reference pictures

  - A picture that precedes the associated IRAP picture in decoding order

And replace the requirement on exact match of GDR recovery point picture to a NOTE (exact wording delegated to editors).

- Constrain that, when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0. [JVET-P0182, JVET-P0221][SW/S. Deshpande]

- Adopt the following improvement and bug fix: [JVET-P0221, JVET-P0589][SW/B. Choi]

  - Update the value range of ilrp_idc[ listIdx ][ rplsIdx ][ i ] to be in the range of 0 to the NumDependentLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1, inclusive.

  - Fix the equation for derivation of DirectDependentLayerIdx, by changing "i < " to "i  <=  ", and "j = i" to "j = i − 1".

- Remove the following constraints: [JVET-P0122][SW/Hendry]

- For intra slices and slices in an IRAP picture, slice_alf_aps_id_luma[ i ] shall not refer to an ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

- For intra slices and slices in an IRAP picture, slice_alf_aps_id_chroma shall not refer to an ALF APS associated with other pictures rather than the picture containing the intra slices or the IRAP picture.

- Add an SEI message that contains only a flag self_contained_cvs_flag in its syntax. The flag equal to 1 indicates that parameter sets are "self-contained" in a manner that enables random access from any CVSS picture without fetching parameter sets from AUs before the CVS (this piece is the same as in HEVC), and additionally, enables sub-layer up-switching at STSA pictures without fetching parameter sets from AUs before the switching point (this piece is not in HEVC). [JVET-P0359][SW/R. Sjöberg]

- Adopt the following clarifications: [JVET-P0184][SW/S. Deshpande]

  - Clarify the process for generating unavailable reference pictures such that it is clear that the process only applies to intra-layer reference pictures.

  - Fix the use of the sps_max_dec_pic_buffering_minus1 syntax element without using an index now that it is an array/list.

- Adopt the following clean-up changes: [JVET-P0243, JVET-P0244, JVET-P0429][SW/W. Wan]

  - Signal a single bit depth to be used for both luma and chroma.

  - Use u(4) for SPS IDs in the SPS syntax as well as in other places for referring to SPS.

  - Code chroma_format_idc with u(3).

  - Code log2_max_pic_order_cnt_lsb_minus4 with u(4).

  - Specify the value range of 1.4	min_qp_prime_ts_minus4 to be 0 to 48, inclusive.

  - Specify the value range of log2_min_luma_coding_block_size_minus2 to be 0 to log2_ctu_size_minus5 + 3, inclusive.

- Add the following constraint flags into the general_constraint_info( ) syntax structure: [JVET-P0366][SW/M. Pettersson]

  - no_trail_constraint_flag

  - no_stsa_constraint_flag

  - no_rasl_constraint_flag

  - no_radl_constraint_flag

  - no_idr_constraint_flag

  - no_cra_constraint_flag

  - no_gdr_constraint_flag

  - no_aps_constraint_flag

- Add the following constraints to ensure that the referencing of ALF parameters from slice header to ALF APS works as expected: [JVET-P0438][SW/J. Chen]

  - It is requirement of bitstream conformance that the values of alf_luma_filter_signal_flag and alf_chroma_filter_signal_flag shall not both be equal to 0.

  - The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be equal to 1.

  - The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be equal to 1.

- Allow an APS NAL unit to be after the last VCL NAL unit of the associated picture, by defining two NUTs for APSs, prefix APS NUT and suffix APS NUT. [JVET-P0588, JVET-P0452][SW/V. Seregin]

- Adopt the following clean-up changes: [JVET-P0181, JVET-P0183][SW/S. Deshpande]

  - Require that bp_nal_hrd_parameters_present_flag and bp_vcl_hrd_parameters_present_flag cannot be both equal to 0, and clean-up the syntax of the BP SEI message accordingly.

  - Add inference rules for du_cpb_removal_delay_increment_length_minus1 and dpb_output_delay_du_length_minus1 if needed.

  - Add a flag to the BP SEI message syntax to specify whether the initial CPB buffering delay values are signalled for each temporal sub-layer or only for the highest temporal sub-layer.

  - Conditionally signal cpb_removal_delay_delta_enabled_flag[ i ] for i in the range of TemporalId to pt_max_sub_layers_minus1−1, inclusive, based on cpb_removal_delay_deltas_present_flag.

  - Specify the length of cpb_removal_delay_delta_idx[ i ] to be Ceil( Log2( num_cpb_removal_delay_deltas_minus1+1) ) bits.

- Adopt the following changes related to the BP, PT, and DUI SEI messages: [JVET-P0202, JVET-P0189, JVET-P0203][SW/ V. Drugeon]

  - Add a loop to the PT and the DUI SEI messages for signalling of CPB timing parameters for sub-layers in DU-based HRD operations, and remove pt_max_sub_layers_minus1 and dui_max_sub_layers_minus1 and use the corresponding syntax element in the BP SEI message.

  - Repeat decoding_unit_hrd_params_present_flag in the BP SEI message and move decoding_unit_cbp_params_in_pic_timing_sei_flag to the BP SEI message, and use these syntax elements in the BP SEI message in syntax conditions in the PT and DUI SEI messages as needed. [JVET-P0202, JVET-P0189][SW/ V. Drugeon]

- Add the following HEVC and AVC SEI messages to ITU-T H.SEI | ISO/IEC 23002-7. [JVET-P0337][SW/M. Sean]

  - User data registered by Recommendation ITU-T T.35 SEI message

  - Film grain characteristics SEI message

  - Frame packing arrangement SEI message

  - Mastering display colour volume SEI message

  - Content light level information SEI message

  - Alternative transfer characteristics information SEI message

  - Ambient viewing environment SEI message

  - Content colour volume SEI message

- Add the following HEVC and AVC SEI messages to ITU-T H.SEI | ISO/IEC 23002-7. [JVET-P0462][SW/R. Skupin]

  - Equirectangular Projection SEI message

  - Sphere rotation SEI message

  - Region-wise packing SEI message

  - Omnidirectional viewport SEI message

- Adopt a new SEI message that provides picture rate information, as proposed in JVET-P0610. Into ITU-T H.SEI | ISO/IEC 23002-7. [JVET-P0610][SW/M. Sychev]

- Adopt a new SEI message that provides shutter interval information, as proposed in JVET-P0338, but add the number of sub-layers to avoid parsing dependency on SPS. Into ITU-T H.SEI | ISO/IEC 23002-7. [JVET-P0338][SW/M. Sean]

- Adopt a new generalized cubemap projection SEI message, as proposed in JVET-P0597, but remove two lines of syntax for the byte alignment. into ITU-T H.SEI | ISO/IEC 23002-7. [JVET-P0597][SW/Y.-H. Lee]

- Adopt a new SEI message that indicates subpicture level information, with the syntax as proposed in JVET-P0984, with the following changes: [JVET-P0984][SW/R. Skupin]

  - Change the reserved 4 bits to reserved 5 bits for byte alignment.

  - Change "shall have" in NOTEs to "has".

  - Add the SPS ID.

  - Correct the wording of where the CPB size information is signalled.

- Adopt a new SEI message that provides information on SAR dynamics, with the syntax as proposed in JVET-P0450 option 2, but drop the cancel flag and the persistence flag. The persistency scope is the set of AUs starting from the current until (exclusive) the next AU associated with the same SEI message. [JVET-P0450][SW/R. Skupin]

- Fix a bug on dependency between subpictures, by fixing some equations in the clause of the derivation process for constructed affine control point motion vector merging candidates and in the clause of the luma integer sample fetching process. [JVET-P0378, JVET-P0572][SW/K. Zhang]

- Adopt the following clean-up changes: [JVET-P0228][SW/B. Choi]

  - Add "&& !vps_all_independent_layers_flag" to the syntax condition for vps_output_layers_mode, and add the inference rule "When not present, the value of vps_output_layers_mode is inferred to be equal to 1."

  - Replace "pic_output_flag" in the following sentence with "PictureOutputFlag": vps_output_layer_mode is equal to 0 and the current access unit contains a picture that has pic_output_flag equal to 1, has nuh_layer_id nuhLid greater than that of the current picture, and belongs to an output layer (i.e., OutputLayerFlag[ GeneralLayerIdx[ nuhLid ] ] is equal to 1)."

- Remove pps_temporal_mvp_enabled_idc, and add a constraint like the following: When no reference picture with the same resolution as the one of the current picture is available, it is a requirement of bitstream conformance that ph_temporal_mvp_enabled_flag shall be equal to 0. [JVET-P0206][SW/T. Nishi]

- Reserve value 3 of log2_ctu_size_minus5. [JVET-P0580][SW/Y. Du]

- As a bug fix, disallow the min CU size to be greater than Min( 64, CTU size ). [JVET-P0578][SW/T.-D. Chuang]

- Adopt the clause 8.1.1 general decoding process text proposed in JVET-P0115-v2, and additionally specify the general decoding process as follows as a means for the decoder to know which OLS is being decoded (including which layers are being output) [JVET-P0115][SW/Y.-K. Wang]:

OlsIdx and HighestTid are given by external means, add a bitstream requirement that the bitstream does not contain any other layers than included in the OLS with OlsIdx and does not include any NAL unit with TemporalId greater than HighestTid.

- Adopt the PTL signalling mechanism in JVET-P0117-v3. [JVET-P0117][SW/Y.-K. Wang]

- Adopt the HRD design in JVET-P0118-v5, as summarized and noted below: [JVET-P0118][SW/Y.-K. Wang]

  - Add vps_max_sub_layers_minus1 (this piece is also in JVET-P0185) and vps_all_layers_same_num_sub_layers_flag to the VPS.

  - On signalling of sequence-level HRD parameters, the following applies:

    - For each independent layer used as an OLS containing only the layer, the sequence-level HRD parameters are signalled in the SPS referred to by the layer. (not in JVET-P0118-v5, suggested and agreed by the HLS BoG).

    - For other cases, sequence-level HRD parameters are signalled in the VPS.

  - DPB parameters (size, reordering, latency) for independent layers when used as output layers are signalled in the SPS (for enabling single-layer bitstreams without referring to VPSs), and in the VPS for other cases. Different DPB sizes may be signalled for a layer that may be used both as an output layer (in some OLSs) and as a non-output layer (in some other OLSs).

  - Sequence-level HRD parameters syntax elements general_nal_hrd_params_present_flag, general_vcl_hrd_params_present_flag, decoding_unit_hrd_params_present_flag, tick_divisor_minus2, decoding_unit_cpb_params_in_pic_timing_sei_flag, bit_rate_scale, cpb_size_scale, cpb_size_du_scale, sub_layer_cpb_params_present_flag, num_layer_hrd_params_minus1, and bp_cpb_cnt_minus1 are signalled at most once for each multi-layer bitstream.

  - A list of OLS HRD parameter syntax structure is signalled, and an index to the list is signalled for each OLS as needed. Different OLS HRD parameter syntax strutures may include different numbers of sub-layer HRD parameter syntax structures.

  - When sub_layer_cpb_params_present_flag is equal to 0, the OLS HRD parameters for the lower sub-layer representations are inferred to be the same as the only set of OLS HRD parameters signalled in the OLS HRD parameters syntax structure for the highest sub-layer representation.

  - The sub-bistream extraction process is no longer a part of the decoding process, thus moved from clause 10 to Annex C, and technically updated to output a sub-bitstream for a target OLS, and the extraction process removes SEI NAL units that contain buffering period, picture timing, or decoding unit information SEI messages that do not apply to the target OLS.

  - Only one set of conformance tests, for testing the conformance of the OLSs specified by the VPS, is specified. Therefore, it is possible that there is no OLS specified by the VPS that included all layers of the entire bitstream, in which case the conformance of the entire bitstream is not specified.

Delegate the following to editors: Update the HRD text, particularly the part on describing the conformance tests, such that it does not appear that VPS would be needed even when the bitstream only has a single layer.

  - Similarly as in MVC, CPB operations are OLS-specific, i.e., there is only one CPB for the entire OLS bitstream. However, the DPB is specified to conceptually consist of a sub-DPB for each layer, and the DPB operates in a layer-specific manner.

  - In the list of bitstream conformance constraints, added that the the DPB output times derived for all pictures in any particular access unit shall be the same.

  - In the derivations of the value of the variable NoOutputOfPriorPicsFlag in the DPB operations texts, the syntax elements pic_width_in_luma_samples, pic_height_in_luma_samples are replaced by pic_width_max_in_luma_samples and pic_height_max_in_luma_samples.

- Adopt the scalable nesting SEI message in JVET-P0190-v2. [JVET-P0190][SW/Y.-K. Wang]

- Specify the following constraints and rules on values of TemporalId and nuh_layer_id of NAL units: [JVET-P0125][SW/Y.-K. Wang]

  - All pictures in an AU shall have the same value of TemporalId.

  - The TemporalId of an SEI NAL unit shall be equal to the the TemporalId of the AU containing the NAL unit.

  - An SPS NAL unit shall have nuh_layer_id equal to the lowest nuh_layer_id value of PPS NAL units that refer to it.

  - A PPS NAL unit shall have nuh_layer_id equal to the lowest nuh_layer_id value of the coded slice NAL units that refer to it.

  - Parameter sets of each type share the same parameter set ID value space, regardless of the nuh_layer_id values.

  - The nuh_layer_id of an APS NAL unit shall be equal to the lowest nuh_layer_id value of the code slice NAL units that refer to it.

  - The nuh_layer_id of an EOB NAL unit shall be equal to 0.

  - The nuh_layer_id of a filler data NAL unit shall be equal to the nuh_layer_id of associated VCL NAL unit.

- Add the set of general constraints and rules on SEI messages of the following categories in section 4 of JVET-P0125-v2 [JVET-P0125][SW/Y.-K. Wang]:

  - Two constraints on containing of SEI messages in SEI NAL units

  - The specifiation of applicable layers of non-scalable-nested SEI messages

  - Three constraints on the value of nuh_layer_id of SEI NAL units

- Add the following constraints and inference rule:

  - When sps_video_parameter_set_id is equal to 0, it is a requirement for bitstream conformance that there shall be only a single layer in the bitstream. [JVET-P0097, JVET-P0205][SW/M. M. Hannuksela]

  - Require that vps_video_parameter_set_id shall be greater than 0. [JVET-P0205][SW/V. Drugeon]

  - Infer vps_max_layers_minus1 to be equal to 0 when not present. [JVET-P0185][SW/ S. Deshpande]

- As a bug fix, remove pps_five_minus_max_num_subblock_merge_cand_plus1, such that the syntax element five_minus_max_num_subblock_merge_cand does not use the "constant slice/picture header parameters in PPS" mechanism introduced by the adoption of JVET-O0238. [JVET-P0152][SW/O. Chubach]

- Adopt the picture header design in JVET-P1006, but disallow PH repetition. [JVET-P1006][SW/W. Wan]

- Adopt the OLS signalling syntax (in the VPS) and semantics in JVET-P1019, with the addition of the following constraint: Each OLS shall have at least one output layer. [JVET-P1019][SW/B. Choi]

- If it is not clearly specified, it needs to be clearly specified that, whenever a decoded picture is output, the conformance window applied shall be the conformance window that is associated with the picture. [SW/Hendry]

- If JVET-P1004 is adopted, adopt the proposed syntax, semantics, and the derivation process for the single_slice_per_subpic_flag in JVET-P1024. Otherwise, adopt in principle to signal the indication of single slice per subpicture and use it to condition the syntax for signalling of rectangular slices when not needed. [JVET-P1024][SW/M. M. Hannuksela]

- Adopt support of mixed IRAP and non-IRAP pictures within an AU, with the following POC design:

  - For independent layers, use POC MSB cycle signalling. In the SPS, there is a flag to control whether picture header has ph_poc_msb_cycle_present_flag, and a length in the SPS. When ph_poc_msb_cycle_present_flag is equal to 1, the POC MSB cycle, u(v) coded, is signalled in the picture header. When POC MSB cycle is present, the POC MSB of the picture is set to poc_msb_cycle * MaxPicOrderCntLsb. [JVET-P0116][SW/Y.-K. Wang]

  - For dependent layers, if there is a picture picA in the same AU in a reference layer of the current layer, the POC is derived as equal to the POC of piA and POC LSB values are required to be cross-layer aligned. Otherwise, the current POC derivation process applies. [JVET-P0101][SW/M. M. Hannuksela]

- Add a constraint to require that each IRAP AU is complete (i.e., there is a picture in each layer present in the CVS) and all pictures in an IRAP AU are IRAP pictures with the same NAL unit type. [JVET-P0116][SW/Y.-K. Wang]

- Always keep VPS and EOB NAL units in the output sub-bitstream in the sub-bitsteam extraction process (unless there is a decision to handle layer ID 0 differently than other layer ID values, where layer ID 0 can only be used by NAL units that apply to all layers, e.g., as proposed in JVET-P0380). [JVET-P0098][SW/M. M. Hannuksela]

- Specify EOS NAL units to be layer specific, i.e., an EOS NAL unit applies only to the layer with the same nuh_layer_id as the EOS NAL unit (unless there is a decision to handle layer ID 0 differently than other layer ID values, where layer ID 0 can only be used by NAL units that apply to all layers, e.g., as proposed in JVET-P0380). [JVET-P0125][SW/Y.-K. Wang]

- Adopt a clarification to the bullet items in setting of PicOutputFlag in the decoding process of one coded picture involving variables derived from the VPS, by adding the condition "sps_video_parameter_set_id is greater than 0", which means that a VPS is referred to by the CLVS. [JVET-P0097][SW/M. M. Hannuksela]

- Add a constraint to require that when vps_independent_layer_flag[ i ] is equal to 0 there shall be at least one value of j such that the value of vps_direct_dependency_flag[ i ][ j ] is equal to 1. [JVET-P0135][SW/X. Ma]

The BoG recommends confirming/revisiting the following items:

- Support of mixed IRAP/non-IRAP slices within a picture, with two competing proposals on the table, one in JVET-P0124/ JVET-P0095, and the other in JVET-P00JVET-P0458

- JVET-P0404 on an SEI message for MPEG-I Part 7 metadata

- JVET-P0506 on max QT/BT/TT size signalling

- Should EOS NAL units be layer specific (i.e., an EOS NAL unit applies only to the layer with the same nuh_layer_id as the EOS NAL unit?

JVET-P0380 is related. It uses nuh_layer_id equal to 0 as a special layer that does not contain VCL NAL units but contains non-VCL NAL units that apply across layers, regardless of which layers are present. It was unclear to some participants at the HLS BoG meeting on Monday 10/7 if the earlier JVET decision on not taking action on JVET-P0380 only applies to its AUD aspect or more generally.

Therefore the BoG requests the track/plenary to confirm the earlier JVET decision, and revisit either only the aspect of whether EOS NAL units should be layer specific, or additionally JVET-P0380 excluding its AUD aspect.

- Issues noted in the HLS BoG meeting minutes, agenda item 6.18.5 (AUD, picture header, slice header parameters signalling), under document JVET-P0687, section "On signalling constant slice header params", item categories 1 and 2, if not already resolved by non-HLS-BoG meeting sessions

- Proposals 2 and 3 of JVET-P0132, if not already resolved by non-HLS-BoG meeting sessions

- JVET-P0446 on HRD bug fix and enhancements for slicing, open GOP and DRAP support

- JVET-P1004 on removal of bricks

- JVET-P0336 on support of ROI (Region-Of-Interest) scalability

# Review of contributions

The HLS BoG meeting minutes are included in the attached document, with change marks relative to JVET-P_Notes_d2.