| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 Oct. 2019 | Document: JVET-P0970-v1 |
| --- | --- |

| Title: | AHG13: Additional ISP Tool-Off tests |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Santiago De-Luxán-Hernández, Valeri George, Gayathri Venugopal, Jens Brandenburg, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Einsteinufer 37 10587 Berlin, Germany | Email: | santiago.de.luxan@hhi.fraunhofer.de firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document provides additional tool-off tests of the ISP coding mode with the LFNST combination proposed in JVET-P0392, for higher QP ranges than for CTC and for lossless coding. The reported losses when turning ISP off are:

- AHG13:			AI: 0.41% (Y), 86% (Enc), 97% (Dec) RA: 0.20% (Y), 98% (Enc), 100% (Dec)

- JVET-P0392 t2:		AI: 0.52% (Y), 85% (Enc), 98% (Dec) RA: 0.30% (Y), 96% (Enc),   99% (Dec)

- QP 27,32,37,42:	AI: 0.60% (Y), 87% (Enc), 98% (Dec) RA: 0.27% (Y), 98% (Enc),   99% (Dec)

- QP 32,37,42,47:	AI: 0.84% (Y), 89% (Enc), 98% (Dec) RA: 0.35% (Y), 98% (Enc), 100% (Dec)

- AHG18 lossless:	AI: 0.67%, 		 84% (Enc), 106% (Dec) RA: 0.12%,        98% (Enc), 100% (Dec)

Table 1: Summary of the results

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Type | Y (%) | EncT (%) | DecT (%) | Y (%) | EncT (%) | DecT (%) |
| AHG13 | 0.41 | 86 | 97 | 0.2 | 98 | 100 |
| JVET-P0392 (T2) | 0.52 | 85 | 98 | 0.3 | 96 | 99 |
| QPs (27-42) | 0.6 | 87 | 98 | 0.27 | 98 | 99 |
| QPs (32-47) | 0.84 | 89 | 98 | 0.35 | 98 | 100 |
| AHG18 lossless | 0.67 | 84 | 106 | 0.12 | 98 | 100 |

# Results

## VTM-6.0 with CTC

For comparison purposes, the results reported by AHG13 on top of VTM-6.0 using the CTC are presented this document as well.

Table 2: Tool-off test results on top of VTM-6.0 using CTC for AI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.06% | -0.16% | 88% | 99% |
| Class A2 | 0.27% | 0.21% | 0.24% | 88% | 97% |
| Class B | 0.40% | 0.36% | 0.30% | 86% | 97% |
| Class C | 0.55% | 0.32% | 0.53% | 84% | 96% |
| Class E | 0.72% | 0.58% | 0.62% | 85% | 97% |
| Overall | 0.41% | 0.29% | 0.32% | 86% | 97% |
| Class D | 0.48% | 0.45% | 0.52% | 84% | 99% |
| Class F | 0.54% | 0.25% | 0.16% | 93% | 97% |

Table 3: Tool-off test results on top of VTM-6.0 using CTC for RA

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | 0.14% | 0.10% | 99% | 100% |
| Class A2 | 0.17% | 0.13% | 0.04% | 98% | 100% |
| Class B | 0.20% | 0.35% | 0.28% | 98% | 101% |
| Class C | 0.26% | 0.31% | 0.27% | 97% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.20% | 0.25% | 0.19% | 98% | 100% |
| Class D | 0.22% | -0.31% | 0.25% | 97% | 100% |
| Class F | 0.36% | 0.10% | 0.41% | 98% | 97% |

## On top of JVET-P0392

When the LFNST was adopted during the Geneva Meeting in March 2019, it was implemented in a mutually exclusive way with respect to ISP. JVET-P0392 proposes to remove this restriction and allow the combination of these two tools in the same CU. No new kernels are added and the existing ones are not retrained. The tool-off test results of ISP on top of Test 2 of this contribution are presented.

Table 4: Tool-Off test results on top of JVET-P0392 (Test 2) for AI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-P0392 Test 4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.19% | -0.48% | 87% | 101% |
| Class A2 | 0.32% | 0.25% | 0.27% | 87% | 98% |
| Class B | 0.48% | 0.49% | 0.42% | 84% | 96% |
| Class C | 0.80% | 0.47% | 0.54% | 83% | 99% |
| Class E | 0.85% | 0.70% | 0.68% | 84% | 98% |
| Overall | 0.52% | 0.37% | 0.32% | 85% | 98% |
| Class D | 0.62% | 0.51% | 0.66% | 83% | 101% |
| Class F | 0.70% | 0.38% | 0.32% | 92% | 99% |

Table 5: Tool-Off test results on top of JVET-P0392 (Test 2) for RA

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over JVET-P0392 Test 1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | 0.28% | 0.15% | 97% | 98% |
| Class A2 | 0.23% | 0.03% | 0.06% | 97% | 99% |
| Class B | 0.28% | 0.26% | 0.32% | 96% | 100% |
| Class C | 0.47% | 0.26% | 0.39% | 96% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.30% | 0.22% | 0.25% | 96% | 99% |
| Class D | 0.37% | -0.21% | 0.43% | 97% | 100% |
| Class F | 0.50% | 0.14% | 0.42% | 96% | 99% |

## VTM-6.0 with higher QP ranges than {22, 37}

### QP = {27, 32, 37, 42}

Table 6: Tool-Off test results with the QP range (27, 42) for AI

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 with (27-42) QP range |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | -0.12% | -0.25% | 90% | 99% |
| Class A2 | 0.55% | 0.22% | 0.27% | 89% | 97% |
| Class B | 0.66% | 0.41% | 0.22% | 86% | 99% |
| Class C | 0.68% | 0.28% | 0.58% | 86% | 97% |
| Class E | 0.89% | 0.61% | 0.57% | 86% | 97% |
| Overall | 0.60% | 0.29% | 0.29% | 87% | 98% |
| Class D | 0.62% | 0.42% | 0.40% | 84% | 96% |
| Class F | 0.62% | 0.22% | 0.31% | 92% | 98% |

Table 7: Tool-Off test results with the QP range (27, 42) for RA

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 with (27-42) QP range |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.17% | 0.20% | 0.18% | 97% | 100% |
| Class A2 | 0.24% | -0.15% | 0.02% | 98% | 99% |
| Class B | 0.30% | 0.40% | 0.31% | 98% | 99% |
| Class C | 0.33% | 0.34% | 0.17% | 97% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.27% | 0.23% | 0.19% | 98% | 99% |
| Class D | 0.26% | -0.38% | 0.64% | 97% | 98% |
| Class F | 0.40% | 0.22% | 0.33% | 98% | 98% |

### QP = {32, 37, 42, 47}

Table 8: Tool-Off test results with the QP range (32, 47) for AI

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 with QP range (32, 47) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.32% | -0.28% | -0.32% | 92% | 100% |
| Class A2 | 0.84% | 0.18% | 0.32% | 90% | 99% |
| Class B | 0.96% | 0.42% | 0.19% | 88% | 98% |
| Class C | 0.87% | 0.27% | 0.47% | 87% | 96% |
| Class E | 1.14% | 0.69% | 0.60% | 88% | 98% |
| Overall | 0.84% | 0.28% | 0.26% | 89% | 98% |
| Class D | 0.83% | 0.33% | 0.43% | 86% | 100% |
| Class F | 0.73% | 0.13% | 0.37% | 93% | 97% |

Table 9: Tool-Off test results with the QP range (32, 47) for RA

|  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 with QP range (32, 47) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.19% | 0.28% | 0.22% | 99% | 103% |
| Class A2 | 0.28% | -0.30% | 0.07% | 98% | 98% |
| Class B | 0.43% | 0.34% | 0.37% | 98% | 100% |
| Class C | 0.42% | 0.33% | 0.14% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.35% | 0.20% | 0.22% | 98% | 100% |
| Class D | 0.39% | 0.09% | 1.21% | 98% | 99% |
| Class F | 0.43% | 0.18% | 0.03% | 98% | 98% |

## On top of the lossless implementation proposed in JVET-P0606

Table 11 shows the results of VTM-6.0 against HEVC RExt using a lossless configuration, as shown by the AGH18 Report in JVET-P0018. It can be observed that VTM-6.0 has a loss of 0.55% in AI and a gain of 0.71% in RA.

Table 10: Results of VTM-6.0 against HEVC RExt using a lossless configuratio. Source: JVET-P0018

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | HM-16.20 RExt | VTM6 |  | HM-16.20 RExt | VTM6 |  |
| Class A1 | 2.2 | 2.2 | -0.75% | 2.3 | 2.3 | -0.83% |
| Class A2 | 1.8 | 1.7 | 2.41% | 1.9 | 1.9 | 1.44% |
| Class B | 2.1 | 2.1 | 0.71% | 2.3 | 2.3 | -1.21% |
| Class C | 2.1 | 2.1 | -0.13% | 2.6 | 2.6 | -1.62% |
| Class D | 2.0 | 1.9 | 1.95% | 2.7 | 2.8 | -1.25% |
| Class E | 3.0 | 2.9 | 0.62% |  |  |  |
| Class F | 5.2 | 5.5 | -5.07% | 35.5 | 39.4 | -4.80% |
| TGM | 8.2 | 12.3 | -29.76% | 106.3 | 112.6 | -10.17% |
| Overall | 2.2 | 2.2 | 0.55% | 2.3 | 2.3 | -0.71% |
| Enc Time[%] | 6022% |  |  | 1529% |  |  |
| Dec Time[%] | 205% |  |  | 167% |  |  |

Table 11: Tool-off test results of ISP against the AHG18 VTM-6.0 lossless anchor

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | ISP off |  | AHG18 | ISP off |  |
| Class A1 | 2.4 | 2.4 | 0.10% | 2.4 | 2.4 | 0.08% |
| Class A2 | 1.7 | 1.7 | 0.30% | 1.9 | 1.9 | 0.06% |
| Class B | 2.3 | 2.3 | 0.59% | 2.5 | 2.4 | 0.16% |
| Class C | 2.0 | 2.0 | 0.55% | 2.6 | 2.6 | 0.13% |
| Class D | 2.0 | 2.0 | 0.65% | 3.0 | 3.0 | 0.13% |
| Class E | 3.0 | 2.9 | 1.93% |  |  |  |
| Class F | 4.5 | 4.5 | 1.12% | 22.2 | 22.1 | 0.50% |
| TGM | 12.0 | 11.9 | 0.39% | 65.6 | 65.4 | 0.17% |
| Overall | 2.3 | 2.3 | 0.67% | 2.4 | 2.4 | 0.12% |
| Enc Time[%] | 84% |  |  | 95% |  |  |
| Dec Time[%] | 106% |  |  | 101% |  |  |