| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0984 |
| --- | --- |

| Title: | AHG17: Subpicture level info for extraction and merging |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Jill Boyce | Email: | {firstname.lastname}@hhi.fraunhofer.de jill.boyce@intel.com |
| Source: | Fraunhofer HHI, Intel |  |  |

_____________________________

# Abstract

The document contains a harmonized proposal after BoG review of JVET-P0448 and proposes syntax for indicating the level of sub-pictures to enable extracting and merging use cases based on documents JVET-O0492, JVET-O555 and JVET-P0448.

It is proposed to add metadata syntax to indicate the level limits per sub-picture that are obeyed by the encoder. The level limits are indicated as a fraction of indicated reference levels, either inferred based on the subpicture size or explicitly signalled.

# Introduction

The document contains a harmonized proposal after BoG review of JVET-P0448 and proposes syntax for indicating the level of sub-pictures to enable extracting and merging use cases based on documents JVET-O0492, JVET-O555 and JVET-P0448.

# Proposed solution

It is proposed to add syntax to indicate the level limits per sub-picture that are obeyed by the encoder during encoding. The subpicture level limits are indicated as a fraction of indicated reference levels wherein the respective reference level fraction is either inferred from subpicture size in samples or explicitly signalled.

It is asserted that such an indication will help in the extraction as well as the merging of subpictures.

A process is also proposed to derive a level for an extracted set of subpictures.

## Syntax

| sub_pic_level_info( ) { | Descriptor |
| --- | --- |
| num_sub_pictures_minus1 | u(7) |
| num_ref_levels_minus1 | u(3) |
| explicit_fraction_present_flag | u(1) |
| reserved_zero_4bits | u(4) |
| for( i = 0; i <= num_ref_levels_minus1; i++ ){ |  |
| ref_level_idc[ i ] | u(8) |
| if (explicit_fraction_present_flag) { |  |
| for( j = 0; j <= num_sub_pictures_minus1; j++ ){ |  |
| ref_level_fraction_minus1[ i ][ j ] | u(8) |
| } |  |
| } |  |
| } |  |

## Semantics

num_sub_pictures_minus1 specifies the number of subpictures in the current bitstream. The value of num_sub_pictures_minus1 shall be in the range of 0 to 127 [Ed. (RS): syntax and semantics to be aligned with the allowed  number of subpictures per picture].

num_ref_levels_minus1 specifies the number of reference levels signalled for each of the num_sub_pictures_minus1 + 1 subpictures.

explicit_fraction_present_flag equal to 1 specifies that the syntax elements num_sub_pictures_minus1 and ref_level_fraction_minus1[ i ] are present. explicit_fraction_present_flag equal to 0 specifies that the syntax elements num_sub_pictures_minus1 and ref_level_fraction_minus1[ i ] are not present.

ref_level_idc[ i ] indicates a level to which each subpicture conforms as specified in Annex A. Bitstreams shall not contain values of ref_level_idc other than those specified in Annex A. Other values of ref_level_idc[ i ] are reserved for future use by ITU-T | ISO/IEC.

It is a requirement of bitstream conformance that the value of ref_level_idc[ i ] is smaller than or equal to ref_level_idc[ k ] for any k greater than i.

ref_level_fraction_minus1[ i ][ j ] plus 1 specifies the fraction of the level limits associated with ref_level_idc[ i ] that the j-th subpicture conforms to as specified in Annex A.4.1. The value of ref_level_fraction_minus1[ i ][ j ] shall be in the range of 0 to 255, inclusive.

The variable SubPicSizeY[ j ] of the j-th subpicture is set equal to sub_pic_width[ j ] * sub_pic_height[ j ].

When not present, the value of ref_level_fraction_minus1[ i ][ j ] is inferred to be equal to Ceil(256 * SubPicSizeY[ j ] ÷ PicSizeInSamplesY * MaxLumaPs ( general_level_idc ) ÷ MaxLumaPs ( ref_level_idc[ i ] ) - 1.

The variable RefLevelFraction[ i ][ j ] is set equal to ref_level_fraction_minus1[ i ][ j ] + 1.

The variables SubPicNumTileCols[ j ] and SubPicNumTileRows[ j ] of the j-th subpicture are derived as follows.

[RS: tbd add derivation of tile cols and rows per subpic pending decision on brick removal.]

The variables SubPicCbpSizeVcl[ i ][ j ], and SubPicCbpSizeNal[ i ][ j ] of the j-th subpicture with respect to ref_level_idc[ i ] are derived as follows.

SubPicCbpSizeVcl[ i ][ j ] = Floor( CpbVclFactor * MaxCPB * RefLevelFraction[ i ][ j ] ÷ 256)

SubPicCbpSizeNal[ i ][ j ] = Floor( CpbNalFactor * MaxCPB * RefLevelFraction[ i ][ j ] ÷ 256)

with MaxCPB derived from ref_level_idc[ i ][ j ] as specified in Annex A.4.2

NOTE 1 – When a subpicture is extracted, the resulting bitstream shall have a CpbSize (either indicated in the VUI parameters or inferred) that is greater than or equal to or SubPicCbpSizeVcl[ i ][ j ] and SubPicCbpSizeNal[ i ][ j ].

It is a requirement of bitstream conformance that the bitstreams resulting from extracting the subpicture j for j in the range of 0 to NumSubPics and conforming to a profile at a specified tier [RS: limit to low tier] and level equal to ref_level_idc[ i ][ j ] for i in the range of 0 to num_ref_level_minus1shall obey the following constraints for each bitstream conformance test as specified in Annex C:

It is a requirement of bitstream conformance that the following constraints are met for the subpicture j and the level equal to ref_level_idc[ i ][ j ], for all j in 0 .. NumSubPics and i in 0 .. num_ref_level_minus1:

- Ceil( 256 * SubPicSizeY[ i ] ÷ RefLevelFraction[ i ][ j ] )  shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.1.

- The value of Ceil( 256 * sub_pic_width[ i ] ÷ RefLevelFraction[ i ][ j ] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of Ceil( 256 * sub_pic_height[ i ] ÷ RefLevelFraction[ i ][ j ] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

- The value of SubPicNumTileCols[ j ] shall be equal to or less than MaxTileCols and of SubPicNumTileRows[ j ] shall be equal to or less than MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.1.

## Derivation of level for subpicture set

For any subpicture set containing one ore more subpictures, the level of the subpicture set is derived as follows.

The variable SubPicSetAccLevelFraction[ i ] for the total level fraction of the subpicture set in units of luma samples for the reference level ref_level_idc[ i ] is derived as follows.

for (i = 0; i <= num_target_level_minus1; i ++)

SubPicSetAccLevelFraction [ i ] = 0

SubPicSetCbpSizeVcl[ i ] = 0

SubPicSetCbpSizeNal[ i ] = 0

for (j = 0; j <= max_sub_pics_minus1; j ++)

if (sub-picture j is in the sub-picture set) {

SubPicSetAccLevelFraction[ i ] += RefLevelFraction[ i ][ j ]

SubPicSetCbpSizeVcl[ i ] += SubPicSetCbpSizeVcl[ i ][ j ]

SubPicSetCbpSizeNal[ i ] += SubPicSetCbpSizeNal[ i ][ j ] 
}

The subpicture set bitstream conforming to a profile at a specified tier [RS: limit to low tier] and a level equal to SubPicSetLevelIdc shall obey the following constraints for each bitstream conformance test as specified in Annex C:

- For the VCL HRD parameters, SubPicSetCbpSizeVcl[ i ] shall be less than or equal to CpbVclFactor * MaxCPB, where CpbVclFactor is specified in Table A.8, and MaxCPB is specified in Table A.6 in units of CpbVclFactor bits.

- For the NAL HRD parameters, SubPicSetCbpSizeVcl[ i ] shall be less than or equal to CpbNalFactor * MaxCPB, where CpbNalFactor is specified in Table A.8, and MaxCPB is specified in Table A.6 in units of CpbNalFactor bits.MaxCPB

[RS: tbd add derivation of SubPicSetNumTiles from tile cols and rows per subpic decision on brick removal.]

The value of the subpicture set sequence level indicator,  SubPicSetLevelIdc, is derived as follows.

SubPicSetLevelIdc = general_level_idc

for (i = num_target_level_minus1; i > = 0; i--)

if (  SubPicSetNumTiles * && SubPicSetAccLevelFraction[ i ] <= 256)

SubPicSetLevelIdc = ref_level_idc[ i ]

NOTE 1 – When a subpicture set is extracted, the resulting bitstream shall have a CpbSize (either indicated in the VUI parameters or inferred) that is equal to or bigger than SubPicCbpSizeVcl[ i ][ j ] and SubPicSetCbpSizeNal[ i ][ j ].

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Intel may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).