| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P0989 |
| --- | --- |

| Title: | Cross-check of JVET-P0617 (Non-CE4: On TPM merge mode in the presence of weighted prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | P.Bordes | Tel: Email: | +33.2.56.85.62.76 philippe.bordes@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports crosscheck results of JVET-P0617 and verifies that the software matches the description. The partial results and encoder/decoder run times for fade sequences ae in-line with the ones showed in the proposal. Some differences come from the use of multi-streams simulations in P0617 but single stream in the cross-check.

# Verification of software

Fade sequences have been generated with the fading tool provided in JVET-L0201-v1 [1] from the regular CTC test sequences. A linear fade is applied to the about first 2 seconds of the regular sequence set, and the experiments are conducted with them. A linear fade-out (sequence to black or white) is applied in the 1st half and a linear fade-in (respectively black or white to sequence) in the 2nd half. The fade strength is limited to the range [0.25, 0.75] instead of [0, 1] to avoid extreme PSNR numbers at the frame level that could cause abnormal rate-distortion graphs.

The WP parameters for encoder configuration, were set as follows:

--WeightedPredMethod=0 --WeightedPredB=1 --WeightedPredP=1

It is verified that the software matches the proposal.

# Results

The performance for the proposal is shown below in comparison to VTM-6.0 with same WP settings. Both BD-rate gains and run times are in line with the proponent results, except some differences come from the use of multi-streams simulations in P0617 but single stream in the cross-check.

Table 1: summary of proposed cross-checked performance – test-2 (Fade to black).

|  | Proposal difference with VTM-6.0 (Fade to black) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | 0.42% | 0.73% | 1.07% | 101% |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.04% | 0.60% | -0.21% | 101% |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 1: summary of proposed cross-checked performance – test-3 (Fade to black).

|  | Proposal difference with VTM-6.0 (Fade to black) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | -1.05% | -0.90% | -0.99% | 103% |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.77% | -0.62% | -0.97% | 103% |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 1: summary of proposed cross-checked performance – test-4 (Fade to black).

|  | Proposal difference with VTM-6.0 (Fade to black) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C | 9.04% | 12.34% | 13.40% | 74% |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 7.83% | 14.14% | 14.41% | 80% |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

# References

- P. Bordes, E. François and F. L. Léannec, AHG13 - Weighted Prediction vs Generalized Bi-prediction with Fade sequences, Joint Video Exploration Team Document, JVET-L0201, Macao, Oct. 2018.

# Patent rights declaration(s)

InterDigital does not have any current or pending patent rights relating to the technology described in this contribution.