| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P1031-v2 |
| --- | --- |

| Title: | Proposed text for MTS related HLS (harmonization of JVET-P0495, JVET-P0501 and JVET-P0569) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu 2747 Park Blvd Palo Alto, CA 94306, USA Hilmi E. Egilmez, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA Yang Wang, Hongbin Liu, Li Zhang, Kai Zhang, Zhipin Deng, Yue Wang Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China | Tel: Email: | xinzzhao@tencent.com hegilmez@qti.qualcomm.com wangyang.cs@bytedance.com |
| Source: | Tencent, Qualcomm Inc., Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the spec text for MTS related HLS based on a harmonized solution of the methods proposed in JVET-P0495, JVET-P0501 and JVET-P0569.

# Proposed specification text modifications

#### 7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… | u(1) |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| sps_implicit_dct2_flag | u(1) |
| if( sps_implicit_dct2_flag = = 0 ) { |  |
| sps_intra_mts_selection | ue(v) |
| sps_inter_mts_selection | ue(v) |
| } |  |
| …… | u(1) |
| } |  |

#### 7.4.3.3	Sequence parameter set RBSP semantics

sps_mts_enabled_flag equal to 1 specifies that sps_explicit_mts_intra_enabled_flag is present in the sequence parameter set RBSP syntax and that sps_explicit_mts_inter_enabled_flag is present in the sequence parameter set RBSP syntax. sps_mts_enabled_flag equal to 0 specifies that sps_explicit_mts_intra_enabled_flag is not present in the sequence parameter set RBSP syntax and that sps_explicit_mts_inter_enabled_flag is not present in the sequence parameter set RBSP syntax.

sps_explicit_mts_intra_enabled_flag equal to 1 specifies that tu_mts_idx may be present in the transform unit syntax for intra coding units. sps_explicit_mts_intra_enabled_flag equal to 0 specifies that tu_mts_idx is not present in the transform unit syntax for intra coding units. When not present, the value of sps_explicit_mts_intra_enabled_flag is inferred to be equal to 0.

sps_explicit_mts_inter_enabled_flag equal to 1 specifies that tu_mts_idx may be present in the transform unit syntax for inter coding units. sps_explicit_mts_inter_enabled_flag equal to 0 specifies that tu_mts_idx is not present in the transform unit syntax for inter coding units. When not present, the value of sps_explicit_mts_inter_enabled_flag is inferred to be equal to 0.

sps_implicit_dct2_flag equal to 0 specifies that sps_intra_mts_selection and sps_inter_mts_selection are present in the bistream. sps_implicit_dct2_flag equal to 1 specifies that sps_intra_mts_selection and sps_inter_mts_selection are not present in the bitstream, and trTypeHor and trTypeVer are inferred as 0 (i.e., only DCT-2 is used for all TUs on which transform skip is not applied).

sps_intra_mts_selection equal to 0 or 1 specifies that that tu_mts_idx is not present in the transform unit syntax for intra coding units. sps_intra_mts_selection equal to 2 specifies that tu_mts_idx may be present in the transform unit syntax for intra coding units. sps_intra_mts_selection equal to 0 further specifies that implicit MTS is not applied for intra prediction residuals and sps_intra_mts_selection equal to 1 further specifies that implicit MTS is applied for intra prediction residuals. When not present, the value of sps_intra_mts_selection is derived as 0.

sps_inter_mts_selection equal to 0 or 1 specifies that tu_mts_idx is not present in the transform unit syntax for inter coding units. sps_inter_mts_selection equal to 2 specifies that tu_mts_idx may be present in the transform unit syntax for inter coding units. sps_inter_mts_selection equal to 0 further specifies that implicit MTS is not applied for inter prediction residuals and sps_inter_mts_selection equal to 1 further specifies that implicit MTS is applied for inter prediciton residuals. When not present, the value of sps_inter_mts_selection is derived as 0.

#### 7.3.8.10	Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag sps_intra_mts_selection = =2 ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag sps_inter_mts_selection = =2 ) ) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] && !( tu_cbf_cb[ x0 ][ y0 ] && tu_joint_cbcr_residual_flag[ x0 ][ y0 ] )) { |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

#### 7.3.8.11	Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| if( ( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( sps_explicit_mts_inter_enabled_flagsps_inter_mts_selection && cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbWidth > 4 ) |  |
| log2ZoTbWidth = 4 |  |
| else |  |
| log2ZoTbWidth = Min( log2TbWidth, 5 ) |  |
| MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) |  |
| if( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( sps_explicit_mts_inter_enabled_flag sps_inter_mts_selection && cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbHeight > 4 ) |  |
| log2ZoTbHeight = 4 |  |
| else |  |
| log2ZoTbHeight = Min( log2TbHeight, 5 ) |  |
| …… |  |

### 8.7.4	Transformation process for scaled transform coefficients

#### 8.7.4.1	General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

……

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and sps_intra_mts_selection is not equal to 0

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32 and sps_inter_mts_selection is not equal to 0

- sps_explicit_mts_intra_enabled_flag sps_intra_mts_selection is equal to 01 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA and lfnst_idx[ x0 ][ y0 ] is equal to 0 and intra_mip_flag[ x0 ][ y0 ] is equal to 0

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT or sps_explicit_mts_intra_enabled_flag is equal to 0 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA, trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(8-975)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(8-976)

- Otherwise (cu_sbt_flag is equal to 1), trTypeHor and trTypeVer are specified in Table 8-15 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-14 depending on tu_mts_idx[ xTbY ][ yTbY ].

# Proposed specification text modifications combined with suggested editorial changes

Both the proposed and editorial changes are highlighted in yellow.

#### 7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… | u(1) |
| sps_mts_enabled_flag | u(1) |
| if( sps_mts_enabled_flag ) { |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |
| sps_explicit_mts_inter_enabled_flag | u(1) |
| } |  |
| sps_implicit_dct2_flag | u(1) |
| if( sps_implicit_dct2_flag = = 0 ) { |  |
| sps_intra_mts_selection | ue(v) |
| sps_inter_mts_selection | ue(v) |
| } |  |
| …… | u(1) |
| } |  |

#### 7.4.3.3	Sequence parameter set RBSP semantics

sps_mts_enabled_flag equal to 1 specifies that sps_explicit_mts_intra_enabled_flag is present in the sequence parameter set RBSP syntax and that sps_explicit_mts_inter_enabled_flag is present in the sequence parameter set RBSP syntax. sps_mts_enabled_flag equal to 0 specifies that sps_explicit_mts_intra_enabled_flag is not present in the sequence parameter set RBSP syntax and that sps_explicit_mts_inter_enabled_flag is not present in the sequence parameter set RBSP syntax.

sps_explicit_mts_intra_enabled_flag equal to 1 specifies that tu_mts_idx may be present in the transform unit syntax for intra coding units. sps_explicit_mts_intra_enabled_flag equal to 0 specifies that tu_mts_idx is not present in the transform unit syntax for intra coding units. When not present, the value of sps_explicit_mts_intra_enabled_flag is inferred to be equal to 0.

sps_explicit_mts_inter_enabled_flag equal to 1 specifies that tu_mts_idx may be present in the transform unit syntax for inter coding units. sps_explicit_mts_inter_enabled_flag equal to 0 specifies that tu_mts_idx is not present in the transform unit syntax for inter coding units. When not present, the value of sps_explicit_mts_inter_enabled_flag is inferred to be equal to 0.

sps_implicit_dct2_flag equal to 0 specifies that sps_intra_mts_selection and sps_inter_mts_selection are present in the bistream. sps_implicit_dct2_flag equal to 1 specifies that sps_intra_mts_selection and sps_inter_mts_selection are not present in the bitstream, and trTypeHor and trTypeVer are inferred as 0 (i.e., only DCT-2 is used for all TUs on which transform skip is not applied).

sps_intra_mts_selection equal to 0 or 1 specifies that that tu_mts_idx is not present in the transform unit syntax for intra coding units. sps_intra_mts_selection equal to 2 specifies that tu_mts_idx may be present in the transform unit syntax for intra coding units. sps_intra_mts_selection equal to 0 further specifies that implicit MTS derivation is not applied for intra predictedblocks and sps_intra_mts_selection greater than 0 further specifies that implicit MTS derivation is applied for ISP or non-ISP coded intra predicted blocks. When not present, the value of sps_intra_mts_selection is derived as 0.

sps_inter_mts_selection equal to 0 or 1 specifies that tu_mts_idx is not present in the transform unit syntax for inter coding units. sps_inter_mts_selection equal to 2 specifies that tu_mts_idx may be present in the transform unit syntax for inter coding units. sps_inter_mts_selection equal to 0 further specifies that implicit MTS derivation is not applied for SBT coded inter predicted blocks and sps_inter_mts_selection greater than 0 further specifies that implicit MTS derivation is applied for SBT coded inter predicted rblocks. When not present, the value of sps_inter_mts_selection is derived as 0.

#### 7.3.8.10	Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| …… |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA && ( tbWidth <= 32 ) && ( tbHeight <= 32 ) && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && ( !cu_sbt_flag ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize ) |  |
| transform_skip_flag[ x0 ][ y0 ] | ae(v) |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag sps_intra_mts_selection = =2 ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag sps_inter_mts_selection = =2 ) ) && ( !transform_skip_flag[ x0 ][ y0 ] ) ) |  |
| tu_mts_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( !transform_skip_flag[ x0 ][ y0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ x0 ][ y0 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| if( tu_cbf_cr[ x0 ][ y0 ] && !( tu_cbf_cb[ x0 ][ y0 ] && tu_joint_cbcr_residual_flag[ x0 ][ y0 ] )) { |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

#### 7.3.8.11	Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| if( ( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( sps_explicit_mts_inter_enabled_flagsps_inter_mts_selection && cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbWidth > 4 ) |  |
| log2ZoTbWidth = 4 |  |
| else |  |
| log2ZoTbWidth = Min( log2TbWidth, 5 ) |  |
| MaxCcbs = 2 * ( 1 << log2TbWidth ) * ( 1<< log2TbHeight ) |  |
| if( tu_mts_idx[ x0 ][ y0 ] > 0 ¦ ¦ ( sps_explicit_mts_inter_enabled_flag sps_inter_mts_selection && cu_sbt_flag && log2TbWidth < 6 && log2TbHeight < 6 ) ) && cIdx = = 0 && log2TbHeight > 4 ) |  |
| log2ZoTbHeight = 4 |  |
| else |  |
| log2ZoTbHeight = Min( log2TbHeight, 5 ) |  |
| …… |  |

### 8.7.4	Transformation process for scaled transform coefficients

#### 8.7.4.1	General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array r[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

……

The variable implicitIntraMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitIntraMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and sps_intra_mts_selection is not equal to 0

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag sps_intra_mts_selection is equal to 01 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA and lfnst_idx[ x0 ][ y0 ] is equal to 0 and intra_mip_flag[ x0 ][ y0 ] is equal to 0

- Otherwise, implicitIntraMtsEnabled implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If cIdx is greater than 0, trTypeHor and trTypeVer are set equal to 0.

- Otherwise, if implicitIntraMtsEnabled implicitMtsEnabled is equal to 1, the following applies:

- If IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT or sps_explicit_mts_intra_enabled_flag is equal to 0 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA, trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(8-975)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(8-976)

- Otherwise, if (cu_sbt_flag is equal to 1 and sps_inter_mts_selection is not equal to 0), trTypeHor and trTypeVer are specified in Table 8-15 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise, trTypeHor and trTypeVer are specified in Table 8-14 depending on tu_mts_idx[ xTbY ][ yTbY ].

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).