| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P1018-v2 |
| --- | --- |

| Title: | Non-CE8: An alternative IBC virtual buffer setting to avoid reference sample wrapping around |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Weijia Zhu, Kai Zhang, Hongbin Liu (Bytedance Inc) Xiaozhong Xu, Xiang Li, Shan Liu (Tencent) |  |  |
| Source: | Bytedance Inc. Tencent |  |  |

_____________________________

# Abstract

This proposal presents an IBC virtual buffer setting to avoid sample-wise IBC reference wrapping around. It is asserted that there is no performance change compared to VTM-6.0 encoder.

# Introduction

The current VVC design uses the IBC virtual buffer concept to clearly describe the reference area for IBC mode. When CTU size is 128x128, a VPDU memory reuse scheme, shown in Figure 1 in the virtual buffer view.

Figure 1. IBC virtual buffer in the current design. Buffer view is illustrated.

When a reference block is across IBC virtual buffer boundary, two times of copy might be needed. It was mentioned that it is desired to have only one copy for each IBC block.

# Proposed scheme

The issue, if there is, can be resolved by extend the IBC virtual buffer size by one CTU, as shown in Figure 2. At the beginning of decoding each VPDU, IBC virtual buffer corresponding to (xCb + ( IbcBufWidthY >> 1 ), yCb) will be reset instead of (xCb, yCb).

The enlarged IBC virtual buffer adds more VPDU’s area as unavailable in the buffer view. It is like a “firebreak” to avoid reference sample wrapping around from the picture view. Thus, all references that need sample wrapping around are invalid.

An illustration when CTU size is 64x64 is shown in Figure 2. Any reference block that is valid (contains no any samples in the grey area) is continuous in the picture view.

(a)                                           (b)

Figure 2. An illustration when CTU size is 64x64 (a) IBC virtual buffer view; (b) Picture view.

Figure 3 shows an example of IBC buffer status when CTU size is 128x128. It is also the case that all reference blocks are continuous in the picture view. The reference area is the same as that in VTM-5.0.

Figure 3. IBC virtual buffer view is illustrated when CTU size is 128x128.

By doing so, the discontinuity of the reference block can be avoided. In some hardware architects, for each IBC reference, only one-time’s copy from the memory is needed instead of two-time.

# Experimental results

Table I. BD-rate of the proposed setting compared with VTM-6.0.

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 96% |
| 4:2:0 TGM | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM-6.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 100% |
| 4:2:0 TGM | 0.00% | 0.00% | 0.00% | 100% | 98% |

Because the current encoder never searches references that need sample-level wrapping around, there is no output bitstream change. Thus, the RD points are the same as the anchor. The decoder of the proposed method in this contribution can decode the bitstream generated by VTM-6.0.

The software patch is attached along with this proposal.

# Changes to the draft text

The changes, marked in grey, are based on JVET-O2001-vE.

7.4.3.3 Sequence parameter set RBSP semantics

…

The variables CtbLog2SizeY, CtbSizeY, MinCbLog2SizeY, MinCbSizeY, IbcBufWidthY, IbcBufWidthC and Vsize are derived as follows:

CtbLog2SizeY = log2_ctu_size_minus5 + 5			(7-15)

CtbSizeY = 1  <<  CtbLog2SizeY					(7-16)

MinCbLog2SizeY = log2_min_luma_coding_block_size_minus2 + 2	(7-17)

MinCbSizeY = 1  <<  MinCbLog2SizeY			(7-18)

IbcBufWidthY = 128 256 * 128 / CtbSizeY			(7-19)

IbcBufWidthC = IbcBufWidthY / SubWidthC		(7-20)

VSize = Min( 64, CtbSizeY )						(7-21)

…

7.4.9.5 Coding unit semantics

…

When x0 % VSize is equal to 0 and y0 % VSize is equal to 0, the following assignments are made for x = x0..x0 + VSize − 1 and y = y0..y0 + VSize − 1:

IbcVirBuf[ 0 ][ (x + ( IbcBufWidthY >> 1 ) ) % IbcBufWidthY ][ y % CtbSizeY ] = −1	        (7-154)

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).