| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 Oct. 2019 | Document: JVET-P1023 |
| --- | --- |

| Title: | Joint solution for the reference picture conditions in DMVR and BDOF (JVET-P0089, JVET-P0191, JVET-P0311, JVET-P0415, JVET-P0546, JVET-P0598) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Huang, Chun-Chi Chen, Kevin Reuzé, Wei-Jung Chien, and Marta Karczewicz (Qualcomm) Na Zhang, Hongbin Liu, Li Zhang, Kai Zhang, Yue Wang (Bytedance) Wei Chen, Yuwen He (InterDigital) Kyohei Unno, Kei Kawamura, Sei Naito (KDDI) Fangdong Chen, Li Wang (Hikvision) | Email: | hanhuang@qti.qualcomm.com zhangna.cynthia@bytedance.com Wei.Chen@InterDigital.com ky-unno@kddi-research.jp chenfangdong@hikvision.com |
| Source: | Bytedance Inc., InterDigital Communications, Inc., KDDI Corp. (KDDI Research, Inc.), Hikvision and Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a joint solution for the reference picture conditions in DMVR and BDOF. It is proposed to disable DVMR and BDOF if any of the reference pictures is not a short-term reference picture and to impose an equal-POC-distance constraint on both DMVR and BDOF. Experimental results report that the average BD-rate changes on Y, Cb and Cr are 0.01%, 0.03% and 0.02%, respectively.

# Introduction

In [1], [2] and [3], it is proposed to apply DMVR and BDOF control based on reference picture types. In [4], [5] and [6], it is proposed to align the POC distance condition in DMVR and BDOF.

In this contribution, a joint solution is proposed: disable DMVR and BDOF if any of the reference pictures is not a short-term reference picture and impose an equal-POC-distance constraint on both DMVR and BDOF.

- Suggested changes to working draft

The proposed changes on top of JVET-O2001-vE are specified as below, with deleted text marked in red strike and newly added text highlighted in yellow.

### 8.5.1 General decoding process for coding units coded in inter prediction mode

…

The decoding process for coding units coded in inter prediction mode consists of the following ordered steps:

- The variable dmvrFlag is set equal to 0 and the variable hpelIfIdx is set equal to 0.

- The motion vector components and reference indices of the current coding unit are derived as follows:

…

- When all of the following conditions are true, dmvrFlag is set equal to 1:

- sps_dmvr_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flag is equal to 0

- general_merge_flag[ xCb ][ yCb ] is equal to 1

- both predFlagL0[ 0 ][ 0 ] and predFlagL1[ 0 ][ 0 ] are equal to 1

- mmvd_merge_flag[ xCb ][ yCb ] is equal to 0

- ciip_flag[ xCb ][ yCb ] is equal to 0

- DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

- RefPicList[ 0 ][ refIdxL0 ] is a short-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is a short-term reference picture.

- BcwIdx[ xCb ][ yCb ] is equal to 0

- Both luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0

- cbWidth is greater than or equal to 8

- cbHeight is greater than or equal to 8

- cbHeight*cbWidth is greater than or equal to 128

- For X being each of 0 and 1, the pic_width_in_luma_samples and pic_height_in_luma_samples of the reference picture refPicLX associated with the refIdxLX are equal to the pic_width_in_luma_samples and pic_height_in_luma_samples of the current picture, respectively.

…

8.5.6 Decoding process for inter blocks

#### 8.5.6.1 General

This process is invoked when decoding a coding unit coded in inter prediction mode.

…

  - The variable currPic specifies the current picture and the variable bdofFlag is derived as follows:

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - sps_bdof_enabled_flag is equal to 1 and slice_disable_bdof_dmvr_flag is equal to 0.

      - predFlagL0[ xSbIdx ][ ySbIdx ] and predFlagL1[ xSbIdx ][ ySbIdx ] are both equal to 1.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ] ) * DiffPicOrderCnt( currPic, RefPicList[ 1 ][ refIdxL1 ] ) is less than 0.

      - DiffPicOrderCnt( currPic, RefPicList[ 0 ][ refIdxL0 ]) is equal to DiffPicOrderCnt( RefPicList[ 1 ][ refIdxL1 ], currPic )

      - RefPicList[ 0 ][ refIdxL0 ] is a short-term reference picture and RefPicList[ 1 ][ refIdxL1 ] is a short-term reference picture.

      - MotionModelIdc[ xCb ][ yCb ] is equal to 0.

      - merge_subblock_flag[ xCb ][ yCb ] is equal to 0.

      - sym_mvd_flag[ xCb ][ yCb ] is equal to 0.

      - ciip_flag[ xCb ][ yCb ] is equal to 0.

      - BcwIdx[ xCb ][ yCb ] is equal to 0.

      - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

      - cbWidth is greater than or equal to 8.

      - cbHeight is greater than or equal to 8.

      - cbHeight * cbWidth is greater than or equal to 128.

      - For X being each of 0 and 1, the pic_width_in_luma_samples and pic_height_in_luma_samples of the reference picture refPicLX associated with the refIdxLX are equal to the pic_width_in_luma_samples and pic_height_in_luma_samples of the current picture, respectively.

      - cIdx is equal to 0.

    - Otherwise, bdofFlag is set equal to FALSE.

…

# Test Result

The proposed methods were tested on top of the VTM-6.0 software following the common test conditions (JVET-N1010) to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. The VTM anchor and testing results were generated using the VTM configurations with --PrintHexPSNR=1 and --SIMD=SSE42 for encoder and --SIMD=SSE42 for decoder. Macro settings as specified in the VTM-6.0 software are not changed. Parallel encoding and decoding were applied to Random Access case for both anchors and test. The BD-rate performance is shown in Table 1.

Table 1 Test result

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-6.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | 0.04% | 100% | 100% |
| Class A2 | 0.02% | 0.00% | 0.01% | 99% | 100% |
| Class B | 0.01% | 0.02% | -0.01% | 100% | 100% |
| Class C | 0.00% | 0.05% | 0.06% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.02% | 100% | 99% |
| Class D | 0.05% | 0.00% | 0.04% | 97% | 94% |
| Class F | 0.01% | 0.01% | 0.01% | 98% | 98% |

# References

- H. Huang, W.-J. Chien, M. Karczewicz, “Non-CE4: DMVR control by reference picture type”, JVET-P0089, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, Oct. 2019.

- N. Zhang, H. Liu, L. Zhang, K. Zhang, Y. Wang, “Non-CE4: Enabling BDOF and DMVR according to reference picture types”, JVET-P0191, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, Oct. 2019.

- F. Chen, L. Wang, “Non-CE4: On enabling condition of BDOF and DMVR”, JVET-P0311, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, Oct. 2019.

- W. Chen, Y. He, “Non-CE4: Unification of DMVR and BDOF enabling conditions”, JVET-P0415, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, Oct. 2019.

- K. Unno, K. Kawamura, S. Naito, “Non-CE4: An applying condition of BDOF”, JVET-P0546, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, Oct. 2019.

- C.-C. Chen, H. Huang, K. Reuzé, W.-J. Chien, M. Karczewicz, “CE4-related: Harmonized CU-level Condition Check in BDOF and DMVR”, JVET-P0598, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Geneva, CH, Oct. 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).