| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P1025-v1 |
| --- | --- |

| Title: | Performance of CE7-1.3b* in Lossless Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tung Nguyen Benjamin Bross | Email: | tung.nguyen@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the results of CE7-1.3b* modifications with 1.75 CCB limit for lossless coding where only TS residual coding is employed (using TS and a QP lower than 5 for 8-bit). Additional informative results have been generated. The first test modifies the Rice code table of CE7.1.3b* by the table of CE7-1.2d* (CE7-1.3b**). The second test is a modified version of CE7-1.2d* (CE7-1.2d**) The results, relative to the AHG18 lossless evaluation software, the as follows.

CE7.1.3b*:	AI: 4.01%	RA: 3.42%
	CE7.1.3b**:	AI: 1.36%	RA: 1.75%
	CE7.1.2d**:	AI: 0.84%	RA: 2.21%

The results indicate that the Rice parameter selection of the adopted CE7-1.3b* is suboptimal for lossless TSRC.

# Results

CE7-1.3b*

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | CE7-1.3b* |  | AHG18 | CE7-1.3b* |  |
| Class A1 | 2,2 | 2,1 | 5,10% | 0,0 | 0,0 | 0,00% |
| Class A2 | 1,7 | 1,6 | 6,03% | 0,0 | 0,0 | 0,00% |
| Class B | 2,1 | 2,0 | 3,86% | 2,3 | 2,1 | 6,03% |
| Class C | 2,1 | 2,0 | 3,09% | 2,6 | 2,5 | 5,28% |
| Class D | 1,9 | 1,9 | 2,62% | 2,8 | 2,6 | 4,99% |
| Class E | 2,9 | 2,9 | 2,38% |  |  |  |
| Class F | 5,5 | 5,7 | -2,70% | 39,4 | 41,5 | -1,79% |
| TGM | 12,3 | 13,0 | -5,15% | 112,6 | 126,2 | -8,69% |
| Overall | 2,2 | 2,1 | 4,01% | 1,5 | 1,4 | 3,42% |
| Enc Time[%] | 61% |  |  | 90% |  |  |
| Dec Time[%] | 104% |  |  | 101% |  |  |

CE7-1.3b**

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | CE7-1.3** |  | AHG18 | CE7-1.3** |  |
| Class A1 | 2,2 | 2,2 | 1,72% | 0,0 | 0,0 | 0,00% |
| Class A2 | 1,7 | 1,7 | 1,97% | 0,0 | 0,0 | 0,00% |
| Class B | 2,1 | 2,1 | 1,26% | 2,3 | 2,2 | 2,76% |
| Class C | 2,1 | 2,1 | 0,96% | 2,6 | 2,5 | 3,10% |
| Class D | 1,9 | 1,9 | 0,23% | 2,8 | 2,7 | 2,76% |
| Class E | 2,9 | 2,9 | 1,10% |  |  |  |
| Class F | 5,5 | 5,8 | -3,87% | 39,4 | 41,8 | -2,72% |
| TGM | 12,3 | 13,2 | -6,20% | 112,6 | 123,3 | -7,52% |
| Overall | 2,2 | 2,2 | 1,36% | 1,5 | 1,4 | 1,75% |
| Enc Time[%] | 61% |  |  | 89% |  |  |
| Dec Time[%] | 100% |  |  | 92% |  |  |

CE7-1.2d**

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | AHG18 | CE7-1.2d** |  | AHG18 | CE7-1.2d** |  |
| Class A1 | 2,2 | 2,2 | 0,54% | 0,0 | 0,0 | 0,00% |
| Class A2 | 1,7 | 1,7 | 0,91% | 0,0 | 0,0 | 0,00% |
| Class B | 2,1 | 2,1 | 0,35% | 2,3 | 2,2 | 2,96% |
| Class C | 2,1 | 2,1 | 0,52% | 2,6 | 2,5 | 4,59% |
| Class D | 1,9 | 2,0 | -0,35% | 2,8 | 2,6 | 4,20% |
| Class E | 2,9 | 2,9 | 2,29% |  |  |  |
| Class F | 5,5 | 5,8 | -3,89% | 39,4 | 41,7 | -1,78% |
| TGM | 12,3 | 13,2 | -6,46% | 112,6 | 126,7 | -9,32% |
| Overall | 2,2 | 2,2 | 0,84% | 1,5 | 1,4 | 2,21% |
| Enc Time[%] | 61% |  |  | 93% |  |  |
| Dec Time[%] | 105% |  |  | 98% |  |  |