| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P1035 |
| --- | --- |

| Title: | AHG15: Further simplification over JVET-1034 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe de Lagrange, Fabrice Le Léannec, Edouard François, Karam Naser | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to remove the copy mode from JVET-1034 quantization matrix (QM) coding method, keeping only prediction mode, where predicting from default all-16 matrix is said to provide the same functionality as copy mode. One syntax element is removed compared to JVET-1034.

# Proposed syntax changes

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| for( scalingListId = 0; scalingListId < 28; scalingListId ++ ) { |  |
| matrixSize = (scalingListId < 2 ) ? 2 : (scalingListId < 8 ) ? 4 : 8 |  |
| scaling_list_copy_mode_flag [ scalingListId ] | u(1) |
| if( !scaling_list_copy_mode_flag [ scalingListId ] ) |  |
| scaling_list_prediction_mode_flag [ scalingListId ] | u(1) |
| if( ( scaling_list_copy_mode_flag [ scalingListId ] ¦ ¦ scaling_list_predition_mode_flag [ scalingListId ] ) && scalingListId != 0 && scalingListId != 2 && scalingListId !=8 ) |  |
| scaling_list_pred_scaling_list_id_delta[ scalingListId ] | ue(v) |
| if( !scaling_list_copyprediction_mode_flag [ scalingListId ]){ |  |
| nextCoef = (scaling_list_predition_mode_flag[ scalingListId ]) ? 0 : 8 |  |
| if( scalingListId > 13 ) { |  |
| scaling_list_dc_coef [ scalingListId ] | se(v) |
| nextCoef = scaling_list_dc_coef [ scalingListId ] + nextCoef |  |
| } |  |
| for( i = 0; i < matrixSize * matrixSize; i++ ) { |  |
| x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ] |  |
| y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ] |  |
| if ( !( matrixId > 25 && x >= 4 && y >= 4) ) { |  |
| scaling_list_delta_coef | se(v) |
| nextCoef = ( nextCoef + scaling_list_delta_coef + 256 ) % 256 |  |
| ScalingList[ scalingListId ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).