| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P1037 |
| --- | --- |

| Title: | Report on HDR Viewing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall and Kenneth Andersson | Email: | asegall@sharplabs.com kenneth.r.andersson@ericsson.com |
| Source: |  |  |  |

_____________________________

# Abstract

This report summarizes the HDR viewing that was performed at the 16th meeting. Preparations for the viewing occurred on October 7th and 8th, and the informal viewing occurred on October 9th.

# Viewing Session Configuration

The viewing was performed with a SONY BVM X300 monitor at the EBU. The monitor was configured to use the PQ (ST 2084) transfer function and BT.2020 color space. The monitor was driven using an SDI interface with a 10-bit input, 1920x1080/50p resolution and 4:2:2 color sampling format.  The monitor applied internal upsampling to convert the input format to the native display resolution of 4096x2160 pixels.

# Content Preparation

Content was converted from YUV to TIF (16-bit RGB) using HDRTools.  During the conversion, the content was tone mapped by applying a linear scaling operation that mapped 4,000 cd/m2 to 1,000 cd/m2.  The data was then loaded into a DVS Clipster and rendered to a DPX container.  The DPX files were then used for playback to the monitor.

The monitor indicated that regions of the content exceeded the 1,000 cd/m2 target.  Visual evaluation showed that one sequence (ShowGirl) appeared to have significant clipping, and that sequence was removed from the test.

Due to time constraints, all sequences were played back at 50Hz and frame rate conversion was not employed.  This resulted in one 25Hz sequence (Balloon) being played back at 50Hz.

The content was played in an “A-B” format for each sequence and QP point.  Viewers were told the order of the anchor and test sequences.

# HDR Viewing Planning

The following tests were identified to be performed.

- VTM6 LMCS on versus VTM6.0 anchor

- VTM6 Anchor versus VTM5 anchor (in terms of chroma quality)

- VTM6 Anchor versus HM16.18

- JVET-P0335: Proposed versus VTM6

- JVET-P0411: Proposed versus VTM6

# Session Design

The design of the tests are as listed below. Tests were limited to HD resolution and the PQ transfer function due to not reaching being able to achieve full frame rate for 4K resolution with the HLG transfer function with the content preparation approach described above.  This meant that the JVET-P0411 viewing was not performed since it included one sequence with HLG transfer function.

Session 1. LMCS on versus VTM6.0 anchor

The following sequences and QP points will be tested:

| Num | Name | fps | Frames | Bit-depth | TF | QP |
| --- | --- | --- | --- | --- | --- | --- |
| HDR1 | Market | 50 | 400 | 10 | PQ | 32,37 |
| HDR2 | Show Girl | 25 | 339 | 10 | PQ | 32,37 |
| HDR3 | Starting | 50 | 500 | 10 | PQ | 32,37 |
| HDR4 | Hurdles | 50 | 500 | 10 | PQ | 32,37 |
| HDR5 | BalloonFestival | 24 | 240 | 10 | PQ | 32,37 |

Session 2. VTM 6.0 anchor versus VTM 5.0 anchor

The following sequences and QP points will be tested:

| Num | Name | fps | Frames | Bit-depth | TF | QP |
| --- | --- | --- | --- | --- | --- | --- |
| HDR1 | Market | 50 | 400 | 10 | PQ | 32,37 |
| HDR2 | Show Girl | 25 | 339 | 10 | PQ | 32,37 |
| HDR3 | Starting | 50 | 500 | 10 | PQ | 32,37 |
| HDR4 | Hurdles | 50 | 500 | 10 | PQ | 32,37 |
| HDR5 | BalloonFestival | 24 | 240 | 10 | PQ | 32,37 |

Session 3. VTM 6.0 anchor versus HM 16.18 anchor

The following sequences and QP points will be tested:

| Num | Name | fps | Frames | Bit-depth | TF | QP |
| --- | --- | --- | --- | --- | --- | --- |
| HDR1 | Market | 50 | 400 | 10 | PQ | 32,37 |
| HDR2 | Show Girl | 25 | 339 | 10 | PQ | 32,37 |
| HDR3 | Starting | 50 | 500 | 10 | PQ | 32,37 |
| HDR4 | Hurdles | 50 | 500 | 10 | PQ | 32,37 |
| HDR5 | BalloonFestival | 24 | 240 | 10 | PQ | 32,37 |

Session 4. JVET-P0335 versus VTM6.0 anchor

The following sequences and QP points will be tested:

| Num | Name | fps | Frames | Bit-depth | TF | QP |
| --- | --- | --- | --- | --- | --- | --- |
| HDR1 | Market | 50 | 400 | 10 | PQ | 32,37 |
| HDR2 | Show Girl | 25 | 339 | 10 | PQ | 32,37 |
| HDR3 | Starting | 50 | 500 | 10 | PQ | 32,37 |
| HDR4 | Hurdles | 50 | 500 | 10 | PQ | 32,37 |
| HDR5 | BalloonFestival | 24 | 240 | 10 | PQ | 32,37 |

# Informal Viewing Comments

Informal viewing was performed at the EBU on October 9th from 3-4PM. The following comments were made by the participants:

- VTM6 LMCS on versus VTM6.0 anchor

- VTM6 Anchor versus VTM5 anchor (in terms of chroma quality)

- VTM6 Anchor versus HM16.18

- JVET-P0335: Proposed versus VTM6

First session JVET-P0335 versus VTM6 anchor:

Comments: Some differences but overall very similar.

Second session VTM6 with LMCS on versus VTM6 with LMCS off:

Comments: Some differences but overall very similar. One participant noted improvements of rope at QP 32 for Balloon for LMCS.

Third session VTM6 vs VTM5:

Comments: Some differences but overall very similar.  One participant noted improvements of rope at QP 32 for Balloon for VTM5.

Additionally, an informal viewing of VTM6 versus HM 16.18 was performed.  In this viewing, the bit-streams were created using the same QP parameter.  This resulted in bit-streams of different bit-rates and of similar (but not the same) objective quality.

Demo session of HEVC vs VVC on request by viewers:

Comments: Significant improvement.

# Acknowledgements

The authors would like to thank the EBU, Frans de Jong, and Paolo Sunna for their helpfulness and support in using the EBU equipment.

The authors would like to thank Alexis Tourapis for his support and guidance on using HDRTools.