| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P2022-v3 |
| --- | --- |

| Title: | Description of Core Experiment 2 (CE2): Palette Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Yung-Hsuan Chao Yu-Chen Sun Jizheng Xu | Tel: Email: | xiaozhongxu@tencent.com yunghsua@qti.qualcomm.com yuchen.s@alibaba-inc.com xujizheng@bytedance.com |
| Source: | CE coordinators |  |  |

_____________________________

Abstract

This document defines Core Experiment 2 (CE2) on palette mode coding, to be performed for the upcoming Jan. 2020 JVET meeting.

- Introduction

For the 16th JVET meeting in Geneva, various methods for palette mode have been proposed, which can be summarized as follows:

- Palette mode escape value coding, according to JVET-P0399, JVET-P0487, JVET-JVET-0529.

- Palette mode predictor reset at CTU row, according to JVET-P0424, JVET-P0573.

- Palette mode simplification for small blocks, according to JVET-P0475.

Accordingly, three sub-tests and one combination test are included in this CE.

- Tools under test

The following tools will be considered in this CE.

| Test | Tester | Document | Tool description |
| --- | --- | --- | --- |
| CE2-1.1 | W. Zhu (ByteDance) | JVET-P0399 | Fixed-length binarization for escape coding simplification. |
| CE2-1.2 | H.-J. Jhu (Kwai) | JVET-P0529 | Escape value is binarized by fixed-length code with a length determine by maximum quantized value no change on escape de-quantization |
| CE2-1.3 | H.-J. Jhu (Kwai) S. Yoo (LGE) | JVET-P0529 JVET-P0487 | Truncated binary code is used for escape value binarization with a length determine by maximum quantized value |
| CE2-2.1 | W. Zhu (ByteDance) H. Jang (LGE) | JVET-P0424, JVET-P0573 | Resetting predictor palette at CTU row |
| CE2-3.1a | Y.-H. Chao (Qualcomm) | JVET-P0475 | Simplified palette predictor update process for small blocks: restrict entries allowed for prediction to the first W x H entries |
| CE2-3.1b | Y.-H. Chao | JVET-P0475 | Simplified palette predictor update process for small blocks: restrict entries allowed for prediction to the first (W x H ) / 2 entries |
| CE2-3.1c | Y.-H. Chao | JVET-P0475 | Simplified palette predictor update process for small blocks: Bypass predictor update for CU size < 8x8 |
| CE2-3.1d | Y.-H. Chao | JVET-P0475 | Simplified palette predictor update process for small blocks: Bypass predictor update for CU size <= 8x8 |

The detailed description of each tools is provided in the next section.

- Description of CE tools

  - JVET-P0399

In this test, escape sample binarization is simplified by using fixed length coding derived from the QP associated with the current block; and the dequantization is also simplified by applying corresponding left shift.

  - JVET-P0529

QP dependent fixed-length binarization for escape coding

In this test, a new binarization method for palette escape coding is tested. The binarization of quantized escape samples is adjusted based on the dynamic range of the quantized values. The fix-length binarization process is used for escape samples. The length of fix-length binarization process is derived as follows:

length = max (1, bitDepth – (max (QpPrimeTsMin, Qp) - 4)/6).

  - JVET-P0529/JVET-P0487

In this test, a new binarization method for palette escape coding is tested. The binarization of quantized escape samples is adjusted based on the dynamic range of the quantized values. The truncated binarization process is used for escape samples.

  - JVET-P0424/JVET-P0573

Predictor palette is reset at the beginning of decoding each CTU row to enable parallel processing. The setting of using multiple tile columns are used to test to evaluate the performance changes caused by resetting. Furthermore, a picture is divided into 4 tile columns.

  - JVET-P0475

Two methods are tested to simplify the palette predictor update for small coding units in order to solve the issue of pipeline latencyIn the first method, the elements in the palette predictor that are allowed to be used in predicting palette table is restricted to the first W x H elements (W and H stands for width and height of the CU). In the second method, palette predictor update is bypasses for CU size < T.

Subtests:

- Restrict the entries in the palette predictor that are allowed to be used in prediction to the first W x H elements

- Restrict the entries in the palette predictor that are allowed to be used in prediction to the first ( W x H ) / 2 elements

- Palette predictor update is bypassed for CU size (W x H ) < 64

- Palette predictor update is bypassed for CU size (W x H ) <= 64

- CE tests

The following tests will be evaluated and studied in this CE.

| Test | Tester | Document | Tool description |  | Cross checker |
| --- | --- | --- | --- | --- | --- |
| CE2-1.1 | W. Zhu (ByteDance) | JVET-P0399 | QP dependent fixed-length binarization for escape coding |  | H. Yang (InterDigital) |
| CE2-1.2 | H.-J. Jhu (Kwai) | JVET-P0529 | Escape value is binarized by fixed-length code with a length determine by maximum quantized value no change on escape de-quantization |  | Y.-H. Chao (Qualcomm) |
| CE2-1.3 | H.-J. Jhu (Kwai) S. Yoo (LGE) | JVET-P0529 JVET-P0487 | Escape value is binarized by truncated binary code with a length determine by maximum quantized value |  | Y.-H. Chao (Qualcomm) |
| CE2-2.1 | W. Zhu (ByteDance) H. Jang (LGE) | JVET-P0424, JVET-P0573 | Resetting predictor palette at CTU row |  | M.G. Sarwer (Alibaba) |
| CE2-3.1a | Y.-H. Chao (Qualcomm) | JVET-P0475 | Simplified palette predictor update process for small blocks: restrict entries allowed for prediction to the first W x H entries |  | H.-J. Jhu (Kwai) |
| CE2-3.1b | Y.-H. Chao | JVET-P0475 | Simplified palette predictor update process for small blocks: restrict entries allowed for prediction to the first (W x H ) / 2 entries | H.-J. Jhu (Kwai) |
| CE2-3.1c | Y.-H. Chao | JVET-P0475 | Simplified palette predictor update process for small blocks: Bypass predictor update for CU size < 8x8 | H.-J. Jhu (Kwai) |
| CE2-3.1d | Y.-H. Chao | JVET-P0475 | Simplified palette predictor update process for small blocks: Bypass predictor update for CU size <= 8x8 | H.-J. Jhu (Kwai) |

  - Software

It is agreed that VTM version VTM-7.0 shall be used for these experiments, as specified in the corresponding output document from this meeting. The following anchors are considered for tests in this CE:

- The proposed methods in this CE shall be tested for Standard QPs (37, 32, 27, 22) of AI, RA, and LDB configurations according to the test condition listed below. IBC, PLT, BDPCM and HashME are enabled.

- CE tests are required for YUV4:4:4 format, with AI, RA, and LDB configurations. The test sequences used for the tests are listed later in this document. IBC, PLT, BDPCM and HashME shall be enabled in the anchor.

- For YUV4:4:4 sequences, tests for both dual-tree enabled (--DualITree=1) and disabled (--DualITree=0) are required.

- For CE2-2.1, tests for CTC condition are required except that one picture is divided into 4 tile columns

  - Test sequences and configurations

The mandatory common test conditions (CTC) as specified in the JVET-O2010 document shall be followed to perform the simulations in this CE. IBC, PLT, BDPCM (=2) and HashME shall be enabled.

For test on YUV4:4:4 sequences, results with both dual tree disabled (--DualITree=0) and enabled (--DualITree=1) should be provided.

The following test sequences in YUV4:4:4 format (based on JCTVC-U1015 + AOV4:4:4 + GlassHalf4:4:4) shall be used when providing results for YUV4:4:4 tests. IBC, PLT, BDPCM and HashME are enabled for all YUV4:4:4 sequences.

| Category | Sequence name | Resolution | Bit depth | Frame rate | Frames to be encoded |
| --- | --- | --- | --- | --- | --- |
| TGM | sc_flyingGraphics_1920x1080_60_8bit* sc_desktop_1920x1080_60_8bit sc_console_1920x1080_60_8bit ChineseEditing_1920x1080_60_8bit | 1920x1080 1920x1080 1920x1080 1920x1080 | 8 8 8 8 | 60 60 60 60 | 0-299* 0-599 0-599 0-599 |
| TGM | sc_web_browsing_1280x720_30_8bit sc_map_1280x720_60_8bit sc_programming_1280x720_60_8bit sc_SlideShow_1280x720_20_8bit | 1280x720 1280x720 1280x720 1280x720 | 8 8 8 8 | 30 60 60 20 | 0-299 0-599 0-599 0-499 |
| A | ArenaOfValor_1920x1080_60_8bit_444 sc_robot_1280x720_30_8bit Glasshalf_3840x2160_24p_10b_444 | 1920x1080 1280x720 3840x2160 | 8 8 10 | 60 30 24 | 0-599 0-299 0-239 |
| M | Basketball_Screen_2560x1440_60p_8b444* MissionControlClip2_2560x1440_60p_8444* MissionControlClip3_1920x1080_60p_8b444 | 2560x1440 2560x1440 1920x1080 | 8 8 8 | 60 60 60 | 322-621* 120-419* 0-599 |
| CC | EBURainFruits_1920x1080_50_10bit* Kimono1_1920x1080_24_10bit* | 1920x1080 1920x1080 | 10 10 | 50 24 | 0-249* 0-119* |
| *Note that the frames used are different from the original number of frames in the sequence TGM: Text and graphics with motion; M: mixed content; A: animation; CC: camera-captured content |  |  |  |  |  |

YUV4:4:4 Test sequences are available from the following locations (please contact JCT-VC/JVET chairs for login information):

- ArenaOfValor YUV4:4:4 and

- GlassHalf YUV4:4:4: ftp.ient.rwth-aachen.de.

- Other YUV4:4:4 sequences:  ftp://hevc@ftp.tnt.uni-hannover.de/testsequences/FrExt-candidate-sequences/upload/screen_content/ScExt-TestSequences

When the random access configuration is used, the period for intra random access frames for each test sequence needs to be specified, as follows:

IntraPeriod: Specifies the intra refresh period in the random access configuration. The intra refresh period is dependent on the frame rate of the test sequence.  A value 16 shall be used for test sequences with a frame rate equal to 20fps, 32 for 24fps, 32 for 30fps, 48 for 50fps, and 64 for 60fps.

For tests in the CE2-1.x category, low QP (2, 7, 12, 17) results shall also be provided, wherein the 1st 100 frames with the same sequences, intra period and starting frame number are applied.

  - Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-rate).

  - Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the default configuration).

- Decoding time (compared to the default configuration).

- Timeline

T0: Release of VTM software: Nov. 13, 2019

T1: Final CE description upload: Nov. 1, 2019

T2: Release of individual test software and crosscheck begins, T2 = T0 + 2 week: Nov. 27, 2019

T3: CE software frozen: Dec. 17, 2019 (T2+1week, or 3 weeks before next JVET meeting, whichever is later)

T4: CE proposal document uploaded (regular document upload deadline – 1 week), Dec. 24, 2019

T5: CE summary report and regular document upload deadline, Dec. 31, 2019

Note: The 17th JVET meeting will be from Tue. 7 – Fri. 17 January 2020 in Brussels, BE.

- Participant list

- References

- JVET-P0399

- JVET-P0529

- JVET-P0487

- JVET-P0081

- JVET-P0166

- JVET-P0475