| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P2024-v2 |
| --- | --- |

| Title: | Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen Ru-Ling Liao Xiaoyu Xiu Haitao Yang | Email: | chunchic@qti.qualcomm.com ruling.lrl@alibaba-inc.com xiaoyuxiu@kwai.com haitao.yang@huawei.com |
| Source: | CE4 coordinators |  |  |

_____________________________

Abstract

This Core Experiment is set up to investigate several techniques on inter prediction with geometric partitioning.

Based on the discussion in 16th JVET meeting, the technique proposed in JVET-P0884/P0885 is suggested to be the common base for this core experiment. The common base will be implemented on top of VTM-7.0, and will be compared against VTM-7.0 with TPM on and off, respectively.

All other techniques to be tested will be implemented on top of the common base and compared against the common base.

Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| Common base | Geometric partitioning for merge mode as in JVET-P0884&JVET-P0885 | JVET-P0884 JVET-P0885 | H. Gao (Huawei) R. - L. Liao (Alibaba) K. Reuzé (Qualcomm) M. Blaser (RWTH Aachen) |  |
| 4-1 | Common base simplification by reducing the number of partition mode to e.g. 64 | JVET-P0884 JVET-P0885 | H. Gao (Huawei) R. - L. Liao (Alibaba) K. Reuzé (Qualcomm) M. Blaser (RWTH Aachen) |  |
| 4-2.1 | Block size restriction of GEO, disable GEO for block size greater than 64×64 | JVET-P0663 | Z. Deng (Bytedance) |  |
| 4-2.2 | Block size restriction of GEO, disable GEO for block size greater than 32×32 | JVET-P0663 | Z. Deng (Bytedance) |  |
| 4-3.1 | Disallow SBT for GEO mode | JVET-P058 JVET-P0248 | H. Huang (Qualcomm) L. Xu (Hikvision) |  |
| 4-3.2 | Disallow SBT for GEO mode when displacement is 0 | JVET-P058 JVET-P0248 | H. Huang (Qualcomm) L. Xu (Hikvision) |  |
| 4-4 | Adaptive blending for screen content | JVET-P0069 | H. Gao (Huawei) M. Blaser (RWTH Aachen) |  |
| 4-5 | GEO with a single mode for 4xN and Nx4 CU | JVET-P0449 | K. Panusopone (Nokia) |  |
| 4-6 | Combination of geometric partitioning and CIIP | JVET-P0449 | S. Blasi (BBC) |  |

Detailed description of tests

- Common base (JVET-P0884/JVET-P0885)

The common base of test CE4-1 is defined as the method descripted in JVET-P0884&JVET-P0885. The following modifications are applied based on the JVET group discussion.

- Turn off TPM mode

- Add TPM angles back into GEO (82 modes in total)

In JVET-P0884&JVET-P0885, the following two simplifications are applied on top of the JVET-P0068 (CE4-1.14 of 16th JVET Meeting in Geneva, CH):

1. The angle of GEO is quantized based on tangent power of 2 slope in order to simplify the on-the-fly computation of weight and motion storage masks. The possible slopes are {¼, ½, 1 2, 4} with horizontal and vertical angles. With the slope based GEO, the computation complexity of geo blending mask derivation is considered as multiplication (up to 2 bits shift) and addition. Besides, the weight values for these angles can be implemented by repeating row-by-row or column-by-column with phase shift. With the proposed angles, one row or column is needed to store per block size and per partition mode. Details of this aspect are described in JVET-P0107 and JVET-P0264

2. OffsetX and OffsetY are pre-calculated at CU level to replace the original rho in JVET-P0068 and applied to the position coordinates. This allows masks of different sizes to be cropped directly from the masks of the largest size, and therefore reduces the storage requirement for weights and motion masks. There are totally 24 masks in each set (one for weights and the other for motion field storage). Each mask per angle is calculated using the same equations in GEO with block width and block height set to 256 and displacement set to 0. Details of this aspect are described in JVET-P0304

In summary, the sampleWeight (for luma) for GEO are derived with the following equations:

weightIdx = (((x + offsetX)<<1) + 1)*Dis[displacementX] + (((y+ offsetY )<<1) + 1))*Dis[displacementY] − rho.																	(1)

weightIdxAbs = Clip3(0, 26, abs(weightIdx)).							(2)

sampleWeight = weightIdx <= 0 ? GeoFilter[weightIdxAbs] : 8 − GeoFilter[weightIdxAbs]	(3)

In Eq. (1), Dis[.] is a look up table with 24 entries and the possible out put values are {0, 1, 2, 4}; rho, offsetX and offsetY are pre-computed based on:

rho = (Dis[displacementX]<< 8) + (Dis[displacementY] << 8)

offsetX=(M-W)≫1φ % 1216 = 68 or (φ % 1216 ≠ 0 and H ≥ W)M-W≫1+φ<1216 ?ρ×W≫3 :-(ρ×W≫3)otherwise

offsetY=M-H≫1+φ<1216 ?ρ×H≫3 :-(ρ×H≫3)φ % 1216 = 68 or (φ % 1216 ≠ 0 and H ≥ W)M-H≫1otherwise

In Eq. (3), GeoFilter[.] is a look up table with 27 entries.

The chroma sampleWeights are subsampled (top-left) from the luma sampleWeights.

The motion storage masks are derived using same method of weights derivation independently.

The minimum allowed block width/height for GEO is 8 luma samples.

- Test 4-1 Reduce the number of partition modes (JVET-P0884/JVET-P0885)

In test 4-1, the GEO of common base is reduced from 82 modes to 64 modes. i.e. 20 angles and 4 distances with removing overlapped partition of regular splitting (binary and ternary tree split).

- Test 4-2 Block size restriction of GEO (JVET-P0663)

In test 4-2.1, on top of GEO common base, GEO mode is always disabled for block size greater than 64×64.

In test 4-2.2, on top of GEO common base, GEO mode is always disabled for block size greater than 32×32.

- Test 4-3 Disallow SBT for GEO mode (JVET-P0583/JVET-P0248)

In test 4-3.1, on top of GEO common base, the SBT is disabled for GEO mode.

In test 4-3.2, on top of GEO common base, the SBT is disabled for GEO mode when displacement is 0.

- Test 4-4 Adaptive blending for screen content (JVET-P0069)

In test 4-4 the blending process is turned off for SCC in order to no over smoothing the sharp edges of screen contents. A SPS-level control flag and a picture header blending-off flag is introduced to control the blending process on and off.

The test 4-4 should be tested for TGM (420) with IBC, HashME and BDPCM on, and for 444 contents with CE8 test condition.

- Test 4-5 Single mode partition for 4xN and Nx4 CU (JVET-P0449)

In test 4-5, the GEO of common base is extended to support 4xN and Nx4 CU. For these CU dimensions, GEO skips wedge_partition_idx syntax, and allows one partitioning angle (either horizontal or vertical) and one distance. For 4xN CU, GEO employs vertical angle blending weights. For Nx4 CU, GEO employs horizontal angle blending weights. Motion vector prediction and blending operation of GEO with a single mode for 4xN and Nx4 CU are aligned with the GEO of common base.

- Test 4-6 Combination of geometric partitioning and CIIP (JVET-P0071)

An intra predictor and an inter predictor are computed for a block. The inter predictor and the intra predictor are combined using the geometric partitioning scheme as in the common base. The intra predictor is used in one of the two GEO partitions, and the inter predictor is used in the other GEO partition. The blending scheme as used in GEO is used to blend the two GEO partitions.

Test condition and evaluation criteria

The common base is requested to be compared against both VTM-7.0 and VTM-7.0 with TPM off, evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in JVET-N1010.

All tests except the common base is requested to be compared with the common base.

Simulation with RA and LDB configurations are mandatory.

Crosscheck report

Crosscheckers shall fill in the table below, and send it to CE coordinators 3 days before T4.

| Test# | Crosschecker | SW match CE description (Y/N) | Text match SW (Y/N) | Simulation results (Y/N) | EncT/DecT match (Y/N) |
| --- | --- | --- | --- | --- | --- |

Explanation of each item in the table

- SW match CE description: Y SW implementation match CE description, N mismatch.

- Text match SW: Y text match SW, N mismatch.

- Simulation results: Y substantial results (all but class A for RA, all but class B for LDB) obtained and match results from tester, N mismatch or no substantial results.

- EncT/DecT match: Y encoder and decoder run time match those from tester, N mismatch.

SW Integration guideline

The software implementation of the common base (JVET-P0884/P0885) will be implemented on top of VTM-7.0. All tests except for the common base will be implemented on top of the common base.

Timeline

| T1 | 1 Nov., 2019 | Finalize CE description (3 weeks after the meeting) |
| --- | --- | --- |
| T2 | 2 weeks after VTM-7 release | Deliver software package for the common base |
| T3 | 2 weeks after VTM-7 release | Deliver software packages for all tests and begin cross-checking |
| T4 | 17 Dec., 2019 | Freeze software revision and provide full results for proponents Begin final crosschecking for cross-checkers |
| T5 | 24 Dec., 2019 | Upload contribution documents with specification text and complete test results (regular document deadline – 1 week) |