| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 16th Meeting: Geneva, CH, 1–11 October 2019 | Document: JVET-P2025 |
| --- | --- |

| Title: | Description of Core Experiment 5 (CE5): Cross Component Adaptative Loop Filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Ching-Yeh Chen Andrew Segall | Tel: Email: | chingyeh.chen@mediatek.com asegall@sharplabs.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to improve and/or enhance cross component loop filtering (CC-ALF) techniques.  These CC-ALF techniques will be evaluated according to both subjective and objective metrics, and testing will be organized into sub-tests to address different aspects of the CC-ALF design.

Evaluation will use the coding conditions defined in the JVET Common Test Conditions (CTC).  These are defined in JVET-P2010 and JVET-P2011, respectively.  If required, additional sequences may be identified and coded with different QPs as needed for subjective viewing.

# Participants

| Nr. | Name | Company | Email |
| --- | --- | --- | --- |

# Test conditions and evaluation criteria

The proposals in the CE will be evaluated relative to a “CE Anchor”.  The software for this anchor will correspond to the VTM-7.0 combined with JVET-P1008 (Option 1) and will be provided as part of the core experiment process.  Furthermore, an evaluation of the CE Anchor compared to VTM-7.0 will also be performed.

Evaluation of the proposals will be performed with respect to subjective quality, objective quality and complexity.

# Tool descriptions

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE5-3.1.1 | Nan Hu nanh@qti.qualcomm.com Xiang Li xlxiangli@tencent.com JVET-P0557, JVET-P0666 |  |
| CE5-3.1.2 | Xiang Li xlxiangli@tencent.com Nan Hu nanh@qti.qualcomm.com JVET-P0557, JVET-P0666 |  |
| CE5-3.2 | Nan Hu nanh@qti.qualcomm.com JVET-P0558 |  |
| CE5-3.3 | Yin Zhao yin.zhao@huawei.com JVET-P0251 |  |
| CE5-1 | Anand Meher Kotra Anand.meher.kotra@huawei.com JVET-P0106 |  |
| CE5-5 | Hua Yang Hua.Yang@InterDigital.com JVET-P0372 |  |

## CE5-1: Simplifications for Cross Component Adaptive Loop Filter (JVET-P0106), A. M. Kotra, anand.meher.kotra@huawei.com

The original filter shape proposed in CE5-2.1 (JVET-P0080) is modified to have only 8 coefficients as shown in the figure below. This modified filter shape is proposed to be tested with sequences containing different Chroma Type.

## CE5-2: Cross component ALF simplification (JVET-P0173), J. Li, jingya.li@sg.panasonic.com

## CE5-3: Simplified CCALF with 6 filter coefficients (JVET-P0251), Y. Zhao, yin.zhao@huawei.com

To further reduce the complexity and align number of multiplication of CC-ALF with chroma ALF, a six-tap filter was proposed, as shown in Fig. X. The six taps are a sub-set of the eight taps of JVET-P1008, and each tap is associated with a unique filter coefficient.

Fig. X Six-tap CC-ALF

## CE5-5: Joint chroma cross-component adaptive loop filtering (JVET-P0372), H. Yang, Y. He, H. Li, Hua.Yang@InterDigital.com

Joint Chroma Cross-Component Adaptive Loop Filter (JC-CCALF) uses only one set of CCALF filter coefficients trained at the encoder to generate one filtered output as the refinement signal, which will be added directly to the Cb component, and be properly weighted and then added to the Cr component.

SAO Luma

ALF Luma

SAO Cb

SAO Cr

Joint chroma

CC ALF

ALF Cb

Y

Cb

Cr

w

ALF Cr

Fig. X. JC-CCALF workflow

## CE5-6: Multiplication removal for cross component adaptive loop filter (JVET-P0557), N. Hu, nanh@qti.qualcomm.com

### CE5-6.1

Filter coefficients are restricted to have values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}, which are signalled as {-7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7} respectively before binarization in order to reduce the signalling overhead. Fixed length codes of 3 bits are used to signal the absolute values of the coefficients. Another sign bit is signalled for non-zero coefficients.

### CE5-6.2

Filter coefficients are restricted to have values: {-8, -4, -2, -1, 0, 1, 2, 4, 8}. A bit is first signalled to indicate whether it is a zero coefficient. If not, a fixed length code of 2 bits is used to signal the absolute value of the coefficient followed by a sign bit.

## CE5-7: Reduced filter shape for cross component adaptive loop filter (JVET-P0558), N. Hu, nanh@qti.qualcomm.com

Alternative filter shape with multiplication removal for CCALF

|  |  | f0 |  |  |
| --- | --- | --- | --- | --- |
|  | f1 | f2 | f3 |  |
| f4 | f5 | f6 | f7 | f8 |
|  | f9 | f10 | f11 |  |
|  |  | f12 |  |  |

The 5x5 diamond filter shape will be used for CCALF on top of the tests in CE5-3.1. Filter coefficients are restricted to have values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}, which are signalled as {-7, -6, -5, -4, -3, -2, -1, 0, 1, 2, 3, 4, 5, 6, 7} respectively before binarization in order to reduce the signalling overhead. Fixed length codes of 3 bits are used to signal the absolute values of the coefficients. Another sign bit is signalled for non-zero coefficients.

## CE5-8 Non-CE5: Combination of JVET-P0470 and JVET-P0557 on simplification of CC-ALF (JVET-P0666) X. Li, xlxiangli@tencent.com

# Expected Gains

- Coding efficiency improvement

- Subjective picture quality preservation

# Evaluation Criteria

1. BD-rate comparison relative to the CE Anchor.

2. Subjective performance relative to the CE Anchor.  Note that any subjective test will use 10 second versions of the sequences, and the proponents shall provide the bitstreams and decoders for the sequences that are chosen for evaluation in the subjective test.

3. Complexity performance relative to the CE Anchor, including:

- Decoding time versus the CE Anchor.  Decoding time shall be measured on the same computer without writing output pictures to disk

- Encoding time versus the CE Anchor.  Encoding times measured using cluster times are acceptable.

- Number of multiplications per luma sample for the tested CC-ALF process

# Subjective test(s)

Note: The subjective test plan below is a draft and subject to change.  It is currently aligned with previous CEs and may not be appropriate.

Expert viewing at the Brussels meeting may be performed for all proposals in the CE.

The following test sequences and conditions will be used in the subjective tests. The sequence length is 10 seconds. This means that the versions of some sequences encoded for the subjective test are longer than those used in the CTC [1].

The subjective test sequences shall be provided by XX 2019 (uploaded to the JVET ftp folder /ce/JVET-P2025_loop_filters), for CE5-1 coding configuration with ALF ON and ALF OFF shall be used. For CE5-2 and CE5-3 coding configuration with ALF ON shall be used. The sequences listed in this section shall also be available at the Geneva meeting from the proponents for their corresponding sub-CEs (CE5-1, CE5-2, CE5-3).

Anchors will be provided by volunteers.

The following SDR sequences and QP will be used for subjective evaluation in CE5-1 sub-category. Test conditions includes both verification of not harming visual quality for very difficult cases for deblocking and verification of subjective benefit for cases with problems that the proposal asserts to solve:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| SDR_01 | FoodMarket | 3840x2160p | 60fps | 600 | RA | 37, 32 |
| SDR_02 | Campfire | 3840x2160p | 30fps | 300 | RA | 37, 32 |
| SDR_03 | Red Kayak* | 1920x1080p | 30fps | 300 | RA | 37, 32 |
| SDR_04 | Cactus | 1920x1080p | 50fps | 500 | LDB | 37, 32 |
| SDR_05 | KristenAndSara | 1280x720p | 60fps | 600 | LDP | 37, 32 |

* First 300 frames of the RedKayak sequence shall be used.

The following SDR and HDR sequences will be used for verification of subjective benefit for cases with problems that the proposal asserts to solve:

| Num | Name | Resolution | fps | Frames | Mode | QP |
| --- | --- | --- | --- | --- | --- | --- |
| HDR_01 | EBU_Hurdles | 1920x1080p | 50fps | 500 | RA | 37, 32 |
| HDR_02 | EBU_Starting | 1920x1080p | 50fps | 500 | RA | 37, 32 |
| HDR_03 | DayStreet | 3840x2160p | 60fps | 600 | RA | 37, 32 |
| SDR_06 | FourPeople | 1280x720p | 60fps | 600 | LDB | 37, 32 |
| SDR_07 | Johnny | 1280x720p | 60fps | 600 | LDB | 37, 32 |

# Test conditions and data to be provided

The test conditions are according to CTC on VTM with ALF turned on and VTM with ALF turned off for CE5-1. The test conditions are according to CTC shall be provided. In addition, the proponents of these sub-tests need to provide results for additional QP and additional test sequences chosen by the CE (see above). The sequences length for the subjective viewing is 10 seconds. The complexity of the proposals according to Section 7 should be reported by the proponents.

# Time-line and Responsibilities

T1: 2019-Nov-X:				   			Final CE description uploaded

T2: VTM7.0 release + 1 week:				CE Anchor is released

T2: VTM7.0 release + 3 weeks:	   			Proponents provide software and cross-checking begins

T3: 2019-Dec-XX: 							Final version of proponent software is provided

T4: Document deadline – 1 w.: 				CE contribution documents including specification text and complete test results are uploaded to the JVET document repository. Sequences for the subjective test are uploaded to the ftp.

# References

- “JVET common test conditions and software reference configurations”, JVET-P2010.

- “Algorithm description for Versatile Video Coding and Test Model 5 (VTM6)”, JVET-P2002.