| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0060_v1 |
| --- | --- |

| Title: | CE4-4: Geometric Inter Prediction with Adaptive Blending for SCC |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Han Gao, Semih Esenlik, Elena Alshina, Anand Meher Kotra, Biao Wang (Huawei), Max Bläser, Johannes Sauer (RWTH Aachen) | Tel: Email: | han.gao@huawei.com semih.esenlik@huawei.com elena.alshina@huawei.com blaeser@ient.rwth-aachen.de |
| Source: | Huawei Technologies Co., LTD, RWTH Aachen University |  |  |

_____________________________

# Abstract

Adaptive blending off method is proposed for geometric inter prediction. In this proposed, the blending filter is turned off for screen content. This CE is implemented on top of CE4 common base. Experimental results shows:

Compared with CE4 common base: 0.19% RA and 0.20% LDB luma gain in Class F, 2.03% RA and 2.82% LDB luma gain in Class TGM(420), and 2.04% RA and 3.20% LDB luma gain in Class TGM(444)

Compared with VTM7.0: 0.51% RA and 0.70% LDB luma gain in Class F, 2.50% RA and 3.26% LDB luma gain in Class TGM(420), and 2.50% RA and 3.91% LDB luma gain in Class TGM(444)

Compared with VTM7.0 TpmOff: 0.76% RA and 1.14% LDB luma gain in Class F, 2.54% RA and 3.33% LDB luma gain in Class TGM(420), and 2.58% RA and 3.96% LDB luma gain in Class TGM(444)

# Introduction

Geometric inter prediction is firstly proposed as a merge mode extension in [1] of 15th JVET Meeting in Gothenburg, SE. It agreed that the combined test proposal of [2], [3] are used as common base of CE4 [4] of 16th JVET Meeting in Brussels, BE.

GEO with adaptive blending for SCC method is proposed in [5]. Since the blending filter may over smoothing the screen content which contains sharp blending edges, a blending off method or hard blending filter is designed for screen content.

# Proposed Method

Compared with CE4 common base, the following changes are applied.

## High level syntax changes

In order to turn off the blending operation of GEO, one sps level control flag and one slice level blending off enable flag is introduced, encoder set both flag as true for screen content.

7.3.2.3	Sequence parameter set RBSP syntax

…

| if( sps_triangle_enabled_flag ) |  |
| --- | --- |
| sps_blending_ctrl_flag | u(1) |

…

7.3.6.1	General slice header syntax

…

| if( sps_geo_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_geo_cand_plus1 ){ |  |
| --- | --- |
| pic_max_num_merge_cand_minus_max_num_geo_cand | ue(v) |
| if( sps_blending_ctrl_flag ) |  |
| slice_blending_off_flag | ue(v) |
| } |  |

## Adaptive blending off operation for GEO

The blending mask of GEO mode is derived using look-up table based on the distance between the sample position and the split line. In adaptive blending off operation, look up table is replaced with the fix weighting value 8 and 0 as:

if slice_blending_off_flag is equal to 0: wValue = weightIdx <= 0 ? GeoFilter[weightIdxAbs] : 8 − GeoFilter[weightIdxAbs] otherwiese (slice_blending_off_flag is equal to 1): wValue = weightIdx < 0 ? 8 : weightIdx >0 ? 0 : 8*( angleIdx >=10 && angleIdx <=20)

# Simulation Results

As the test condition descripted in [4], the proposed method is simulated for class F, class TGM(420) and 444 content with CE2 [5] test condition and compared with CE4 common base, VTM7.0 and VTM7.0 TpmOff in following Tables. Details results are in the attached excel sheets.

Table 1 CE4-4 Compared with CE4 common base

| Class | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| ClassF | -0.19% | -0.12% | -0.13% | 100% | 100% | -0.20% | -0.12% | 0.03% | 100% | 101% |
| TGM420 (1080p) | -2.03% | -1.52% | -1.46% | 99% | 99% | -2.82% | -2.19% | -2.10% | 99% | 100% |
| TGM444 (1080p) | -2.04% | -1.92% | -1.84% | 98% | 99% | -3.20% | -2.99% | -2.93% | 98% | 101% |

Table 2 CE4-4 Compared with VTM7.0

| Class | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| ClassF | -0.51% | -0.51% | -0.46% | 100% | 99% | -0.70% | -1.03% | -0.27% | 102% | 98% |
| TGM420 (1080p) | -2.50% | -2.19% | -2.12% | 99% | 99% | -3.26% | -2.95% | -2.94% | 101% | 99% |
| TGM444 (1080p) | -2.50% | -2.49% | -2.44% | 98% | 99% | -3.91% | -3.88% | -3.76% | 99% | 100% |

Table 3 CE4-4 Compared with VTM7.0 TPM off

| Class | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| ClassF | -0.76% | -0.92% | -0.85% | 102% | 100% | -1.14% | -1.10% | -1.43% | 105% | 99% |
| TGM420 (1080p) | -2.54% | -2.33% | -2.27% | 101% | 99% | -3.33% | -3.08% | -3.02% | 103% | 99% |
| TGM444 (1080p) | -2.58% | -2.67% | -2.63% | 100% | 99% | -3.96% | -4.03% | -3.97% | 102% | 100% |

# Conclusion

The proposed adaptive blending off method shows coding performance increase without additional encoder or decoding time increment. Compared the results in table 1 and table 2, it can be conclude that major coding gain for is from adaptive blending off (TPM is removed in common base). It’s proposed to be adopted in the next version of VVC draft.

# Reference

- S. Esenlik, H. Gao, A. Filippov, V. Rufitskiy, A. M. Kotra, B. Wang, E. Alshina, M. Bläser, J. Sauer  “Non-CE4: Geometrical partitioning for inter blocks”, JVET-O0489, Gothenburg, SE, 3–12 July 2019

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Blaser, J. Sauer, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning”, JVET-P0884, Geneva, CH, 1–11 October 2019

- K. Reuzé, C.-C Chen, H. Huang, W.-J Chien, V. Seregin, M. Karczewicz, R.-L Liao, J. Chen, Y. Ye, J. Luo, M. Blaser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304)”, JVET-P0885, Geneva, CH, 1–11 October 2019

- C.-C. Chen, R.-L. Liao, X. Xiu, H. Yang, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning”, JVET-P2024, Geneva, CH, 1–11 October 2019

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Blaser, J. Sauer, “CE4: CE4-1.7, CE4-1.8: GEO and TPM Blending Off for SCC”, JVET-P0069, Geneva, CH, 1–11 October 2019

- X. Xu, Y.-H. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 2 (CE2): Palette Mode Coding”, JVET-P2022, Geneva, CH, 1–11 October 2019

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

RWTH Aachen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, would be prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form), or make them available free of charge (per box 1 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form). We are however aware that also other entities may have current or pending patent rights relating to the technology described in this contribution.