| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0067_v2 |
| --- | --- |

| Title: | CE2-1.3: Truncated binarization of palette escape value |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang Sunmi Yoo, Jie Zhao, Junghak Nam, Jungah Choi, Seung Hwan Kim, Jaehyun Lim | Email: | {jhuhong-jheng, xiaoyuxiu, yiwenchen, tsung-chuanma, xianglinwang}@kwai.com sunmi.yoo@lge.com |
| Source: | Kwai Inc., LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the results of Core Experiment (CE) 2-1.3 [1]. In this test, a new binarization method for palette escape coding is tested. The binarization of quantized escape samples is adjusted based on the dynamic range of the quantized values. The truncated binarization process is used for escape samples. The proposed method is implemented and evaluated with VTM7[2] by following CE2 test conditions. Results are reported as below:

Dual tree OFF:

Standard QP (22, 27, 32, 37): -0.1% AI, -0.04% RA, -0.03% LB

Low QP (2, 7, 12, 17): -1.32% AI, -0.87% RA, -0.6% LB

Dual tree ON:

Standard QP (22, 27, 32, 37): -0.04% AI, 0.00% RA, -0.05% LB

Low QP (2, 7, 12, 17): -0.76% AI, -0.57% RA, -0.43% LB

Introduction

In current VVC [3], the palette coding is beneficial when there are limited pixels and the texture is homogeneous in a CU when the coded image is computer generated. If there is a pixel which has different values from other pixels, it is coded not as a regular palette entry, but an escape value. The sample values of each component of the escape value are coded and transmitted after quantized. Each escape sample value is binarized with EG3, which has the bin length from 4 to 18 when the sample value is 0 to (1 << bitdepth) – 1, where the bitdepth in the current VTM is set to 10. Then the binarized symbol is bypass coded. In the worst case, the escape value is coded with 54 bits, whereas it needs only 30 bits when the pixel is coded with FL coding. Therefore, the efficient escape value coding is provided in this proposal.

Proposal

In the proposed method, the escape value is binarized with TB, with a maximum value (cMax) equal to Q((1 << bitdepth) – 1), where Q(x) indicates quantization. If the Qp is equal to 4, the input and output value of Q(x) are the same. Once the bitdepth and the Qp are transmitted, the quantized cMax can be easily calculated, or it can be pre-calculated and referenced by being tabulated for each Qp (0 to 63). Moreover, when the cMax is not equal to power of 2, some values can be represented with the less bin length by 1 when using TB instead of FL. The bin length comparison between EG3 and TB for the regular Qps (22, 27, 32, 37) are shown in Table 1. Using TB can be more efficient since EG3 is biased to the low values.

Table 1 Required bin length according to Qps

| QP | Maximum value | Bin length after binarized with synVal equal to 0 to maximum value |  |
| --- | --- | --- | --- |
|  |  | EG3 | TB |
| 22 | 32 | 4~8 | 5~6 |
| 27 | 18 | 4~6 | 4~5 |
| 32 | 10 | 4~6 | 3~4 |
| 37 | 6 | 4~6 | 2~3 |

Simulation Results

The proposed methods were implemented based on the VTM-7.0 reference software [2] and tested using the test condition and evaluation criteria that are used in CE2 [1] are applied.

Table 4: Results of Test2 for standard QP (22, 27, 32, 37)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.10% | -0.07% | -0.05% | 97% | 99% | -0.24% | -0.22% | -0.16% | 98% | 98% |
| TGM | -0.02% | -0.08% | -0.05% | 97% | 96% | -0.07% | -0.03% | -0.05% | 97% | 96% |
| A | -0.03% | -0.03% | 0.00% | 98% | 95% | -0.04% | 0.01% | -0.02% | 97% | 98% |
| M | -0.03% | 0.01% | -0.02% | 98% | 97% | -0.06% | -0.02% | -0.02% | 97% | 96% |
| CC | 0.00% | -0.01% | 0.01% | 98% | 97% | 0.00% | 0.00% | 0.00% | 98% | 96% |
| Overall | -0.04% | -0.04% | -0.03% | 98% | 97% | -0.10% | -0.06% | -0.06% | 98% | 97% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.05% | -0.11% | -0.06% | 100% | 102% | -0.16% | 0.02% | -0.09% | 100% | 101% |
| TGM | 0.00% | 0.00% | 0.09% | 100% | 100% | -0.03% | 0.09% | 0.03% | 100% | 102% |
| A | 0.00% | 0.00% | 0.04% | 100% | 101% | 0.07% | 0.09% | 0.09% | 100% | 102% |
| M | 0.06% | 0.14% | 0.20% | 100% | 99% | -0.03% | 0.13% | 0.01% | 100% | 101% |
| CC | 0.01% | -0.03% | 0.02% | 100% | 102% | 0.00% | 0.00% | -0.01% | 100% | 100% |
| Overall | 0.00% | -0.01% | 0.05% | 100% | 101% | -0.04% | 0.07% | 0.00% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -0.09% | -0.13% | -0.13% | 102% | 103% | -0.03% | 0.00% | 0.03% | 101% | 103% |
| TGM | -0.21% | 0.07% | 0.38% | 100% | 103% | -0.21% | 0.02% | 0.00% | 100% | 101% |
| A | 0.03% | 0.16% | -0.09% | 103% | 105% | 0.07% | 0.16% | 0.06% | 102% | 104% |
| M | 0.08% | -0.03% | 0.33% | 101% | 104% | 0.09% | 0.41% | 0.43% | 101% | 101% |
| CC | 0.02% | -0.04% | -0.01% | 101% | 102% | -0.01% | 0.01% | 0.02% | 101% | 102% |
| Overall | -0.05% | 0.00% | 0.11% | 101% | 103% | -0.03% | 0.11% | 0.10% | 101% | 102% |

Table 5: Results of Test2 for low QP (2, 7, 12, 17)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -1.47% | -1.32% | -1.38% | 97% | 99% | -2.83% | -3.28% | -3.19% | 98% | 102% |
| TGM | -0.54% | -0.47% | -0.46% | 98% | 96% | -0.93% | -0.84% | -1.16% | 98% | 97% |
| A | -0.42% | -0.26% | -0.32% | 98% | 99% | -0.52% | -0.87% | -0.92% | 98% | 100% |
| M | -0.96% | -1.12% | -1.15% | 98% | 98% | -1.48% | -1.81% | -1.78% | 99% | 96% |
| CC | 0.00% | 0.00% | -0.02% | 97% | 99% | 0.00% | 0.01% | 0.00% | 98% | 98% |
| Overall | -0.76% | -0.70% | -0.74% | 97% | 98% | -1.32% | -1.53% | -1.60% | 98% | 99% |
|  | Random access Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -1.42% | -1.51% | -1.48% | 100% | 101% | -2.10% | -2.27% | -2.17% | 100% | 100% |
| TGM | -0.28% | -0.20% | -0.32% | 100% | 101% | -0.52% | -0.43% | -0.69% | 100% | 102% |
| A | -0.24% | -0.37% | -0.38% | 102% | 102% | -0.34% | -0.47% | -0.47% | 102% | 105% |
| M | -0.52% | 0.29% | 0.25% | 100% | 103% | -0.79% | -0.65% | -0.59% | 101% | 102% |
| CC | -0.01% | -0.01% | 0.04% | 103% | 105% | 0.00% | -0.01% | 0.01% | 104% | 105% |
| Overall | -0.57% | -0.44% | -0.47% | 101% | 102% | -0.87% | -0.88% | -0.91% | 101% | 103% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | DualTree = 1 |  |  |  |  | DualTree = 0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| TGM | -1.05% | -1.11% | -1.09% | 102% | 102% | -1.46% | -1.70% | -1.60% | 101% | 103% |
| TGM | -0.21% | -0.16% | -0.19% | 101% | 102% | -0.19% | -0.29% | -0.37% | 100% | 102% |
| A | -0.46% | -0.45% | -0.47% | 104% | 103% | -0.54% | -0.46% | -0.52% | 104% | 104% |
| M | -0.17% | 0.07% | 0.10% | 102% | 103% | -0.46% | -0.26% | -0.35% | 102% | 101% |
| CC | 0.00% | 0.01% | 0.01% | 103% | 99% | 0.01% | -0.01% | 0.01% | 103% | 98% |
| Overall | -0.43% | -0.39% | -0.39% | 102% | 102% | -0.60% | -0.64% | -0.65% | 102% | 102% |

References

- X. Xu, Y.-H. Chao, Y.-C. Sun and J. Xu, “ Description of Core Experiment 2 (CE2): Palette Mode Coding”, JVET-O2022, 16th meeting, Geneva, CH, 1–11 October 2019

- VTM-7.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0.

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vE, the 16th JVET meeting, Geneva, CH, 1–11 October 2019

Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).