| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0097-v1 |
| --- | --- |

| Title: | CE5-1.2: Cross-component ALF simplification with 6-tap filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Chong Soon Lim 202 Bedok South Ave 1 #02-11 Singapore, 469332 | Tel: Email: | +65 6552 5423 jingya.li@sg.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is to reduce cross-component ALF (CCALF) filter shape from 8-tap to 6-tap so that number of operations could be further reduced by 25% and internal memory transaction is also simplified. A side benefit is that virtual boundary handling of CCALF can also be removed for 4:2:0 format, Simulation results are as below:

Anchor (VTM7.0):		 								All Intra:  0.15%, -7.70%, -6.29%

RA:	   0.12%, -9.39%, -8.87%

LDB:     0.12%, -11.39%, -8.20%

Anchor (CE5 common base):							All Intra:  -0.02%, 0.68%, 0.88%

RA:	   -0.02%, 0.93%, 1.08%

LDB:     -0.01%, 1.95%, 1.54%

# Introduction

In last Geneva meeting, a joint contribution (JVET-P1008) was proposed to simplify the design of cross-component ALF proposed in Gothenburg meeting (JVET-O0636). In JVET-P1008, an 8-tap filter (Figure 1b) is used to replace the 18-tap filter (Figure 1a) in JVET-O0636 to reduce number of operations. In this proposal, a further simplification is proposed to use 6-tap filter (Figure 1c) to (1) further reduce number of operation (2) reduce internal memory transaction cost from storage of luma reconstructed samples to CCALF processing unit (3) remove virtual boundary handling of CCALF for 4:2:0 format.

Figure 1: CCALF filter

# Experimental results

Coding performance reported as below. The CTC (common test conditions) defined in [1] was followed. Note that enc/dec time is not reliable.

Table 1: Simulation Results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | -4.57% | -4.34% | 102% | 103% |
| Class A2 | 0.10% | -7.26% | -11.73% | 101% | 103% |
| Class B | 0.13% | -12.49% | -9.27% | 99% | 100% |
| Class C | 0.16% | -3.56% | -3.06% | 104% | 105% |
| Class E | 0.20% | -8.80% | -2.12% | 96% | 97% |
| Overall | 0.15% | -7.70% | -6.29% | 100% | 102% |
| Class D | 0.08% | -3.29% | -1.40% | 99% | 99% |
| Class F | 0.13% | -5.67% | -5.92% | 103% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | -5.17% | -4.91% | 99% | 99% |
| Class A2 | 0.13% | -9.79% | -15.45% | 102% | 102% |
| Class B | 0.15% | -16.39% | -12.18% | 101% | 102% |
| Class C | 0.08% | -3.49% | -2.78% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | -9.39% | -8.87% | 101% | 101% |
| Class D | 0.05% | -3.80% | -2.76% | 100% | 100% |
| Class F | 0.13% | -3.76% | -3.68% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.16% | -17.59% | -14.85% | 99% | 101% |
| Class C | 0.10% | -3.83% | -3.06% | 104% | 103% |
| Class E | 0.06% | -11.13% | -3.95% | 99% | 99% |
| Overall | 0.12% | -11.39% | -8.20% | 101% | 101% |
| Class D | 0.12% | -4.81% | -3.53% | 102% | 101% |
| Class F | 0.08% | -6.87% | -5.24% | 101% | 102% |

# Discussions

This contribution proposes a 6-tap filter for CCALF. coding impact is 0.15%, -7.70%, -6.29% (AI), 0.12%, -9.39%, -8.87% (RA), and 0.12%, -11.39%, -8.20% (LDB) for Y, Cb, Cr components resprectively with VTM7.0 as anchor. Encoding and decoding time impact is not reliable.

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-M1010, the 13th JVET meeting, Jan. 2019, Marrakesh, MA

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).