| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0102-v1 |
| --- | --- |

| Title: | AHG16: A study of bin to bit ratio for VTM7.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou | Email: | minhua.zhou@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution reports the bin to bit ratio of the VTM7.0 for both the CTC and low QP setting (QP =2, 7, 12, 17).  The average results are summarized as follows:

- For  CTC AI/RA/LDB configurations,

  - the weighted bin to bit ratio in the VTM7.0 is 19.97%/22.10% /17.54% higher than the HM16.19, respectively.

  - the unweighted bin to bit ratio in the VTM7.0 is 12.80%/14.41%/13.16% higher than the HM16.19, respectively.

- For low-QP AI/RA/LDB configurations,

  - the weighted bin to bit ratio in the VTM7.0 is 17.72%/15.40%/17.80% higher than the HM16.19, respectively.

  - the unweighted bin to bit ratio in the VTM7.0  is 9.84%/6.75%/8.97% higher than the HM16.19, respectively.

As far as the low-QP AI configuration is concerned, the weighted and un-weighted bin to bit ratio of the VTM7.0 is about 18% and 10% higher than that of HM16.19, respectively. The numbers here are roughly same as that of VTM6.0.

In the weighted bin to bit ratio, a bypass bin is counted at 0.25 context coded bins; in the unweight bin to bit ratio, a bypass bin and a context coded bin carry an equal weight (1:1). All the percentage numbers are measured against the HM16.19.

# Introduction

The bin to bit ratio of the VVC has been studied since VTM4.0 for both the CTC and low QP coding (QP =2, 7, 12, 17).  This contribution reports the bin to bit ratio of VTM7.0 relative to HM16.19.

# Experimental results

Tests were carried out by using the VTM7.0 and HM16.19 reference software. The CTC (common test conditions) defined in [1] was followed. Additional tests were conducted for low QPs of 2, 7, 12 and 17 with a sequence length of 100 frames. The same test sequences and coding parameter settings were used for HM16.19.

The following data are collected on the decoder side:

- The peak context coded bin rate, bypass bin rate and bit-rate of a coded sequence

- The average context coded bin rate, bypass bin rate and bit-rate over a coded sequence.

- The average context coded bin rate, bypass bin rate and bit-rate consumed by the transform coefficients coding over a coded sequence.

In addition to the BD-rate, the following measurements are computed based on the collected data:

- Weighted BD-binrate for the peak case.

- Weighted BD-binrate for the average case.

- Weighted BD-binrate for the transform coefficients coding part.

- Weighted bin to bit ratio for the peak case.

- Unweighted bin to bit ratio for the peak case, for the average case, and for the transform coefficients coding part.

In the weighted measurements, a bypass bin is counted as 0.25 context coded bins. In the unweighted measurements, a bypass bin and a context coded bin carry an equal weight (1:1).

The bin to bit ratio of the VTM7.0 is roughly same as that of VTM6.0. As far as the low-QP AI configuration is concerned, the weighted and un-weighted bin to bit ratio of the VTM7.0 is about 18% and 10% higher than that of HM16.19, respectively.

The AI/RA/LDB summary results for the VVC common testing conditions (CTC) and the low-QP setting are provided in Table 1, and Table 2, respectively. For detailed results, please refer to the attached spreadsheets.

Table 1: Summary results for CTC (VTM7.0 vs. HM16.19)

Table 2: Summary results for low QPs (VTM7.0 vs. HM16.19)

# References

[1] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

[2] B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vE, the 16th  JVET Meeting, October 2019, Geneva, CH

[3] M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio in VTM4.0 and HM16.19”, JVET-N0049, the 14th JVET meeting, March 2019, Geneva, CH

[4] M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio for VTM5.0”, JVET-O0068, the 15th JVET meeting, July 2019, Gothenburg, SE

[5] M. Zhou, “AHG16/Non-CE7: A study of bin to bit ratio for VTM6.0”, JVET-P0050, the 16th  JVET Meeting, October 2019, Geneva, CH

# Patent rights declaration(s)

Broadcom Inc. does not have current or pending patent rights relating to the technology described in this contribution.