| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0111-v2 |
| --- | --- |

| Title: | A proposal of an 8-bit profile |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ye-Kui Wang Bytedance, San Diego, CA, USA Jiexi Wang Bytedance, Beijing, China Xiang Li Tencent, Palo Alto, CA, USA | Tel: Email: | lizhang.idm@bytedance.com yekui.wang@bytedance.com wangjiexi@bytedance.com xlxiangli@tencent.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

It is asserted that hardware decoders of VVC take time to implement, while it'd be desirable to deploy VVC as earlier as possible, initially using software decoders. However, the implementation cost of software decoders for 10-bit decoding is asserted to be significantly higher than that for 8-bit decoding due to the need of SIMD optimizations etc. Therefore, this contribution proposes to specify an 8-bit profile for VVC.

# Proposal

The proposed text changes on top of the latest VVC draft text are provided below, including some editorial changes to existing texts for profile definitions, as well as text changes for the proposed 8-bit profile. The most relevant parts that have been added or modified are highlighted in yellow.

  - Overview of profiles, tiers and levels

Profiles, tiers and levels specify restrictions on bitstreams and hence limits on the capabilities needed to decode the bitstreams. Profiles, tiers and levels may also be used to indicate interoperability points between individual decoder implementations.

Each profile specifies a subset of algorithmic features and limits that shall be supported by all decoders conforming to that profile.

NOTE – Encoders are not required to make use of any particular subset of features supported in a profile.

Each level of a tier specifies a set of limits on the values that may be taken by the syntax elements of this Specification. The same set of tier and level definitions is usally used with all profiles, but individual implementations may support a different tier and within a tier a different level for each supported profile. For any given profile, a level of a tier generally corresponds to a particular decoder processing load and memory capability.

In this annex, phrases like "a bitstream conforming to" should be intepreted as "the bitstream of an OLS conforming to".

  - Profiles

    - Main 8 profile

Bitstreams conforming to the Main 8 profile shall obey the following constraints:

- Referenced SPSs shall have chroma_format_idc equal to 0 or 1 only.

- Referenced SPSs shall have bit_depth_minus8 equal to 0.

- general_level_idc and sublayer_level_idc[ i ] for all values of i in referenced SPSs shall not be equal to 255 (which indicates level 8.5).

- The tier and level constraints specified for the Main 8 profile in clause A.4, as applicable, shall be fulfilled.

Conformance of a bitstream to the Main 8 profile is indicated by general_profile_idc being equal to 1.

Decoders conforming to the Main 8 profile at a specific level (identified by a specific value of general_level_idc) of a specific tier (identified by a specific value of general_tier_flag) shall be capable of decoding all bitstreams and sublayer representations for which all of the following conditions apply:

- The bitstream or sublayer representation is indicated to conform to the Main 8 profile.

- The bitstream or sublayer representation is indicated to conform to a level that is not level 8.5 and is lower than or equal to the specified level.

- The bitstream or sublayer representation is indicated to conform to a tier that is lower than or equal to the specified tier.

    - Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Referenced SPSs shall have chroma_format_idc equal to 0 or 1 only.

- Referenced SPSs shall have bit_depth_minus8 in the range of 0 to 2, inclusive.

- general_level_idc and sublayer_level_idc[ i ] for all values of i in referenced SPSs shall not be equal to 255 (which indicates level 8.5).

- The tier and level constraints specified for the Main 10 profile in clause A.4, as applicable, shall be fulfilled.

Conformance of a bitstream to the Main 10 profile is indicated by general_profile_idc being equal to 2.

Decoders conforming to the Main 10 profile at a specific level (identified by a specific value of general_level_idc) of a specific tier (identified by a specific value of general_tier_flag) shall be capable of decoding all bitstreams and sublayer representations for which all of the following conditions apply:

- The bitstream or sublayer representation is indicated to conform to the Main 10 or Main 8 profile.

- The bitstream or sublayer representation is indicated to conform to a level that is not level 8.5 and is lower than or equal to the specified level.

- The bitstream or sublayer representation is indicated to conform to a tier that is lower than or equal to the specified tier.

    - Main 4:4:4 10 profile

Bitstreams conforming to the Main 4:4:4 10 profile shall obey the following constraints:

- Referenced SPSs shall have chroma_format_idc in the range of 0 to 3, inclusive.

- Referenced SPSs shall have bit_depth_minus8 in the range of 0 to 2, inclusive.

- general_level_idc and sublayer_level_idc[ i ] for all values of i in referenced SPSs shall not be equal to 255 (which indicates level 8.5).

- The tier and level constraints specified for the Main 4:4:4 10 profile in clause A.4, as applicable, shall be fulfilled.

Conformance of a bitstream to the Main 4:4:4 10 profile is indicated by general_profile_idc being equal to 3.

Decoders conforming to the Main 4:4:4 10 profile at a specific level (identified by a specific value of general_level_idc) of a specific tier (identified by a specific value of general_tier_flag) shall be capable of decoding all bitstreams and sublayer representations for which all of the following conditions apply:

- The bitstream or sublayer representation is indicated to conform to the Main 4:4:4 10, Main 10, or Main 8 profile.

- The bitstream or sublayer representation is indicated to conform to a level that is not level 8.5 and is lower than or equal to the specified level.

- The bitstream or sublayer representation is indicated to conform to a tier that is lower than or equal to the specified tier.

# Patent rights declaration(s)

Bytedance Inc. does not have any current or pending patent rights relating to the technology proposed in this contribution (to the extent of the personal awareness of the authors).