| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0143-v2 |
| --- | --- |

| Title: | CE3-related: Issue of Level mapping in transform skip residual |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yusuke Kato Kiyofumi Abe Tadamasa Toma | Tel: Email: | +81-80-3083-1171 kato.yusuke003@jp.panasonic.com abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to define a new flag in SPS to enable or disable Level mapping in transform skip residual coding. According to the results reported in this contribution, Level mapping has coding loss on camera captured contents for whole QP range including LossLess setting, while it has good coding gain on screen contents. Therefore, this contribution proposes to disable Level mapping from  ClassA to ClassE and enable it on ClassF and ClassTGM in all test conditions. Simulation results of Lossless coding, CTC, and LowQP are reportedly shown as follows:

Lossless:

Overall (ClassA-E):     -0.54%AI,  -0.68%RA,  -1.32%LB

CTC:

Overall:     　　　　　-0.00%AI,  -0.02%RA,  -0.02%LB

LowQP:

Overall:     　　　　　-0.03%AI,  -0.XX%RA,  -0.01%LB

Introduction

In VTM7.0, the transform skip (TS) residual levels of a coefficient group (CG) are coded in the following three passes:

- Pass 1: for (n=0, n <= numSbCoeff – 1 && CCB>=4, n++)

sig_coeff_flag

coeff_sign_flag

Gt 1 flag (abs_level_gtx_flag[0])

parity flag (par_level_flag)

FirstPassBypassPos = n

- Pass 2: for (n=0, n <= FirstPassBypassPos && CCB>=4, n++)

Gt 3 flag (abs_level_gtx_flag[1])

Gt 5 flag (abs_level_gtx_flag[2])

Gt 7 flag (abs_level_gtx_flag[3])

Gt 9 flag (abs_level_gtx_flag[4])

SecondPassBypassPos = n

- Pass 3: for (n = 0; n < =numSbCoeff - 1; n++)

baseLevel = (n >= FirstpassBypassPos ? 0 : (n >= SecondpassBypassPos ? 2 : 10)

abs_remainder[n]

if (n < FirstpassBypassPos)

inverse level mapping

Proposal

According to the simulation results in section 3, Level mapping has coding loss on camera captured contents for whole QP range including LossLess setting, while it has good coding gain on screen contents. Therefore, it is proposed to define a new flag in SPS to enable or disable Level mapping, and turn off this flag in sequences from ClassA to ClassE and turn on it in sequences of ClassF and ClassTGM .

The proposed syntax of TS is as follows:

- Pass 1: for (n=0, n <= numSbCoeff – 1 && CCB>=4, n++)

sig_coeff_flag

coeff_sign_flag

Gt 1 flag (abs_level_gtx_flag[0])

parity flag (par_level_flag)

FirstPassBypassPos = n

- Pass 2: for (n=0, n <= FirstPassBypassPos && CCB>=4, n++)

Gt 3 flag (abs_level_gtx_flag[1])

Gt 5 flag (abs_level_gtx_flag[2])

Gt 7 flag (abs_level_gtx_flag[3])

Gt 9 flag (abs_level_gtx_flag[4])

SecondPassBypassPos = n

- Pass 3: for (n = 0; n <= numSbCoeff - 1; n++)

baseLevel = (n >= FirstpassBypassPos ? 0 : (n >= SecondpassBypassPos ? 2 : 10)

abs_remainder[n]

if (n < FirstpassBypassPos && sps_level_mapping_enabled_flag )

inverse level mapping

Simulation results

Table 1, 2 and 3 show the lossless, CTC and LowQP results when level mapping is disabled, respectively. In this test, QP value in configuration file is set to be 4. By introducing a enabling flag as proposed,  ClassA to ClassE can get the coding gain on these tables while ClassF and ClassTGM keep same performance as VTM-7.0. Runtime of LowQP is not accurate because of different clusters.

Table 1: Simulation results of level mapping turning off in lossless

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.0 | Q0143 |  | VTM7.0 | Q0143 |  | VTM7.0 | Q0143 |  |
| Class A1 | 2.2 | 2.2 | -0.24% | 2.2 | 2.2 | -0.29% |  |  |  |
| Class A2 | 1.6 | 1.6 | -0.06% | 1.7 | 1.7 | -0.12% |  |  |  |
| Class B | 2.2 | 2.2 | -0.51% | 2.3 | 2.4 | -0.71% | 2.3 | 2.4 | -0.71% |
| Class C | 1.9 | 2.0 | -0.67% | 2.4 | 2.5 | -1.36% | 2.4 | 2.5 | -1.36% |
| Class D | 1.9 | 1.9 | -0.42% | 2.8 | 2.8 | -1.37% | 2.7 | 2.8 | -1.39% |
| Class E | 2.8 | 2.8 | -1.16% |  |  |  | 3.1 | 3.2 | -2.28% |
| Class F | 5.3 | 5.3 | 0.00% | 33.7 | 33.6 | -0.67% | 50.7 | 50.5 | -0.73% |
| TGM | 11.8 | 11.4 | 3.19% | 107.1 | 99.3 | 5.80% | 124.9 | 114.0 | 7.09% |
| Overall | 2.1 | 2.1 | -0.54% | 2.2 | 2.2 | -0.68% | 2.6 | 2.6 | -1.32% |
| Enc Time[%] | 96% |  |  | 97% |  |  | 96% |  |  |
| Dec Time[%] | 98% |  |  | 97% |  |  | 95% |  |  |

Table 2: Simulation results of level mapping turning off in CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | -0.01% | 100% | 100% |
| Class A2 | 0.01% | -0.02% | -0.06% | 100% | 100% |
| Class B | 0.00% | -0.02% | -0.08% | 100% | 100% |
| Class C | -0.02% | 0.06% | 0.11% | 100% | 100% |
| Class E | 0.01% | -0.09% | -0.03% | 101% | 100% |
| Overall | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class D | -0.01% | 0.01% | -0.02% | 101% | 100% |
| Class F | 0.38% | 0.37% | 0.44% | 100% | 100% |
| TGM | 0.78% | 0.93% | 0.88% | 98% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.05% | 0.02% | 101% | 100% |
| Class A2 | -0.02% | -0.09% | -0.01% | 100% | 100% |
| Class B | 0.00% | -0.01% | -0.06% | 100% | 100% |
| Class C | -0.03% | -0.04% | -0.20% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.02% | -0.07% | 100% | 100% |
| Class D | -0.04% | -0.29% | -0.04% | 100% | 100% |
| Class F | 0.28% | 0.46% | 0.35% | 100% | 100% |
| TGM | 0.55% | 0.66% | 0.70% | 99% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.14% | -0.04% | 99% | 98% |
| Class C | 0.00% | -0.30% | -0.05% | 104% | 98% |
| Class E | -0.07% | 0.68% | -0.36% | 100% | 97% |
| Overall | -0.03% | 0.01% | -0.12% | 101% | 98% |
| Class D | -0.05% | 0.43% | -0.32% | 104% | 102% |
| Class F | 0.19% | 0.18% | 0.83% | 100% | 99% |
| TGM | 0.37% | 0.42% | 0.40% | 100% | 100% |

Table 3: Simulation results of level mapping turning off in LowQP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.03% | -0.03% | 99% | 100% |
| Class A2 | 0.00% | -0.01% | -0.01% | 100% | 100% |
| Class B | -0.02% | -0.01% | -0.03% | 99% | 99% |
| Class C | -0.03% | -0.05% | -0.03% | 99% | 99% |
| Class E | -0.09% | -0.15% | -0.21% | 100% | 99% |
| Overall | -0.03% | -0.05% | -0.06% | 99% | 99% |
| Class D | 0.01% | -0.04% | -0.04% | 100% | 100% |
| Class F | 0.11% | 0.14% | 0.17% | 98% | 99% |
| TGM | 1.53% | 1.28% | 1.32% | 97% | 101% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  |  | DecT |
| Class A1 | -0.04% | -0.03% |  | -0.02% |  | 99% |  | 98% |
| Class A2 | 0.00% | 0.00% |  | 0.00% |  | 100% |  | 99% |
| Class B | -0.02% |  | 0.00% |  | -0.03% |  | 96% |  |  | 94% |
| Class C | -0.02% |  | -0.05% |  | -0.06% |  | 102% |  |  | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.02% |  | -0.02% |  | -0.03% |  | 99% |  |  | 97% |
| Class D | -0.04% |  | -0.03% |  | -0.11% |  | 96% |  |  | 95% |
| Class F | 0.05% |  | 0.25% |  | 0.11% |  | 98% |  |  | 97% |
| TGM | 1.06% |  | 1.16% |  | 1.18% |  | 104% |  |  | 108% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.01% | 0.01% | 100% | 100% |
| Class C | -0.01% | -0.01% | 0.00% | 99% | 100% |
| Class E | -0.02% | 0.00% | 0.00% | 100% | 100% |
| Overall | -0.01% | -0.01% | 0.01% | 100% | 100% |
| Class D | -0.08% | -0.01% | -0.11% | 99% | 100% |
| Class F | 0.05% | 0.13% | 0.11% | 99% | 100% |
| TGM | 1.00% | 0.62% | 0.59% | 99% | 102% |

In the first implementation, level mapping was still running in RDOQTS function.

Table4 and Table5 show the CTC and LowQP results of RDOQTS bugfix version of Table2 and Table3.

This change doesn’t affect simulation results of lossless mode since RDOQTS is disabled in lossless.cfg.

Table 4: Simulation results of level mapping turning off plus RDOQTS bugfix in CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | 0.00% | 101% | 101% |
| Class A2 | 0.00% | -0.02% | -0.03% | 101% | 101% |
| Class B | 0.00% | -0.01% | -0.07% | 101% | 102% |
| Class C | -0.02% | 0.05% | 0.14% | 101% | 101% |
| Class E | 0.01% | -0.05% | -0.03% | 101% | 100% |
| Overall | 0.00% | -0.01% | 0.00% | 101% | 101% |
| Class D | -0.01% | 0.00% | -0.04% | 100% | 100% |
| Class F | 0.37% | 0.35% | 0.53% | 100% | 101% |
| TGM | 0.78% | 0.93% | 0.87% | 98% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | 0.03% | 100% | 100% |
| Class A2 | -0.03% | -0.08% | 0.04% | 100% | 100% |
| Class B | 0.00% | 0.03% | -0.10% | 100% | 100% |
| Class C | -0.03% | -0.01% | -0.08% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | -0.04% | 100% | 100% |
| Class D | -0.06% | -0.22% | -0.23% | 100% | 99% |
| Class F | 0.24% | 0.43% | 0.32% | 99% | 99% |
| TGM | 0.51% | 0.66% | 0.68% | 98% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.45% | -0.01% | 100% | 100% |
| Class C | -0.01% | -0.35% | -0.01% | 100% | 100% |
| Class E | -0.06% | 0.83% | -0.21% | 100% | 100% |
| Overall | -0.02% | -0.10% | -0.06% | 100% | 100% |
| Class D | -0.04% | 0.45% | -0.27% | 99% | 100% |
| Class F | 0.14% | 0.24% | 1.07% | 100% | 100% |
| TGM | 0.37% | 0.43% | 0.40% | 99% | 100% |

Table 5: Simulation results of level mapping turning off plus RDOQTS bugfix in LowQP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.03% | -0.04% | 99% | 99% |
| Class A2 | 0.00% | -0.01% | 0.00% | 99% | 98% |
| Class B | -0.02% | -0.03% | -0.04% | 99% | 98% |
| Class C | -0.03% | -0.06% | -0.04% | 99% | 99% |
| Class E | -0.09% | -0.16% | -0.21% | 99% | 99% |
| Overall | -0.03% | -0.05% | -0.06% | 99% | 99% |
| Class D | 0.01% | -0.07% | -0.04% | 98% | 95% |
| Class F | 0.11% | 0.17% | 0.18% | 99% | 99% |
| TGM | 1.48% | 1.26% | 1.34% | 96% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.01% | 0.00% | -0.01% | 99% | 99% |
| Class C | -0.02% | -0.06% | -0.04% | 99% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.05% | -0.05% | -0.14% | 99% | 98% |
| Class F | 0.06% | 0.11% | 0.03% | 99% | 98% |
| TGM | 1.15% | 1.14% | 1.13% | 98% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.02% | 0.00% | 99% | 98% |
| Class C | -0.01% | -0.01% | -0.01% | 99% | 99% |
| Class E | -0.01% | -0.01% | -0.01% | 99% | 98% |
| Overall | -0.01% | -0.01% | -0.01% | 99% | 98% |
| Class D | -0.08% | -0.04% | -0.15% | 99% | 99% |
| Class F | -0.06% | 0.06% | 0.05% | 99% | 99% |
| TGM | 0.93% | 1.01% | 1.02% | 98% | 101% |

For information, level mapping turning off on CE3-1.2 is tested. Table6 shows the simulation results of lossless mode.

It can be shown that level mapping off also has good coding gains in lossless mode over CE3-1.2.

Table 6: Simulation results of level mapping turning off on CE3-1.2 vs CE3-1.2

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ratio |  | bit-rate savings | Ratio |  | bit-rate savings |
|  | CE3-1.2 | Q0143+CE312 |  | CE3-1.2 | Q0143+CE312 |  |
| Class A1 | 2.3 | 2.3 | -0.20% | 2.3 | 2.3 | -0.22% |
| Class A2 | 1.7 | 1.7 | -0.06% | 1.8 | 1.8 | -0.11% |
| Class B | 2.2 | 2.2 | -0.40% | 2.4 | 2.4 | -0.59% |
| Class C | 2.0 | 2.0 | -0.50% | 2.5 | 2.5 | -1.13% |
| Class D | 2.0 | 2.0 | -0.29% | 2.8 | 2.9 | -1.15% |
| Class E | 2.9 | 2.9 | -0.98% |  |  |  |
| Class F | 5.5 | 5.5 | 0.03% | 34.8 | 34.6 | -0.56% |
| TGM | 12.3 | 11.9 | 2.62% | 115.7 | 108.8 | 4.55% |
| Overall | 2.2 | 2.2 | -0.43% | 2.3 | 2.3 | -0.57% |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay B |  |  |
|  | Ratio |  | bit-rate savings |
|  | CE3-1.2 | Q0143+CE312 |  |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | 2.4 | 2.4 | -0.59% |
| Class C | 2.5 | 2.5 | -1.14% |
| Class D | 2.8 | 2.8 | -1.17% |
| Class E | 3.2 | 3.3 | -1.97% |
| Class F | 52.1 | 51.8 | -0.58% |
| TGM | 136.0 | 124.2 | 6.41% |
| Overall | 2.6 | 2.7 | -1.12% |
| Enc Time[%] | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  |

Conclusion

Level mapping generally shows coding loss on camera captured contents in Lossless, CTC and LowQP modes, while it has good coding gain on screen contents. So, we suggest to define sps level flag to enable or disable level mapping, and turn off this flag in sequences from ClassA to ClassE and turn on it in sequences of ClassF and ClassTGM .

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).