| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0161 |
| --- | --- |

| Title: | On constant shift and offset in MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2782 kenji.kondo@sony.com Masaru.Ikeda@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

This contribution proposes to replace an adaptive shift operation and offset with constant shift and offset in a matrix-based intra prediction (MIP). In current VTM, MIP uses the shift operations that the number of shift changes depending on MIP-sizeId and MIP mode. Also MIP uses adaptive offset factor "fO" that the value changes depending on MIP-sizeId and MIP mode. To simplify, this contribution proposes to use constant those values. Two cases are tested that the table bit range are in 7 bit (test-sW5) and 8 bit (test-sW6). For the test-sW5, experimental results of 0.04% and 0.02% for AI and RA conditions, are reported. For the test-sW6, experimental results of 0.00% and 0.00% for AI and RA conditions, are reported.

Introduction

In the last meeting, several techniques were proposed to simplify MIP. For MIP simplification, test results were reported in core experiment (CE) 3-2 [1]. In the [1], to use constant shift was proposed. Also, a contribution, JVET-P0136 proposed to use a constant offset factor in [2]. And, a contribution, JVET-P0625 proposed to use power of two for offset factor to calculate without multiplication in [3].

Proponents of above techniques were encouraged to perform further study on these proposals, based on the floating-point models of the newly adopted matrices. As following the decision, this contribution studies to unify those techniques.

Proposals

As described in following subclause, two techniques are proposed that uses constant shift and offset. To apply the technique, it’s needed to modify weights of matrices. The weights are calculated from floating-point data. The floating-point data was provided by HHI.

In the VTM-7.0 design, the weight table is defined in range of unsigned 7-bit. In this contribution, two cases are proposed that the table is of unsigned 7-bit (test-sW5) and unsigned 8-bit (test-sW6). The test-sW6 has higher precision than the test-sW7. Therefor, it can suppress coding efficiency loss. On the other had, it increases the size of table. To know a good trade-off, those cases are tested.

VTM-7.0

In the draft of Versatile Video Coding [4], the equations to compute prediction are defined as following.

p[ 0 ] = pTemp[ 0 ] − ( 1  <<  ( BitDepth − 1 ) )
p[ x ] = pTemp[ x ] − pTemp[ 0 ]	for x = 1..inSize − 1	(256)

oW = ( 1 << ( sW − 1 ) ) − fO * (i = 0inSize - 1p i )	(257)

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*predSize+x *p i  ) + 
	oW ) >> sW ) + pTemp[ 0 ]			(258)

Where, sW is a value called weight shift, and fO is a value called offset factor. The sW and the fO are depending on mipSizeId and modeId of MIP. Those values are derived by referring the table.

Test case: test-sW5

In this test case, the sW is set to 5, and the fO is set to 48. The reason of the number 48 is to replace a multiplication by add and shift operation because 48 is represented with (1<<5)+(1<<4). With this change, the equations of section-2.1 are modified as following.

p[ 0 ] = ( 1  <<  ( BitDepth − 1 ) )  − pTemp[ 0 ]
p[ x ] = pTemp[ x ] − pTemp[ 0 ]	for x = 1..inSize − 1	(256)

sum =  (i = 0inSize - 1p i )

oW = ( 1 << 4 ) − (sum << 5)  − (sum << 4) 	(257)

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*predSize+x *p i  ) + 
	oW ) >> 5 ) + pTemp[ 0 ]		 (258)

Test case: test-sW6

In this test case, the sW is set to 6, and the fO is set to 96. As same as above case of test-sW5, 96 is could be replaced by add and shift operation. With this change, the equations of section-2.1 are modified as following.

p[ 0 ] = ( 1  <<  ( BitDepth − 1 ) )  − pTemp[ 0 ]
p[ x ] = pTemp[ x ] − pTemp[ 0 ]	for x = 1..inSize − 1	(256)

sum =  (i = 0inSize - 1p i )

oW = ( 1 << 5 ) − (sum << 6)  − (sum << 5) 	(257)

predMip[ x ][ y ] = ( ( ( i = 0inSize - 1mWeight i  y*predSize+x *p i  ) + 
	oW ) >> 6 ) + pTemp[ 0 ]		 (258)

Experiments

The introduced techniques had been implemented on a reference software VTM-7.0 [6]. For these tests. the JVET common test condition [5] was followed. To compare, the VTM-7.0 was used as an anchor, and the BD-rates were calculated.

For the simulations, same platform machines in PC cluster (Intel Core i7-7700K for encoding, Core i5-2500K for decoding) were used.

For a runtime measurement, the user time in the output log was used. For a random-access case and an all intra case, a parallel encoding is used. To measure encoding time in parallel encoding, the total of all user time for each frame were calculated.

Results of case test-sW5

For test-sW5, the results of BD-rate and runtime are shown in Table 1.

Table 1: Test results for test-sW5

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.10% | 0.14% | 98% | 100% |
| Class A2 | 0.03% | 0.00% | -0.05% | 99% | 100% |
| Class B | 0.03% | 0.01% | -0.07% | 100% | 100% |
| Class C | 0.02% | 0.03% | -0.03% | 100% | 99% |
| Class E | 0.05% | -0.01% | 0.00% | 100% | 99% |
| Overall | 0.04% | -0.01% | -0.01% | 99% | 100% |
| Class D | 0.02% | -0.07% | -0.06% | 100% | 99% |
| Class F | 0.01% | 0.02% | 0.04% | 99% | 100% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.08% | 0.03% | 100% | 101% |
| Class A2 | -0.03% | 0.05% | -0.02% | 100% | 100% |
| Class B | 0.02% | 0.09% | 0.25% | 100% | 100% |
| Class C | 0.04% | -0.11% | -0.11% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.01% | 0.06% | 100% | 100% |
| Class D | -0.02% | -0.23% | -0.06% | 101% | 100% |
| Class F | -0.01% | 0.09% | 0.09% | 101% | 100% |

Results of case test-sW6

For test-sW6, the results of BD-rate and runtime are shown in Table 2.

Table 2: Test results for test-sW6

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.19% | 0.15% | 98% | 100% |
| Class A2 | 0.00% | 0.04% | -0.05% | 99% | 100% |
| Class B | 0.00% | 0.00% | -0.07% | 100% | 100% |
| Class C | 0.02% | 0.05% | 0.07% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.11% | 100% | 99% |
| Overall | 0.00% | -0.01% | 0.03% | 99% | 100% |
| Class D | 0.01% | 0.03% | 0.02% | 100% | 100% |
| Class F | 0.00% | 0.03% | 0.13% | 99% | 100% |
|  |  |  | Random access |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.08% | 0.00% | 100% | 100% |
| Class A2 | -0.02% | -0.04% | -0.05% | 99% | 100% |
| Class B | 0.01% | 0.02% | 0.08% | 100% | 100% |
| Class C | 0.01% | 0.05% | -0.04% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 100% |
| Class D | 0.01% | -0.18% | 0.17% | 101% | 100% |
| Class F | -0.02% | 0.09% | 0.05% | 101% | 100% |

Conclusion

This contribution proposed to replace weight shift and offset depending on mipSizeId and modeId of MIP with constant values. With these techniques, the complexity is reduced and the text of VVC is simplified by removing two tables of sW and fO. The proposed techniques were implemented based on VTM-7.0 software. Experimentally the coding efficiency impact is tested. The simulation results were showed that were 0.04% and 0.02% for test-sW5, AI and RA case, 0.00% and 0.00% for test-sW6, AI and RA case. For a runtime performance, no change was confirmed. With these results, it is recommended to adopt the techniques of case test-sW5 or test-sW6.

References

- Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki, Junyan Huo, Yanzhuo Ma, Fuzheng Yang, Shuai Wan and Yuanfang Yu, “CE3-2: On rounding shift of MIP”, Document of JVET-P0056, Geneva, Switzerland, October 2019

- J. Huo, H. Wang, Y. Sun, Y. Ma, S. Wan, F. Yang, Y. Yu, Y. Liu,  Non-CE 3: MIP simplification, Document of JVET-P0136, Geneva, Switzerland, October 2019

- Thibaud Biatek, Adarsh K. Ramasubramonian, Geert Van der Auwera and Marta Karczewicz, “Non-CE3: simplified MIP with power-of-two offset”, Document of JVET-P0625, Geneva, Switzerland, October 2019

- Benjamin Bross, Jianle Chen, Shan Liu and Ye-Kui Wang, “Versatile Video Coding (Draft 7)”, Document of JVET-P2001-vE, Geneva, Switzerland, October 2019

- Frank Bossen, Jill Boyce, Karsten Suehring, Xiang Li and Vadim Seregin, “JVET common test conditions and software reference configurations for SDR video”, Document of JVET-N1010, Geneva, Switzerland, March 2019

- VTM-7.0 software, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).