| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0170-v1 |
| --- | --- |

| Title: | AHG9: On inter-layer referencing of ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Chen-Yen Lai, Ching-Yeh Chen, Lulin Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {olena.chubach, jenny.lai, chingyeh.chen, lulin.chen, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to add support for inter-layer referencing between a direct reference layer and the current layer for adaptive loop filter (ALF) adaptation parameter set (APS). For that, two syntax designs are proposed.

In the first design, it is proposed to signal an on-off control flag in the sequence parameter set (SPS) to indicate whether inter-layer referencing of ALF is allowed in a current coding layer video sequence (CLVS). If inter-layer referencing of ALF is allowed in the current CLVS, an index specifying the reference layer NAL unit ID is signalled. After that, available APSs with aps_params_type equal to ALF_APS from the reference layer may be referenced by all coded pictures in the current CLVS.

In the second design, the syntax in the SPS is the same as that in the first design. In addition, an on-off control flag is signalled in the picture header (PH)/slice header (SH) in the current CLVS, indicating whether inter-layer referencing of ALF is enabled for the current picture/slice. If inter-layer referencing of ALF is allowed in both the SPS and the current picture/slice, available ALF APSs from the reference layer are referenced by the current picture/slice. Otherwise, if inter-layer referencing of ALF is allowed in the SPS but disabled in the current picture/slice, the ALF APSs from the reference layer cannot be referenced by the current picture/slice, and only ALF APSs in the current CLVS are referenced by the current picture/slice.

# Introduction

In the current VVC design [1], ALF is signalled in APSs with aps_params_type equal to ALF_APS. At the same time, according to the current VVC Draft 7, APSs within one layer cannot be referenced by another layer. Therefore, if two CLVSs use the same ALF APSs, signalling becomes inefficient.

# Proposed methods

In VVC Draft 7, inter-layer referencing is supported for reference picture list (RPL). An inter-layer reference picture (ILRP) can be used as a long-term reference picture in the RPL. In this case, only pictures from the same access unit (AU) with the current picture, and with nuh_layer_id less than the nuh_layer_id of the current layer, are considered as valid reference pictures for RPL.

In this contribution, it is proposed to add support of inter-layer referencing for ALF APSs in VVC. That is, the ALF APSs can reference APSs of the same type from different layers. However, in order to simpify data access within different layers, similar constraints to those noted above are suggested for inter-layer referencing of ALF APSs. That is:

- Only APSs from a direct reference layer can be referenced.

- The referenced APSs shall be the APSs available in a picture unit (PU) which is in the same AU with the current picture, and with nuh_layer_id less than the nuh_layer_id of the current picture.

For that, two kinds of syntax designs are proposed. The first design allows controlling inter-layer referencing on SPS level, and the second design further extends on/off control to a PH/SH level.

## Signalling on/off control and reference layer for inter-layer referencing of ALF in the SPS

In this design, an on-off flag is signalled in the SPS, to indicate whether inter-layer referencing of ALF is allowed in the CLVS. If inter-layer referencing of ALF is allowed in the CLVS, an index, e.g. sps_alf_inter_layer_idx, is signalled. This index indicates which reference layer from the list of the direct reference layers is signalled. Therefore, the signalled index shall be constrained in the range of 0 to NumDirectRefLayers[GeneralLayer[nuh_layer_id]]. This way, if inter-layer referencing of ALF is enabled in the SPS, all available ALF APSs from the reference layer may be referenced by all coded pictures in the CLVS.

The related syntax modifications are highlighted in blue.

Table 1. Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_alf_enabled_flag | u(1) |
| if (sps_alf_enabled_flag) { |  |
| sps_alf_inter_layer_ref_enabled_flag | u(1) |
| if (sps_alf_inter_layer_ref_enabled_flag) |  |
| sps_alf_inter_layer_idx | ue(v) |
| } |  |
| … |  |

sps_alf_inter_layer_ref_enabled_flag equal to 0 specifies that the ALF data of any direct reference layer are not referenced by any coded slice in the CLVS. sps_alf_inter_layer_ref_enabled_flag equal to 1 specifies that the ALF data of a direct reference layer may be referenced by coded slices in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of sps_alf_inter_layer_ref_enabled_flag is inferred to be equal to 0. When sps_alf_inter_layer_ref_enabled_flag is not present, the value of sps_alf_inter_layer_ref_enabled_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of sps_alf_inter_layer_ref_enabled_flag shall be equal to 0.

sps_alf_inter_layer_idx specifies the index of a layer in the list of direct reference layers that all APS NAL units with aps_params_type equal to ALF_APS from this direct reference layer may be referenced by coded slices in the CLVS. The value of sps_alf_inter_layer_idx shall be in the range of 0 to NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1, inclusive.

pic_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slices associated with the PH refers to. If sps_alf_inter_layer_ref_enabled_flag is equal to 1, pic_alf_aps_id_luma[ i ]  specifies the i-th adaptation_parameter_set_id of the luma component of the ALF APS in the layer with index equal to sps_aps_inter_layer_id to the list of direct reference layers. Otherwise, the TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to pic_alf_aps_id_luma[ i ]  shall be less than or equal to the TemporalId of the picture associated with PH.

pic_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slices associated with the PH refers to. If sps_alf_inter_layer_ref_enabled_flag is equal to 1, pic_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the chroma component of the ALF APS in the layer with index equal to sps_aps_inter_layer_id to the list of direct reference layers. Otherwise, the TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to pic_alf_aps_id_chroma  shall be less than or equal to the TemporalId of the picture associated with PH.

slice_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slice refers to. If sps_alf_inter_layer_ref_enabled_flag is equal to 1, slice_alf_aps_id_luma[ i ]  specifies the i-th adaptation_parameter_set_id of the luma component of the ALF APS in the layer with index equal to sps_aps_inter_layer_id to the list of direct reference layers. Otherwise, the TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_luma[ i ] is not present, the value of slice_alf_aps_id_luma[ i ] is inferred to be equal to the value of pic_alf_aps_id_luma[ i ].

slice_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slice refers to. If sps_alf_inter_layer_ref_enabled_flag is equal to 1, slice_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the chroma component of the ALF APS in the layer with index equal to sps_aps_inter_layer_id to the list of direct reference layers. Otherwise, the TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_chroma is not present, the value of slice_alf_aps_id_chroma is inferred to be equal to the value of pic_alf_aps_id_chroma.

## Optional disabling of the inter-layer referencing in the PH or SH

In addition to flags which allow enabling inter-layer referencing and the reference layer indices signalled in the SPS, it is proposed to signal one enabling flag in each PH/SH, to indicate the on-off control of inter-layer referencing in the current picture/slice. In this case, even though inter-layer referencing for ALF is enabled in the SPS, some pictures/slices in a CLVS may reference APSs from a different layer, while others reference APSs from the current layer. This will depend on whether the flag indicating inter-layer referencing for ALF is enabled in the PH/SH. Therefore, APS ID (e.g. pic_alf_aps_id_luma[] and  pic_alf_aps_id_chroma in PH / slice_alf_aps_id_luma[] and slice_alf_aps_id_chroma in SH), which will be signalled after the flag indicating inter-layer referencing on/off in the PH/SH (ph_alf_inter_layer_ref_enabled_flag/slice_alf_inter_layer_ref_enabled_flag), will indicate that APS IDs are from the reference layer only when inter-layer referencing is enabled.

The related syntax modifications are highlighted in blue.

Table 2. Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_alf_enabled_flag | u(1) |
| if (sps_alf_enabled_flag) { |  |
| sps_alf_inter_layer_ref_enabled_flag | u(1) |
| if (sps_alf_inter_layer_ref_enabled_flag) |  |
| sps_alf_inter_layer_idx | ue(v) |
| } |  |
| … |  |

sps_alf_inter_layer_ref_enabled_flag equal to 0 specifies that the ALF data of any direct reference layer are not referenced by any coded slice in the CLVS. sps_alf_inter_layer_ref_enabled_flag equal to 1 specifies that the ALF data of a direct reference layer may be referenced by coded slices in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of sps_alf_inter_layer_ref_enabled_flag is inferred to be equal to 0. When sps_alf_inter_layer_ref_enabled_flag is not present, the value of sps_alf_inter_layer_ref_enabled_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of sps_alf_inter_layer_ref_enabled_flag shall be equal to 0.

sps_alf_inter_layer_idx specifies the index of a layer in the list of direct reference layers that all APS NAL units with aps_params_type equal to ALF_APS from this direct reference layer may be referenced by coded slices in the CLVS. The value of sps_alf_inter_layer_idx shall be in the range of 0 to NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1, inclusive.

Table 3. Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … | … |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| if (sps_alf_inter_layer_ref_enabled_flag) |  |
| ph_alf_inter_layer_ref_enabled_flag | u(1) |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |
| … | … |

ph_alf_inter_layer_ref_enabled_flag equal to 0 specifies that the ALF data of any direct reference layer are not referenced by the current picture. ph_alf_inter_layer_ref_enabled_flag equal to 1 specifies that the ALF data of a direct reference layer may be referenced by the current picture. When ph_alf_inter_layer_ref_enabled_flag is not present, the value of ph_alf_inter_layer_ref_enabled_flag is inferred to be equal to 0.

pic_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slices associated with the PH refers to. If ph_alf_inter_layer_ref_enabled_flag is equal to 1, pic_alf_aps_id_luma[ i ]  specifies the i-th adaptation_parameter_set_id of the luma component of the ALF APS in the layer with index equal to sps_aps_inter_layer_id to the list of direct reference layers. Otherwise, the TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to pic_alf_aps_id_luma[ i ]  shall be less than or equal to the TemporalId of the picture associated with PH.

pic_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slices associated with the PH refers to. If ph_alf_inter_layer_ref_enabled_flag is equal to 1, pic_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the chroma component of the ALF APS in the layer with index equal to sps_aps_inter_layer_id to the list of direct reference layers. Otherwise, the TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to pic_alf_aps_id_chroma  shall be less than or equal to the TemporalId of the picture associated with PH.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag && !pic_alf_enabled_present_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| if (sps_alf_inter_layer_ref_enabled_flag) |  |
| slice_alf_inter_layer_ref_enabled_flag | u(1) |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| … |  |

slice_alf_inter_layer_ref_enabled_flag equal to 0 specifies that the ALF data of any direct reference layer are not referenced by the current slice. slice_alf_inter_layer_ref_enabled_flag equal to 1 specifies that the ALF data of a direct reference layer may be referenced by the current slice. When slice_alf_inter_layer_ref_enabled_flag is not present, the value of slice_alf_inter_layer_ref_enabled_flag is inferred to be equal to the value of slice_alf_inter_layer_ref_enabled_flag.

slice_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slice refers to. If slice_alf_inter_layer_ref_enabled_flag is equal to 1, slice_alf_aps_id_luma[ i ]  specifies the i-th adaptation_parameter_set_id of the luma component of the ALF APS in the layer with index equal to sps_aps_inter_layer_id to the list of direct reference layers. Otherwise, the TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_luma[ i ] is not present, the value of slice_alf_aps_id_luma[ i ] is inferred to be equal to the value of pic_alf_aps_id_luma[ i ].

slice_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slice refers to. If slice_alf_inter_layer_ref_enabled_flag is equal to 1, slice_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the chroma component of the ALF APS in the layer with index equal to sps_aps_inter_layer_id to the list of direct reference layers. Otherwise, the TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_chroma is not present, the value of slice_alf_aps_id_chroma is inferred to be equal to the value of pic_alf_aps_id_chroma.

# References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 7)” Document of Joint Video Experts Team, JVET-P2001.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).