| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0174-v1 |
| --- | --- |

| Title: | AHG9: Fix on high-level syntax related to coding tree constraints |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Chen-Yen Lai, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Address line 3 | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, jenny.lai, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 7, when the specified maximum multi-type tree (MTT) depth is equal to 0, a partial CU across the right or bottom boundary of the current picture may be inferred to be further split while having all split types disabled, as reported by Ticket #584. In this contribution, it is proposed that the specified maximum MTT depth shall be greater than 0 when the specified minimum QT size is greater than the specified minimum coding block size. When the specified maximum MTT depth is equal to 0 (MTT split is disabled), the minimum QT size is inferred to be equal to the specified minimum coding block size, reportedly corresponding to the CU partitioning structure in HEVC.

# Introduction

In VVC Draft 7 [1], it is a requirement of bitstream conformance that picture size specified by the syntax elements pic_width_in_luma_samples and pic_height_in_luma_sample in the PPS shall be integer multiples of Max( 8, MinCbSizeY ), where the variable MinCbSizeY specifies the minimum luma coding block size. When a CU is partially across the right or bottom boundary of the current picture, it is inferred to be further partitioned into more than one smaller CU. The across-picture-boundary CU can be further split by either quadtree (QT) split or binary tree (BT) split. However, ternary tree (TT) split is disabled in an across-picture-boundary CU. The CU partitioning shall be recusively performed on across-picture-boundary CUs until the resulting CUs are either entirely inside picture boundries or entirely outside picture boundries.

When the specified maximum hierarchy depth for multi-type tree (MTT) split is equal to 0, both BT split and TT split are disabled. A CU with block size less than or equal to the specified minimum QT size cannot be further split into smaller CUs. It may cause a problem with partitioning the CUs along picture boudaries when the current picture width and height are not integer mutiples of the specified minimum QT size. An across-picture-boundary CU may be inferred to be split while having all split types disabled.

# Proposed modifications

In this contribution, it is proposed that the maximum MTT depth shall be greater than 0 when the minimum QT size is greater than the minimum coding block size. The BT split is thus enabled for further splitting the across-picture-bounndary CUs with block size less than or equal to the minimum QT size. In the proposed syntax modifications, when the specified maximum MTT depth is equal to 0 (MTT split is disabled), the minimum QT size is inferred to be equal to the minimum coding block size. The resulting CU partitioning structure corresponds to the CU partitioning structure in HEVC, where each CU can only be futher partitioned by QT split and the minimum QT size is equal to the specified minimum coding block size.

The modifed syntax tables and semantics are provided in Secs 2.1 and 2.2, respectively.

## Modified syntax tables

The modifed syntax tables based on revised VVC Draft 7 [2] are provided below. The modified parts are further marked by magenta.

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| if( ChromaArrayType != 0 ) |  |
| qtbtt_dual_tree_intra_flag | u(1) |
| log2_min_luma_coding_block_size_minus2 | ue(v) |
| partition_constraints_override_enabled_flag | u(1) |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

#### 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| … |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( partition_constraints_override_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

## Modified semantics

The semantic modifications based on revised VVC Draft 7 [2] are provided below. The modified parts are further marked by magenta.

#### 7.4.3.3 Sequence parameter set RBSP semantics

sps_log2_diff_min_qt_min_cb_intra_slice_luma specifies the default difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum coding block size in luma samples for luma CUs in slices with slice_type equal to 2 (I) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by pic_log2_diff_min_qt_min_cb_luma present in PHs referring to the SPS. The value of sps_log2_diff_min_qt_min_cb_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When sps_log2_diff_min_qt_min_cb_intra_slice_luma is not present, the value of sps_log2_diff_min_qt_min_cb_intra_slice_luma is inferred to be equal to 0. The base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU is derived as follows:

MinQtLog2SizeIntraY = sps_log2_diff_min_qt_min_cb_intra_slice_luma + MinCbLog2SizeY	(56)

sps_log2_diff_min_qt_min_cb_inter_slice specifies the default difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum luma coding block size in luma samples for luma CUs in slices with slice_type equal to 0 (B) or 1 (P) referring to the SPS. When partition_constraints_override_enabled_flag is equal to 1, the default difference can be overridden by pic_log2_diff_min_qt_min_cb_luma present in PHs referring to the SPS. The value of sps_log2_diff_min_qt_min_cb_inter_slice shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When sps_log2_diff_min_qt_min_cb_inter_slice is not present, the value of sps_log2_diff_min_qt_min_cb_inter_slice is inferred to be equal to 0. The base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU is derived as follows:

MinQtLog2SizeInterY = sps_log2_diff_min_qt_min_cb_inter_slice + MinCbLog2SizeY	(57)

#### 7.4.3.6 Picture header RBSP semantics

pic_log2_diff_min_qt_min_cb_intra_slice_luma specifies the difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum coding block size in luma samples for luma CUs in the slices with slice_type equal to 2 (I) associated with the PH. The value of pic_log2_diff_min_qt_min_cb_intra_slice_luma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When not present, the value of pic_log2_diff_min_qt_min_cb_luma is inferred to be equal to sps_log2_diff_min_qt_min_cb_intra_slice_luma if partition_constraints_override_flag is equal to 1 and inferred to be equal to 0 otherwise.

pic_log2_diff_min_qt_min_cb_inter_slice specifies the difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum luma coding block size in luma samples for luma CUs in the slices with slice_type equal to 0 (B) or 1 (P) associated with the PH. The value of pic_log2_diff_min_qt_min_cb_inter_slice shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When not present, the value of pic_log2_diff_min_qt_min_cb_luma is inferred to be equal to sps_log2_diff_min_qt_min_cb_inter_slice if partition_constraints_override_flag is equal to 1 and inferred to be equal to 0 otherwise.

pic_log2_diff_min_qt_min_cb_intra_slice_chroma specifies the difference between the base 2 logarithm of the minimum size in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA and the base 2 logarithm of the minimum coding block size in luma samples for chroma CUs with treeType equal to DUAL_TREE_CHROMA in slices with slice_type equal to 2 (I) associated with the PH. The value of pic_log2_diff_min_qt_min_cb_intra_slice_chroma shall be in the range of 0 to CtbLog2SizeY − MinCbLog2SizeY, inclusive. When not present, the value of pic_log2_diff_min_qt_min_cb_intra_slice_chroma is inferred to be equal to sps_log2_diff_min_qt_min_cb_intra_slice_chroma if partition_constraints_override_flag is equal to 1 and inferred to be equal to 0 otherwise.

# Conclusion

The contribution proposes a bug fix for Ticket #584 related to the CU partitioning along picture boundaries when MTT split is disabled. In the proposed method, the CU partitioning structure is reportedly reduced to the quadtree-based CU partitioning structure in HEVC when the specified MTT depth is equal to 0.

# References

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-P2001, 16th Meeting: Geneva, CH, 1–11 October 2019.

- Y.-K. Wang, B. Bross, and V. Drugeon, “AHG2: Editorial input on VVC draft text,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-Q0041, 17th Meeting: Brussels, BE, 7–17 January 2020.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).