| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0176-v1 |
| --- | --- |

| Title: | AHG9: Overhead reduction for picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes modified high-level syntax for reducing overhead bits. The proposed modifications are summarized as follows:

- Signal a new syntax element mixed_slice_types_in_pic_flag in the PPS. When mixed_slice_types_in_pic_flag is equal to 0, the slice type (B, P, or I) that is present in the current picture referring to the PPS is coded in the picture header. The syntax values related to the unused slice types are further skipped in the picture header.

- Skip coding mixed_nalu_types_in_pic_flag when the number of slices indicated by the PPS is equal to 1.

# Introduction

In VVC Draft 7 [1], some of coding parameters originally present in the slice header in VVC Draft 6 are moved to the new picture header structure. In a picture header, the coding parameters for all different slice types (I, P, and B) are signalled in order to support all slice types that may be present in a current picture. However, it is quite common that each coded picture in a coded video sequence consists of slices with the same slice type. The bits may be wasted for coding the syntax values in a picture header corresponding to the unused slice type(s) in the current picture.

This contribution proposes modified high-level syntax for reducing overhead bits in the picture header and the slice header. This contribution further proposes to skip coding the value of the syntax element mixed_nalu_types_in_pic_flag in the PPS when the number of slices indicated by the PPS is equal to 1.

# Proposed modifications

In the proposed modification, a syntax element mixed_slice_types_in_pic_flag is added to the PPS to specify whether each picture referring to the PPS may have more than one slice type present in the current picture. When mixed_slice_types_in_pic_flag is equal to 0, the slice type (B, P, or I) that is present in the current picture is coded by a new syntax element ph_slice_type in the associated picture header. The values of the syntax elements in the picture header related to the coding tools that are only applied to the unused slice types are further skipped. The value of the syntax element slice_type in the slice header can be inferred from the coded value of ph_slice_type.

In another proposed modification, the syntax element mixed_nalu_types_in_pic_flag is moved after the syntax element no_pic_partition_flag in the PPS. When the number of slices indicated by the PPS is equal to 1, mixed_nalu_types_in_pic_flag is not coded and is inferrd to be equal to 0.

The modifed syntax tables and semantics are provided in Secs 2.1 and 2.2, respectively.

## Modified syntax tables

The modifed syntax tables based on VVC Draft 7 [1] are provided below. The modified parts are further highlighted in magenta.

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| … |  |
| mixed_nalu_types_in_pic_flag | u(1) |
| … |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| …. |  |
| if( rect_slice_flag & & num_slices_in_pic_minus1 > 0 ) { |  |
| mixed_nalu_types_in_pic_flag | u(1) |
| mixed_slice_types_in_pic_flag | u(1) |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

#### 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| if( !mixed_slice_types_in_pic_flag ) |  |
| ph_slice_type | ue(v) |
| … |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( partition_constraints_override_flag ) { |  |
| if( mixed_slice_types_in_pic_flag ¦ ¦ ph_slice_type != I ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| } |  |
| if( mixed_slice_types_in_pic_flag ¦ ¦ ph_slice_type = = I ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| if( mixed_slice_types_in_pic_flag ¦ ¦ ph_slice_type != I ) { |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| pic_temporal_mvp_enabled_flag | u(1) |
| if(!pps_mvd_l1_zero_idc && (mixed_slice_types_in_pic_flag ¦ ¦ ph_slice_type == B) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| pic_five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| pic_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag && (mixed_slice_types_in_pic_flag ¦ ¦ ph_slice_type == B)) |  |
| pic_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag && (mixed_slice_types_in_pic_flag ¦ ¦ ph_slice_type == B)) |  |
| pic_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| pic_disable_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } |  |
| … |  |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.3.7.1 General slice header syntax

| slice_header ( ) { | Descriptor |
| --- | --- |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| if( mixed_slice_types_in_pic_flag |  |
| slice_type | ue(v) |
| … |  |
| } |  |

## Modified semantics

The semantic modifications based on VVC Draft 7 [1] are provided below. The modified parts are further highlighted in magenta.

#### 7.4.3.4 Picture parameter set RBSP semantics

mixed_nalu_types_in_pic_flag equal to 1 specifies that each picture referring to the PPS has more than one VCL NAL unit and that the VCL NAL units do not have the same value of nal_unit_type and that the picture is not an IRAP picture. mixed_nalu_types_in_pic_flag equal to 0 specifies that each picture referring to the PPS has one or more VCL NAL units and the VCL NAL units of each picture refering to the PPS have the same value of nal_unit_type. When mixed_nalu_types_in_pic_flag is not present, the value of mixed_nalu_types_in_pic_flag is inferred to be equal to 0.

When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of mixed_nalu_types_in_pic_flag shall be equal to 0.

For each slice with a nal_unit_type value nalUnitTypeA in the range of IDR_W_RADL to CRA_NUT, inclusive, in a picture picA that also contains one or more slices with another value of nal_unit_type (i.e., the value of mixed_nalu_types_in_pic_flag for the picture picA is equal to 1), the following applies:

–	The slice shall belong to a subpicture subpicA for which the value of the corresponding subpic_treated_as_pic_flag[ i ] is equal to 1.

–	The slice shall not belong to a subpicture of picA containing VCL NAL units with nal_unit_type not equal to nalUnitTypeA.

–	For all the following PUs in the CLVS in decoding order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA shall include any picture preceding picA in decoding order in an active entry.

mixed_slice_types_in_pic_flag equal to 1 specifies that each picture referring to the PPS may have more than one slice type present in the current picture. mixed_slice_types_in_pic_flag equal to 0 specifies that each picture referring to the PPS has one type present in the current picture. When mixed_slice_types_in_pic_flag is not present, the value of mixed_slice_types_in_pic_flag is inferred to be equal to 0.

#### 7.4.3.6 Picture header RBSP semantics

ph_slice_type specifies the value of slice_type in all the slice headers in the current picture when mixed_slice_types_in_pic_flag is equal to 0.

#### 7.4.8.1 General slice header semantics

slice_type specifies the coding type of the slice according to Table 9. When slice_type is not present, the value of slice_types is inferred to be equal to the value of ph_slice_type.

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 7),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-P2001, 16th Meeting: Geneva, CH, 1–11 October 2019.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).