| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0179-v2 |
| --- | --- |

| Title: | AHG9: Bitstream conformance requirement related to RPR scaling ratio for worst case MC memory bandwidth reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tzu-Der Chuang, Olena Chubach, Ching-Yeh Chen, Chi-Wei Hsu, Lulin Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {peter.chuang, olena.chubach, chingyeh.chen, cw.hsu, lulin.chen,, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In VVC, to support reference picture resampling (RPR), the worst case motion compensation (MC) memory bandwidth is more than doubled compared to RPR-off, which complicates the hardware design significantly. In this contribution, to prevent worst case MC memory bandwidth increase for supporting RPR, five methods of bitstream conformance requirement are proposed. It is strongly suggested that one of the five methods should be adopted in VVC for substantially reducing hardware complexity to support RPR.

Introduction

In VVC Draft 7, RPR and scalability support were adopted to allow MC from a reference picture with size different from the current picture. The scaling ratio RefPicScale, is derived from the scaling window size as follows.

The variables PicOutputWidthL and PicOutputHeightL are derived as follows:

PicOutputWidthL = pic_width_in_luma_samples − ( scaling_win_right_offset + scaling_win_left_offset )

PicOutputHeightL = pic_height_in_luma_samples − ( scaling_win_bottom_offset + scaling_win_top_offset )

The reference picture scaling ratios RefPicScale[ i ][ j ][ 0 ] and RefPicScale[ i ][ j ][ 1 are constructed as follows.

fRefWidth is set equal to PicOutputWidthL of the reference picture RefPicList[ i ][ j ] in luma samples

fRefHeight is set equal to PicOutputHeightL of the reference picture RefPicList[ i ][ j ] in luma samples

RefPicScale[ i ][ j ][ 0 ] = ( ( fRefWidth  <<  14 ) + ( PicOutputWidthL  >>  1 ) ) / PicOutputWidthL

RefPicScale[ i ][ j ][ 1 ] = ( ( fRefHeight  <<  14 ) + ( PicOutputHeightL  >>  1 ) ) / PicOutputHeightL

The RefPicScale is used in MC process for on-the-fly upsampling and downsampling. For example, in MC process, the reference sample position (refxL, refyL) are derived as follows.

- The scalingRatio[ 0 ] and scalingRatio[ 1 ] are the RefPicScale[ i ][ j ][ 0 ] and RefPicScale[ i ][ j ][ 1 ]. A luma location ( xSb, ySb ) specifying the top-left sample of the current coding subblock relative to the top-left luma sample of the current picture. For each luma sample location ( xL, yL) inside the current coding subblock, the corresponding prediction luma sample position (refxL, refyL) is derived as follows:

The variable fRefLeftOffset is set equal to ( scaling_win_left_offset  <<  10 ), where scaling_win_left_offset is the scaling_win_left_offset for the reference picture.

The variable fRefTopOffset is set equal to ( scaling_win_top_offset  <<  10 ), where scaling_win_top_offset is the scaling_win_top_offset for the reference picture.

…… (scaling_win_left_offset and scaling_win_top_offset below are for the current picture)

refxSbL = ( ( ( xSb − scaling_win_left_offset )  <<  4 ) + refMvLX[ 0 ] ) * scalingRatio[ 0 ]

refxL = ( ( Sign( refxSbL ) * ( ( Abs( refxSbL ) + 128 )  >>  8 )
		+ xL * ( ( scalingRatio[ 0 ] + 8 )  >>  4 ) ) + fRefLeftOffset + 32 )  >>  6

refySbL = ( ( ( ySb − scaling_win_top_offset ) <<  4 ) + refMvLX[ 1 ] ) * scalingRatio[ 1 ]

refyL = ( ( Sign( refySbL ) * ( ( Abs( refySbL ) + 128 )  >>  8 ) + yL *
		( ( scalingRatio[ 1 ] + 8 )  >>  4 ) ) + fRefTopOffset + 32 )  >>  6

In video codec implementation, the required MC memory bandwidth is an important parameter for the entire system. Usually, the worst case MC memory bandwidth can be derived by using picture size. For example, the worst case MC memory bandwidth for a current picture is equal to CurPicWidth * CurPicHeight * WorstCaseBlockBW / WorstCaseBlockSize, where CurPicWidth and CurPicHeight are the width and height of the current picture, respectively. In VVC, the WorstCaseBlockSize is 64 (16x4 or 4x16 block), and the WorstCaseBlockBW is 506 ((16+7)*(4+7)*2 for a 16x4/4x16 bi-predicted block). As long as the picture size is determined, the required MC memory bandwidth can also be determined. However, when RPR is used, the worst case MC memory bandwidth will be larger than expected. For example, Fig. 1 shows an example of reference picture structure and scaling window setting. Arrows show the picture referencing. All five pictures have the same picture size. For the picture with picture order count (POC) equal to 0 or 4, the scaling window size is the same as the picture size. For the rest pictures, the scaling window width and height are half of the picture width and height, respectively. The effective scaling ratio is 2 in horizontal and vertical direction (e.g. scalingRatio[ 0 ] and scalingRatio[ 1 ] are equal to 32768). For a 16x4 luma block, if the effective scaling ratio is 2, its corresponding reference block is a (16*2+7)*(4*2+7) block when the 8-tap interpolation filter is used, which requires 2.31x bandwidth (WorstCaseBlockBW=1170) compared to RPR-off (WorstCaseBlockBW=506). Therefore, for the picture with POC equal to 4, the worst case MC bandwidth is not change, but for POC 1, 2, or 3, the worst case MC bandwidth is 2.31x of that of POC 4. The entire worst case MC bandwidth increases a lot when RPR is applied.

Fig. 1. Example of reference picture structure and scaling window setting

Proposed bitstream conformance requirement

In this contribution, four methods are proposed to prevent the worst case MC memory bandwidth increase.

Method-1

The worst case MC memory bandwidth of a current picture is equal to CurPicWidth * CurPicHeight * WorstCaseBlockBW / WorstCaseBlockSize. Therefore, if the following conditions are satisfied, the worst case MC memory bandwidth is not increased. Both 16x4 and 4x16 blocks are used to calculate the worst case MC memory bandwidth.

CurPicWidth * CurPicHeight * (16 * ScalingRatioX + 7) * (4 * ScalingRatioY + 7) / (16 * 4) <= SpsMaxPicWidth * SpsMaxPicHeight * (16 + 7) * (4 + 7) / (16 * 4),

CurPicWidth * CurPicHeight * (4 * ScalingRatioX + 7) * (16 * ScalingRatioY + 7) / (4 * 16) <= SpsMaxPicWidth * SpsMaxPicHeight * (4 + 7) * (16 + 7) / (4 * 16),

where the ScalingRatioX is equal to (refPicOutputWidthL / PicOutputWidthL), the ScalingRatioY is equal to (refPicOutputHeightL / PicOutputHeightL), SpsMaxPicWidth is the maximum picture width signalled in SPS, and SpsMaxPicHeight is the maximum picture height signalled in SPS.

After formula simplification, it can be rewritten as follows.

CurPicWidth * CurPicHeight * (16 * refPicOutputWidthL + 7 * PicOutputWidthL) * (4 * refPicOutputHeightL + 7 * PicOutputHeightL) <= SpsMaxPicWidth * SpsMaxPicHeight * 253 * PicOutputWidthL * PicOutputHeightL

CurPicWidth * CurPicHeight * (4 * refPicOutputWidthL + 7 * PicOutputWidthL) * (16 * refPicOutputHeightL + 7 * PicOutputHeightL) <= SpsMaxPicWidth * SpsMaxPicHeight * 253 * PicOutputWidthL * PicOutputHeightL

The proposed text for the scaling window sizes is as follows and highlighted in yellow.

scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset specify the offsets, in units of luma samples, that are applied to the picture size for scaling ratio calculation. When scaling_window_flag is equal to 0, the values of scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset are inferred to be equal to 0.

The value of scaling_win_left_offset + scaling_win_right_offset shall be less than pic_width_in_luma_samples, and the value of scaling_win_top_offset + scaling_win_bottom_offset shall be less than pic_height_in_luma_samples.

The variables PicOutputWidthL and PicOutputHeightL are derived as follows:

PicOutputWidthL = pic_width_in_luma_samples −	(74)
			( scaling_win_right_offset + scaling_win_left_offset )

PicOutputHeightL = pic_height_in_luma_samples −	(75)
			( scaling_win_bottom_offset + scaling_win_top_offset )

When scaling_window_flag is equal to 1, let refPicOutputWidthL and refPicOutputHeightL be the PicOutputWidthL and PicOutputHeightL, respectively, of a reference picture of the current picture referring to this PPS. It’s a requirement of bitstream conformance that the following conditions are satisfied:

–     pic_width_in_luma_samples * pic_height_in_luma_samples * (16 * refPicOutputWidthL + 7 * PicOutputWidthL) * (4 * refPicOutputHeightL + 7 * PicOutputHeightL) shall be smaller than or equal to pic_width_max_in_luma_samples * pic_height_max_in_luma_samples height * 253 * PicOutputWidthL * PicOutputHeightL

–     pic_width_in_luma_samples * pic_height_in_luma_samples * (4 * refPicOutputWidthL + 7 * PicOutputWidthL) * (16 * refPicOutputHeightL + 7 * PicOutputHeightL) shall be smaller than or equal to pic_width_max_in_luma_samples * pic_height_max_in_luma_samples height * 253 * PicOutputWidthL * PicOutputHeightL

Method-2

Method-2 is modified from Method-1. The filter length of the interpolation filter is not considered for simplifying the constraint. The proposed text is as follows and highlighted in yellow.

scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset specify the offsets, in units of luma samples, that are applied to the picture size for scaling ratio calculation. When scaling_window_flag is equal to 0, the values of scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset are inferred to be equal to 0.

The value of scaling_win_left_offset + scaling_win_right_offset shall be less than pic_width_in_luma_samples, and the value of scaling_win_top_offset + scaling_win_bottom_offset shall be less than pic_height_in_luma_samples.

The variables PicOutputWidthL and PicOutputHeightL are derived as follows:

PicOutputWidthL = pic_width_in_luma_samples −	(74)
			( scaling_win_right_offset + scaling_win_left_offset )

PicOutputHeightL = pic_height_in_luma_samples −	(75)
			( scaling_win_bottom_offset + scaling_win_top_offset )

When scaling_window_flag is equal to 1, let refPicOutputWidthL and refPicOutputHeightL be the PicOutputWidthL and PicOutputHeightL, respectively, of a reference picture of the current picture referring to this PPS. It’s a requirement of bitstream conformance that all of the following conditions are satisfied:

–     pic_width_in_luma_samples * pic_height_in_luma_samples * refPicOutputWidthL * refPicOutputHeightL shall be smaller than or equal to pic_width_max_in_luma_samples * pic_height_max_in_luma_samples height * PicOutputWidthL * PicOutputHeightL

Method-3

Method-3 is further simplified from Method-2 by dividing the required condition in Method 2 into two conditions, to keep the calculation within 32 bits for 8K video. The filter length of the interpolation filter is not considered. The proposed text is as follows and highlighted in yellow.

scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset specify the offsets, in units of luma samples, that are applied to the picture size for scaling ratio calculation. When scaling_window_flag is equal to 0, the values of scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset are inferred to be equal to 0.

The value of scaling_win_left_offset + scaling_win_right_offset shall be less than pic_width_in_luma_samples, and the value of scaling_win_top_offset + scaling_win_bottom_offset shall be less than pic_height_in_luma_samples.

The variables PicOutputWidthL and PicOutputHeightL are derived as follows:

PicOutputWidthL = pic_width_in_luma_samples −	(74)
			( scaling_win_right_offset + scaling_win_left_offset )

PicOutputHeightL = pic_height_in_luma_samples −	(75)
			( scaling_win_bottom_offset + scaling_win_top_offset )

When scaling_window_flag is equal to 1, let refPicOutputWidthL and refPicOutputHeightL be the PicOutputWidthL and PicOutputHeightL, respectively, of a reference picture of the current picture referring to this PPS. It’s a requirement of bitstream conformance that all of the following conditions are satisfied:

–    pic_width_in_luma_samples * refPicOutputWidthL shall be smaller than or equal to pic_width_max_in_luma_samples * PicOutputWidthL

–    pic_height_in_luma_samples * refPicOutputHeightL shall be smaller than or equal to pic_height_max_in_luma_samples * PicOutputHeightL

Method-4

In the case of reference picture is full-HD (e.g. 1920x1080) and the current picture is quarter-size of full-HD (e.g. 960x544), the scaling window of reference picture is 1920x1080 and the scaling window of the current picture if 960x540, the height constraint is not satisfied. Therefore, the constraint in Method-3 can be loose a little bit and modified as follow.

–    (pic_width_in_luma_samples – Max( 8, MinCbSizeY )) * refPicOutputWidthL shall be smaller than or equal to pic_width_max_in_luma_samples * PicOutputWidthL

–    (pic_height_in_luma_samples – Max( 8, MinCbSizeY )) * refPicOutputHeightL shall be smaller than or equal to pic_height_max_in_luma_samples * PicOutputHeightL

Method-5

The method-5 is a simplified version of method-1. The 8x8 block is used to derive the constraint.

CurPicWidth * CurPicHeight * (8 * ScalingRatioX + 7) * (4 * ScalingRatioY + 7) / (8 * 8) <= SpsMaxPicWidth * SpsMaxPicHeight * (8 + 7) * (8 + 7) / (8 * 8)

After formula simplification, it can be rewritten as follows.

CurPicWidth * CurPicHeight * (8 * refPicOutputWidthL + 7 * PicOutputWidthL) * (8 * refPicOutputHeightL + 7 * PicOutputHeightL) <= SpsMaxPicWidth * SpsMaxPicHeight * 225 * PicOutputWidthL * PicOutputHeightL

The proposed text for the scaling window sizes is as follows and highlighted in yellow.

scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset specify the offsets, in units of luma samples, that are applied to the picture size for scaling ratio calculation. When scaling_window_flag is equal to 0, the values of scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset are inferred to be equal to 0.

The value of scaling_win_left_offset + scaling_win_right_offset shall be less than pic_width_in_luma_samples, and the value of scaling_win_top_offset + scaling_win_bottom_offset shall be less than pic_height_in_luma_samples.

The variables PicOutputWidthL and PicOutputHeightL are derived as follows:

PicOutputWidthL = pic_width_in_luma_samples −	(74)
			( scaling_win_right_offset + scaling_win_left_offset )

PicOutputHeightL = pic_height_in_luma_samples −	(75)
			( scaling_win_bottom_offset + scaling_win_top_offset )

When scaling_window_flag is equal to 1, let refPicOutputWidthL and refPicOutputHeightL be the PicOutputWidthL and PicOutputHeightL, respectively, of a reference picture of the current picture referring to this PPS. It’s a requirement of bitstream conformance that the following conditions are satisfied:

–     pic_width_in_luma_samples * pic_height_in_luma_samples * (8 * refPicOutputWidthL + 7 * PicOutputWidthL) * (8 * refPicOutputHeightL + 7 * PicOutputHeightL) shall be smaller than or equal to pic_width_max_in_luma_samples * pic_height_max_in_luma_samples height * 225 * PicOutputWidthL * PicOutputHeightL

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).