| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0184-v1 |
| --- | --- |

| Title: | AHG9: On signalling of wrap-around motion compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Yao Chiu, Chun-Chia Chen, Chih-Wei Hsu, Lulin Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {colin.chiu, chunchia.chen, cw.hsu, lulin.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC, horizontal wrap-around motion compensation (MC), where a wrap-around offset is specified, is allowed and suggested to be used for 360-degree video in equi-rectangular projection (ERP) format. In VVC Draft 7, pictures referring to the same sequence parameter set (SPS) must share the same wrap-around offset. That is, it is constant throughout all pictures in the sequence, which may cause some problems for wrap-around operations when reference picture resampling (RPR) is enabled or inter-layer reference picture (ILRP) is used for inter prediction. In this contribution, to solve the issue easily and quickly, it is proposed to allow the wrap-around MC only when RPR is disabled and no ILRP is used.

# Introduction

In VVC Draft 7 [1], horizontal wrap-around MC is designed to improve the coding efficiency of 360-degree video in ERP format. One example of the horizontal wrap-around MC is shown in Figure 1. The ERP width is the width of the original ERP picture, which is also the wrap-around offset signalled in SPS. The encoded picture is both left and right padded to the ERP picture. During MC, when a part of the reference block is outside the left boundary of reference picture, the position of “out-of-boundary” part is taken as the original position plus wrap-around offset as shown in Figure 1. For another case, when a part of the reference block is outside the right boundary of reference picture, the position of “out-of-boundary” part is taken as the original position minus wrap-around offset.

Figure 1 – Example of horizontal wrap-around MC in VVC

On the other hand, RPR is designed to provide scalability including spatial resolution scalability, SNR scalability, etc. An example of RPR is shown in Figure 2, where the current picture is predicted from reference pictures of different sizes.

Figure 2 －An example of reference picture resampling

The RPR may be activated by two means. The first one is that when the ref_pic_resampling_enabled_flag signalled in SPS is equal to true, the pictures referring to the SPS may have different resolutions. Therefore, the reference picture could have a different size from the current picture. The second one is when inter-layer reference picture (ILRP) is used as a reference picture for inter prediction of current picture. According to the definition of ILRP, reference picture should have nuh_layer_id less than that of the current picture and pictures in a different coded layer (that is, different nuh_layer_id) may have a different resolution. Therefore, the ILRP could have a different size from the current picture.

# Problem description

As described in the previous section, a wrap-around offset is used to calculate the horizontal wrap-around position of reference block and is ERP width for one picture. When ref_pic_resampling_enabled_flag signalled in SPS is true or ILRP is used, the picture size of reference pictures may be different from that of current picture. As a result, the wrap-around offset shall varies depending on the reference picture size. However, in the VVC Draft 7 [1], the wrap-around offset is signalled in SPS. Namely, pictures referring to the same SPS must share the same wrap-around offset, which means a wrong wrap-around offset may be used when ref_pic_resampling_enabled_flag is true or ILRP is used. The example is shown in Figure 3.

Figure 3 －An example showing wrap-around MC goes wrong

Moreover, there is one bitstream conformance requirement for sps_ref_wraparound_enabled_flag as follows.

When the value of ( CtbSizeY / MinCbSizeY + 1) is greater than or equal to ( pic_width_in_luma_samples / MinCbSizeY − 1 ), where pic_width_in_luma_samples is the value of pic_width_in_luma_samples in any PPS that refers to the SPS, the value of sps_ref_wraparound_enabled_flag shall be equal to 0.

However, sps_ref_wraparound_enabled_flag is signalled in SPS and pic_width_in_luma_samples may change between pictures, so it is impossible to check the bitstream conformance requirement when picture size changes.

# Proposed method

To solve the issue mentioned above, it is proposed to support wrap-around MC only when ref_pic_resampling_enabled_flag is false and no ILRP is used for inter prediction. In this case, it is guaranteed the resolutions of reference pictures are all the same as current picture resolution when the wrap-around MC is activated, and a fixed wrap-around offset can be signalled in SPS without any problem. Furthermore, it is possible to check the bitstream conformance requirement for sps_ref_wraparound_enabled_flag once at SPS level.

The modification of working draft is shown as below (Marked as highlighted):

Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … … |  |
| if(! ref_pic_resampling_enabled_flag && ! inter_layer_ref_pics_present_flag){ |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| } |  |
| … … |  |
| } |  |

Sequence parameter set RBSP semantics:

ref_pic_resampling_enabled_flag equal to 1 specifies that reference picture resampling may be applied when decoding coded pictures in the CLVSs referring to the SPS. ref_pic_resampling_enabled_flag equal to 0 specifies that reference picture resampling is not applied when decoding pictures in CLVSs referring to the SPS.

inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CLVS. inter_layer_ref_pic_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0.

sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that horizontal wrap-around motion compensation is not applied. When the value of ( CtbSizeY / MinCbSizeY + 1) is greater than or equal to ( pic_width_max_in_luma_samples / MinCbSizeY − 1 ), the value of sps_ref_wraparound_enabled_flag shall be equal to 0. When sps_ref_wraparound_enabled_flag is not present, the value of sps_ref_wraparound_enabled_flag is inferred to be equal to 0.

[Ed. (YK): Instead of specifying the value constraint this way, it's probably better to move sps_ref_wraparound_enabled_flag and sps_ref_wraparound_offset_minus1 from the SPS to the PPS.]

sps_ref_wraparound_offset_minus1 plus 1 specifies the offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. The value of ref_wraparound_offset_minus1 shall be in the range of ( CtbSizeY / MinCbSizeY ) + 1 to ( pic_width_max_in_luma_samples / MinCbSizeY ) − 1, inclusive. When sps_ref_wraparound_offset_minus1 is not present, the value of sps_ref_wraparound_offset_minus1 is inferred to be equal to 0.

Note that the normative constraint of sps_ref_wraparound_enabled_flag descripted in VVC Draft 7 is wrong. It is corrected in this proposal, marked as highlighted.

# Conclusion

This contribution proposes to modify the signalling of wrap-around MC in VVC. It is found that in VVC Draft 7 there are problems for wrap-around MC when the resolutions of reference picture and current picture are different. The proposed method solves the issue by disallowing wrap-around MC when ref_pic_resampling_enabled_flag is enabled or ILRP is used. It is suggested to adopt the proposed method into VVC.

# References

[1] JVET-P2001-vE, Versatile Video Coding (Draft 7), Geneva, November 2019

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).