| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0187-v1 |
| --- | --- |

| Title: | CE3-related: Rice parameter derivation in residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In the current VTM7.0, the rice parameter is always set to 1 in transform skip residual coding (TSRC), which decreases the bit savings in lossless coding since using rice parameter equal to 1 is not efficient especially when the coefficients (i.e., residuals) tend to be larger. This contribution proposes four methods to improve TSRC by using rice parameter other than 1.

Method 1.1 reuses the rice parameter table of regular residual coding (RRC) in the rice parameter derivation of TSRC, the rice parameter derivation is as below.

rice_tsrc = RRC_rice_table[ Clip3(0, 31, absSum – 2*baseLevel + 5)]

Based on Method 1.1, Method 1.2 applies normalization to the sum of the absolute coefficients in the local template for TSRC, which further considers the number of in-the-same-TB coefficients in the local template.

rice_tsrc = RRC_rice_table[ Clip3(0, 31, ((numAvail == 1)? 2* absSum: absSum) – 2*baseLevel + 5)]

Method 2.1 removes the rice parameter table in RRC. The rice parameter derivation is as below.

rice_rrc = Clip3(0, 39, absSum – 5*baseLevel) >> 3

rice_tsrc = Clip3(0, 39, absSum – 2*baseLevel + 5) >> 3

Based on Method 2.1, Method 2.2 applies normalization to the sum of the absolute coefficients in the local template for TSRC, which further considers the number of in-the-same-TB coefficients in the local template.

rice_rrc = Clip3(0, 39, absSum – 5*baseLevel) >> 3

rice_tsrc = Clip3(0, 39, ((numAvail == 1)? 2* absSum: absSum) – 2*baseLevel + 5) >> 3

The compression performance of the four tests are listed below.

Method 1.1:
[classes CTC including A1, A2, B, C, E]
VTM7.0 - standard QPs: { luma BD-rate = -0.01%(AI), 0.00%(RA), -0.02%(LB) }
VTM7.0 - low QPs:    { luma BD-rate = -0.07%(AI), X.XX%(RA), -0.02%(LB) }
VTM7.0 - lossless:     {     bit-rate = -4.90%(AI), -3.69%(RA), -2.39%(LB) }
[class TGM]
VTM7.0 - standard QPs: { luma BD-rate = -0.37%(AI), -0.18%(RA), -0.27%(LB) }
VTM7.0 - low QPs:    { luma BD-rate = -1.09%(AI), -1.82%(RA), -0.94%(LB) }
VTM7.0 - lossless:     {     bit-rate = -3.80%(AI), -6.00%(RA), -6.00%(LB) }

Method 1.2:
[classes CTC including A1, A2, B, C, E]
VTM7.0 - standard QPs: { luma BD-rate = -0.01%(AI), -0.01%(RA), -0.04%(LB) }
VTM7.0 - low QPs:    { luma BD-rate = -0.07%(AI), X.XX%(RA), -0.02%(LB) }
VTM7.0 - lossless:     {     bit-rate = -5.15%(AI), -3.92%(RA), -2.35%(LB) }
[class TGM]
VTM7.0 - standard QPs: { luma BD-rate = -0.35%(AI), -0.21%(RA), -0.29%(LB) }
VTM7.0 - low QPs:    { luma BD-rate = -1.33%(AI), -1.97%(RA), -1.14%(LB) }
VTM7.0 - lossless:     {     bit-rate = -4.20%(AI), -6.26%(RA), -6.33%(LB) }

Method 2.1:
[classes CTC including A1, A2, B, C, E]
VTM7.0 - standard QPs: { luma BD-rate = 0.01%(AI), 0.00%(RA), X.XX%(LB) }
VTM7.0 - low QPs:    { luma BD-rate = -0.01%(AI), X.XX%(RA), -0.06%(LB) }
VTM7.0 - lossless:     {     bit-rate = -4.89%(AI), -3.32%(RA), -2.49%(LB) }
[class TGM]
VTM7.0 - standard QPs: { luma BD-rate = -0.37%(AI), -0.16%(RA), -0.28%(LB) }
VTM7.0 - low QPs:    { luma BD-rate = -1.45%(AI), -2.14%(RA), -1.18%(LB) }
VTM7.0 - lossless:     {     bit-rate = -4.69%(AI), -6.63%(RA), -6.75%(LB) }

Method 2.2:
[classes CTC including A1, A2, B, C, E]
VTM7.0 - standard QPs: { luma BD-rate = 0.01%(AI), -0.01%(RA), 0.01%(LB) }
VTM7.0 - low QPs:    { luma BD-rate = -0.02%(AI), X.XX%(RA), -0.06%(LB) }
VTM7.0 - lossless:     {     bit-rate = -5.19%(AI), -3.58%(RA), -6.94%(LB) }
[class TGM]
VTM7.0 - standard QPs: { luma BD-rate = -0.38%(AI), -0.10%(RA), -0.11%(LB) }
VTM7.0 - low QPs:    { luma BD-rate = -1.69%(AI), -2.33%(RA), -1.27%(LB) }
VTM7.0 - lossless:     {     bit-rate = -5.08%(AI), -6.89%(RA), -6.94%(LB) }

# Introduction

In the current VTM7.0 [1], lossless coding is achieved by forcing encoder to choose transform skip mode. However, in the 16th JVET meeting, it was observed that the bit savings using TSRC for transform skip is not as good as those using RRC for transform skip. Moreover, after JVET-P0562 [2] was adopted, the rice parameter is always set to 1. This simplification makes the performance differences between using TSRC and RRC even larger because in lossless coding, the coefficients (i.e., residuals) tend to be larger, which requires a larger rice parameter for the coefficient coding. This contribution proposes four methods to improve the coding gain of TSRC.

# Proposed methods

## Method 1.1: TSRC reuses RRC’s rice parameter table without normalization

In Method 1.1, the rice parameter of TSRC is derived as below:

rice_tsrc = RRC_rice_table[ Clip3(0, 31, absSum – 2*baseLevel + 5)],

where RRC_rice_table is the rice parameter table for RRC, variable absSum is the sum of the absolute neighbouring coefficients in the local template, and variable baseLevel is the base level of the current coefficient.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code: - If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 locSumAbs = 0 if( xC > 0 ) { locSumAbs += AbsLevel[ xC − 1 ][ yC ] (xxxx) } if( yC > 0 ) { locSumAbs += AbsLevel[ xC ][ yC - 1 ] } locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 2 + 5 ) - Otherwise, locSumAbs = 0 if( xC < (1 << log2TbWidth) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < (1 << log2TbWidth) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < (1 << log2TbHeight) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1494) } if( yC < (1 << log2TbHeight) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < (1 << log2TbHeight) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 124. When baseLevel is equal to 0, the variable ZeroPos[ n ] is derived as follows: ZeroPos[ n ] = ( QState < 2 ? 1 : 2 ) << cRiceParam (1495) Table 124 – Specification of cRiceParam based on locSumAbs locSumAbs; 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15. cRiceParam; 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1; 1; 1; 1; 2; 2. locSumAbs; 16; 17; 18; 19; 20; 21; 22; 23; 24; 25; 26; 27; 28; 29; 30; 31. cRiceParam; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 3; 3; 3; 3. … #### 9.3.3.11 Binarization process for abs_remainder[ ] Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the binarization of the syntax element. The variables lastAbsRemainder and lastRiceParam are derived as follows: - If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0. - Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this clause. The rice parameter cRiceParam is derived as follows: - The variable currBaseLevel is derived as: - If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 currBaseLevel = (n <= lastScanPosPass2) ? 10 : (n <= lastScanPosPass1)? 2 : 0 - Otherwise, currBaseLevel = 4 - The rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to currBaseLevel, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs. …

## Method 1.2: TSRC reuses RRC’s rice parameter table with normalization

Based on Method 1.1, it is proposed to further apply normalization to the sum of the neighbouring coefficients in the local template for TSRC.

In Method 1.2, the rice parameter of TSRC is derived as below:

rice_tsrc = RRC_rice_table[ Clip3(0, 31, absSum_norm – 2*baseLevel + 5)],

where RRC_rice_table is the rice parameter table for RRC, variable absSum_norm is the normalized sum of the absolute neighbouring coefficients in the local template, and variable baseLevel is the base level of the current coefficient. The variable absSum_norm is derived as

absSum_norm = (numAvail == 1)? 2 * absSum : absSum,

where the variable numAvail is the number of in-the-same-TB neighbouring coefficients in the local template and variable absSum is the sum of the absolute neighbouring coefficients in the local template.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code: - If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 locSumAbs = 0 numAvail = 0 if( xC > 0 ) { locSumAbs += AbsLevel[ xC − 1 ][ yC ] (xxxx) numAvail += 1 } if( yC > 0 ) { locSumAbs += AbsLevel[ xC ][ yC - 1 ] numAvail += 1 } locSumAbs = (numAvail == 1)? 2 * locSumAbs : locSumAbs locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 2 + 5 ) - Otherwise, locSumAbs = 0 if( xC < (1 << log2TbWidth) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < (1 << log2TbWidth) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < (1 << log2TbHeight) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1494) } if( yC < (1 << log2TbHeight) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < (1 << log2TbHeight) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 124. When baseLevel is equal to 0, the variable ZeroPos[ n ] is derived as follows: ZeroPos[ n ] = ( QState < 2 ? 1 : 2 ) << cRiceParam (1495) Table 124 – Specification of cRiceParam based on locSumAbs locSumAbs; 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15. cRiceParam; 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1; 1; 1; 1; 2; 2. locSumAbs; 16; 17; 18; 19; 20; 21; 22; 23; 24; 25; 26; 27; 28; 29; 30; 31. cRiceParam; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 3; 3; 3; 3. … #### 9.3.3.11 Binarization process for abs_remainder[ ] Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the binarization of the syntax element. The variables lastAbsRemainder and lastRiceParam are derived as follows: - If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0. - Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this clause. The rice parameter cRiceParam is derived as follows: - The variable currBaseLevel is derived as: - If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 currBaseLevel = (n <= lastScanPosPass2)? 10 : (n <= lastScanPosPass1)? 2 : 0 - Otherwise, currBaseLevel = 4 - The rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to currBaseLevel, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs. …

## Method 2.1: remove rice table in both RRC and TSRC without normalization

Method 2.1 proposes to remove the rice parameter table in both RRC and TSRC.

In Method 2.1, the rice parameter derivation in RRC is as below.

rice_rrc = Clip3(0, 39, absSum – 5*baseLevel) >> 3,

where variable absSum is the sum of the absolute neighbouring coefficients in the local template, and variable baseLevel is the base level of the current coefficient.

In Method 2.1, the rice parameter derivation in TSRC is as below.

rice_tsrc = Clip3(0, 39, absSum – 2*baseLevel + 5) >> 3,

where variable absSum is the sum of the absolute neighbouring coefficients in the local template, and variable baseLevel is the base level of the current coefficient.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code: - If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 locSumAbs = 0 if( xC > 0 ) { locSumAbs += AbsLevel[ xC − 1 ][ yC ] (xxxx) } if( yC > 0 ) { locSumAbs += AbsLevel[ xC ][ yC - 1 ] } locSumAbs = Clip3( 0, 39, locSumAbs − baseLevel * 2 + 5 ) - Otherwise, locSumAbs = 0 if( xC < (1 << log2TbWidth) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < (1 << log2TbWidth) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < (1 << log2TbHeight) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1494) } if( yC < (1 << log2TbHeight) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < (1 << log2TbHeight) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } locSumAbs = Clip3( 0, 39, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as follows: cRiceParam = locSumAbs >> 3 When baseLevel is equal to 0, the variable ZeroPos[ n ] is derived as follows: ZeroPos[ n ] = ( QState < 2 ? 1 : 2 ) << cRiceParam (1495) Table 124 – Specification of cRiceParam based on locSumAbs locSumAbs; 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15. cRiceParam; 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1; 1; 1; 1; 2; 2. locSumAbs; 16; 17; 18; 19; 20; 21; 22; 23; 24; 25; 26; 27; 28; 29; 30; 31. cRiceParam; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 3; 3; 3; 3. … #### 9.3.3.11 Binarization process for abs_remainder[ ] Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the binarization of the syntax element. The variables lastAbsRemainder and lastRiceParam are derived as follows: - If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0. - Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this clause. The rice parameter cRiceParam is derived as follows: - The variable currBaseLevel is derived as: - If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 currBaseLevel = (n <= lastScanPosPass2)? 10 : (n <= lastScanPosPass1)? 2 : 0 - Otherwise, currBaseLevel = 4 - The rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to currBaseLevel, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs. …

## Method 2.2: remove rice table in both RRC and TSRC with normalization

Based on Method 2.1, it is proposed to further apply normalization to the sum of the neighbouring coefficients in the local template for TSRC.

In Method 2.2, the rice parameter derivation in RRC is the same as Method 2.1.

In Method 2.2, the rice parameter derivation in TSRC is as below.

rice_tsrc = Clip3(0, 39, absSum_norm – 2*baseLevel + 5) >> 3,

where variable absSum_norm is the normalized sum of the absolute neighbouring coefficients in the local template, and variable baseLevel is the base level of the current coefficient. The variable absSum_norm is derived as

absSum_norm = (numAvail == 1)? 2 * absSum : absSum,

where the variable numAvail is the number of in-the-same-TB neighbouring coefficients in the local template and variable absSum is the sum of the absolute neighbouring coefficients in the local template.

#### 9.3.3.2 Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ] Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the Rice parameter cRiceParam. Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code: - If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 locSumAbs = 0 numAvail = 0 if( xC > 0 ) { locSumAbs += AbsLevel[ xC − 1 ][ yC ] (xxxx) numAvail += 1 } if( yC > 0 ) { locSumAbs += AbsLevel[ xC ][ yC - 1 ] numAvail += 1 } locSumAbs = (numAvail == 1)? 2 * locSumAbs : locSumAbs locSumAbs = Clip3( 0, 39, locSumAbs − baseLevel * 2 + 5 ) - Otherwise, locSumAbs = 0 if( xC < (1 << log2TbWidth) − 1 ) { locSumAbs += AbsLevel[ xC + 1 ][ yC ] if( xC < (1 << log2TbWidth) − 2 ) locSumAbs += AbsLevel[ xC + 2 ][ yC ] if( yC < (1 << log2TbHeight) − 1 ) locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ] (1494) } if( yC < (1 << log2TbHeight) − 1 ) { locSumAbs += AbsLevel[ xC ][ yC + 1 ] if( yC < (1 << log2TbHeight) − 2 ) locSumAbs += AbsLevel[ xC ][ yC + 2 ] } locSumAbs = Clip3( 0, 39, locSumAbs − baseLevel * 5 ) Given the variable locSumAbs, the Rice parameter cRiceParam is derived as follows: cRiceParam = locSumAbs >> 3 When baseLevel is equal to 0, the variable ZeroPos[ n ] is derived as follows: ZeroPos[ n ] = ( QState < 2 ? 1 : 2 ) << cRiceParam (1495) Table 124 – Specification of cRiceParam based on locSumAbs locSumAbs; 0; 1; 2; 3; 4; 5; 6; 7; 8; 9; 10; 11; 12; 13; 14; 15. cRiceParam; 0; 0; 0; 0; 0; 0; 0; 1; 1; 1; 1; 1; 1; 1; 2; 2. locSumAbs; 16; 17; 18; 19; 20; 21; 22; 23; 24; 25; 26; 27; 28; 29; 30; 31. cRiceParam; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 2; 3; 3; 3; 3. … #### 9.3.3.11 Binarization process for abs_remainder[ ] Input to this process is a request for a binarization for the syntax element abs_remainder[ n ], the colour component cIdx, the current sub-block index i, and the luma location ( x0, y0 ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight. Output of this process is the binarization of the syntax element. The variables lastAbsRemainder and lastRiceParam are derived as follows: - If this process is invoked for the first time for the current sub-block index i, lastAbsRemainder and lastRiceParam are both set equal to 0. - Otherwise (this process is not invoked for the first time for the current sub-block index i), lastAbsRemainder and lastRiceParam are set equal to the values of abs_remainder[ n ] and cRiceParam, respectively, that have been derived during the last invocation of the binarization process for the syntax element abs_remainder[ n ] as specified in this clause. The rice parameter cRiceParam is derived as follows: - The variable currBaseLevel is derived as: - If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1 currBaseLevel = (n <= lastScanPosPass2)? 10 : (n <= lastScanPosPass1)? 2 : 0 - Otherwise, currBaseLevel = 4 - The rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to currBaseLevel, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs. …

# Experiment results

The proposed methods have been implemented on VTM7.0. All of the proposed methods were tested under common test condition (CTC) [3] using standard QPs (22, 27, 32, 27), CTC using low QPs (2, 7, 12, 17), and lossless CTC (QP=4, lossless configuration settings, and disabling BDPCM in all sequences). For low QP tests and lossless tests, 100 frames are tested in AI, RA, and LB. Table 1, Table 2, and Table 3 show the results of Method 1.1 under CTC standard QPs, CTC low QPs, and lossless CTC, respectively. Table 4, Table 5, and Table 6 show the results of Method 1.2 under CTC standard QPs, CTC low QPs, and lossless CTC, respectively. Table 7, Table 8, and Table 9 show the results of Method 2.1 under CTC standard QPs, CTC low QPs, and lossless CTC, respectively. Table 10, Table 11, and Table 12 show the results of Method 2.2 under CTC standard QPs, CTC low QPs, and lossless CTC, respectively.

Table 1. Method 1.1 under CTC standard QPs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.01% | 0.03% | 99% | 97% |
| Class A2 | 0.00% | -0.02% | -0.02% | 101% | 101% |
| Class B | 0.00% | 0.06% | -0.07% | 101% | 100% |
| Class C | -0.04% | 0.02% | 0.03% | 99% | 98% |
| Class E | 0.00% | -0.02% | -0.07% | 100% | 99% |
| Overall | -0.01% | 0.01% | -0.02% | 100% | 99% |
| Class D | -0.05% | -0.10% | -0.02% | 100% | 96% |
| Class F | -0.13% | -0.13% | 0.11% | 101% | 100% |
| TGM | -0.37% | -0.31% | -0.33% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | 0.03% | 100% | 99% |
| Class A2 | -0.03% | -0.05% | 0.04% | 100% | 103% |
| Class B | 0.01% | 0.09% | 0.11% | 100% | 103% |
| Class C | -0.01% | -0.07% | -0.04% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.04% | 100% | 102% |
| Class D | -0.06% | -0.23% | -0.12% | 100% | 102% |
| Class F | -0.15% | -0.13% | -0.07% | 100% | 103% |
| TGM | -0.18% | -0.20% | -0.17% | 100% | 106% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.40% | 0.03% | 100% | 99% |
| Class C | -0.01% | -0.18% | -0.23% | 100% | 100% |
| Class E | -0.04% | 0.28% | -0.46% | 100% | 99% |
| Overall | -0.02% | -0.16% | -0.18% | 100% | 100% |
| Class D | -0.08% | 0.24% | -0.06% | 100% | 100% |
| Class F | -0.03% | -0.54% | 0.29% | 100% | 99% |
| TGM | -0.27% | -0.36% | -0.28% | 100% | 98% |

Table 2. Method 1.1 under CTC low QPs

To be updated

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0-lowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.02% | -0.03% | 100% | 100% |
| Class A2 | 0.00% | -0.02% | -0.02% | 101% | 99% |
| Class B | -0.03% | -0.05% | -0.08% | 100% | 100% |
| Class C | -0.12% | -0.12% | -0.10% | 97% | 98% |
| Class E | -0.14% | -0.22% | -0.27% | 98% | 101% |
| Overall | -0.07% | -0.08% | -0.10% | 99% | 100% |
| Class D | -0.14% | -0.18% | -0.14% | 100% | 102% |
| Class F | -1.25% | -0.72% | -0.70% | 99% | 100% |
| TGM | -1.09% | -1.00% | -1.00% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0-lowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.01% | -0.02% | 100% | 98% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | 1% | 18% |
| Class B | -0.01% | -0.03% | -0.05% | 100% | 99% |
| Class C | -0.05% | -0.04% | -0.03% | 100% | 105% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | 40% | 72% |
| Class D | -0.09% | -0.08% | -0.18% | 100% | 103% |
| Class F | -1.57% | -0.68% | -0.78% | 100% | 102% |
| TGM | -1.82% | -1.09% | -1.09% | 100% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0-lowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | -0.01% | 100% | 103% |
| Class C | -0.02% | 0.01% | -0.01% | 99% | 99% |
| Class E | -0.04% | 0.02% | 0.01% | 100% | 100% |
| Overall | -0.02% | 0.01% | 0.00% | 100% | 101% |
| Class D | -0.07% | -0.06% | -0.14% | 100% | 101% |
| Class F | -0.53% | -0.54% | -0.61% | 100% | 100% |
| TGM | -0.94% | -0.74% | -0.77% | 100% | 100% |

Table 3. Method 1.1 under lossless CTC

|  |  | All Intra |  |  | Random Access |  |  | Low delay B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  |  | ratio |  |  | ratio |  |  |
|  | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings |
| Class A1 | 2.2 | 2.3 | -4.63% | 2.2 | 2.4 | -4.71% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.12% | 1.7 | 1.8 | -6.71% |  |  |  |
| Class B | 2.2 | 2.3 | -3.85% | 2.3 | 2.4 | -2.30% | 2.3 | 2.4 | -2.33% |
| Class C | 1.9 | 2.0 | -4.76% | 2.4 | 2.5 | -2.40% | 2.4 | 2.5 | -2.31% |
| Class D | 1.9 | 2.0 | -5.26% | 2.8 | 2.8 | -2.53% | 2.7 | 2.8 | -2.41% |
| Class E | 2.8 | 2.9 | -3.90% |  |  |  | 3.1 | 3.2 | -2.60% |
| Class F | 5.3 | 5.5 | -3.74% | 33.7 | 34.8 | -2.70% | 50.7 | 51.9 | -2.43% |
| TGM | 11.8 | 12.3 | -3.80% | 107.1 | 115.8 | -6.00% | 124.9 | 135.7 | -6.00% |
| Overall with Classes D, F, TGM excluded | 2.1 | 2.2 | -4.90% | 2.2 | 2.3 | -3.69% | 2.6 | 2.6 | -2.39% |
| Enc Time[%] |  | 100% |  |  | 102% |  |  | 101% |  |
| Dec Time[%] |  | 100% |  |  | 101% |  |  | 104% |  |

Table 4. Method 1.2 under CTC standard QPs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.03% | 100% | 97% |
| Class A2 | 0.00% | -0.04% | -0.02% | 101% | 100% |
| Class B | 0.00% | 0.05% | -0.07% | 100% | 99% |
| Class C | -0.03% | 0.05% | 0.01% | 100% | 99% |
| Class E | 0.00% | -0.04% | -0.08% | 101% | 101% |
| Overall | -0.01% | 0.01% | -0.03% | 100% | 99% |
| Class D | -0.04% | -0.05% | -0.08% | 100% | 97% |
| Class F | -0.15% | -0.15% | 0.06% | 100% | 100% |
| TGM | -0.35% | -0.28% | -0.32% | 100% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.01% | 100% | 101% |
| Class A2 | -0.04% | -0.07% | 0.02% | 100% | 104% |
| Class B | 0.02% | 0.01% | 0.09% | 100% | 101% |
| Class C | -0.01% | -0.07% | -0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.04% | 0.03% | 100% | 101% |
| Class D | -0.05% | -0.30% | -0.04% | 100% | 100% |
| Class F | -0.13% | -0.10% | -0.06% | 100% | 101% |
| TGM | -0.21% | -0.25% | -0.20% | 101% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.42% | -0.02% | 101% | 102% |
| Class C | -0.02% | -0.07% | -0.12% | 100% | 101% |
| Class E | -0.13% | 0.23% | -0.51% | 100% | 98% |
| Overall | -0.04% | -0.14% | -0.17% | 100% | 101% |
| Class D | -0.01% | 0.18% | 0.03% | 100% | 99% |
| Class F | -0.15% | -0.47% | 0.38% | 100% | 99% |
| TGM | -0.29% | -0.35% | -0.42% | 100% | 98% |

Table 5. Method 1.2 under CTC low QPs

To be updated

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0-lowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.02% | -0.03% | 101% | 101% |
| Class A2 | 0.00% | -0.02% | -0.02% | 101% | 100% |
| Class B | -0.03% | -0.04% | -0.05% | 100% | 101% |
| Class C | -0.13% | -0.13% | -0.12% | 97% | 95% |
| Class E | -0.13% | -0.22% | -0.25% | 98% | 101% |
| Overall | -0.07% | -0.08% | -0.09% | 99% | 99% |
| Class D | -0.17% | -0.18% | -0.15% | 100% | 101% |
| Class F | -1.32% | -0.80% | -0.74% | 99% | 103% |
| TGM | -1.33% | -1.24% | -1.24% | 101% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0-lowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1350.70% | 219.42% | 512.98% | 32% | 65% |
| Class A2 | 1619.27% | 1140.12% | 482.65% | 32% | 64% |
| Class B | -0.01% | -0.03% | -0.04% | 100% | 101% |
| Class C | -0.05% | -0.03% | -0.02% | 100% | 103% |
| Class E |  |  |  |  |  |
| Overall | 593.98% | 271.89% | 199.11% | 63% | 85% |
| Class D | -0.10% | -0.02% | -0.13% | 100% | 101% |
| Class F | -1.56% | -0.60% | -0.66% | 100% | 101% |
| TGM | -1.97% | -1.29% | -1.28% | 100% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0-lowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.01% | 0.00% | 100% | 103% |
| Class C | -0.03% | -0.01% | -0.02% | 100% | 100% |
| Class E | -0.03% | -0.04% | 0.03% | 101% | 105% |
| Overall | -0.02% | -0.02% | 0.00% | 100% | 103% |
| Class D | -0.10% | -0.01% | -0.09% | 100% | 100% |
| Class F | -0.59% | -0.55% | -0.67% | 101% | 103% |
| TGM | -1.14% | -0.90% | -0.84% | 101% | 101% |

Table 6. Method 1.2 under lossless CTC

|  |  | All Intra |  |  | Random Access |  |  | Low delay B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  |  | ratio |  |  | ratio |  |  |
|  | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings |
| Class A1 | 2.2 | 2.4 | -5.04% | 2.2 | 2.4 | -5.09% |  |  |  |
| Class A2 | 1.6 | 1.7 | -9.03% | 1.7 | 1.8 | -7.44% |  |  |  |
| Class B | 2.2 | 2.3 | -3.97% | 2.3 | 2.4 | -2.37% | 2.3 | 2.4 | -2.40% |
| Class C | 1.9 | 2.0 | -4.87% | 2.4 | 2.5 | -2.35% | 2.4 | 2.5 | -2.25% |
| Class D | 1.9 | 2.0 | -5.41% | 2.8 | 2.8 | -2.49% | 2.7 | 2.8 | -2.36% |
| Class E | 2.8 | 2.9 | -3.73% |  |  |  | 3.1 | 3.2 | -2.41% |
| Class F | 5.3 | 5.5 | -3.75% | 33.7 | 34.8 | -2.69% | 50.7 | 52.0 | -2.39% |
| TGM | 11.8 | 12.4 | -4.20% | 107.1 | 116.1 | -6.26% | 124.9 | 136.2 | -6.33% |
| Overall with Classes D, F, TGM excluded | 2.1 | 2.2 | -5.15% | 2.2 | 2.3 | -3.92% | 2.6 | 2.6 | -2.35% |
| Enc Time[%] |  | 102% |  |  | 104% |  |  | 101% |  |
| Dec Time[%] |  | 99% |  |  | 102% |  |  | 104% |  |

Table 7. Method 2.1 under CTC standard QPs

To be updated

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.11% | 0.11% | 99% | 101% |
| Class A2 | 0.02% | -0.04% | -0.01% | 100% | 100% |
| Class B | 0.02% | 0.02% | -0.05% | 99% | 99% |
| Class C | -0.01% | -0.02% | -0.04% | 101% | 99% |
| Class E | 0.01% | -0.05% | 0.02% | 101% | 99% |
| Overall | 0.01% | -0.03% | 0.00% | 100% | 99% |
| Class D | -0.01% | 0.02% | -0.03% | 101% | 96% |
| Class F | -0.14% | -0.17% | -0.09% | 99% | 100% |
| TGM | -0.37% | -0.30% | -0.33% | 101% | 102% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.05% | 100% | 100% |
| Class A2 | -0.02% | 0.14% | 0.06% | 100% | 106% |
| Class B | 0.00% | 0.06% | 0.10% | 100% | 100% |
| Class C | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.05% | 100% | 102% |
| Class D | 0.02% | -0.01% | 0.03% | 99% | 102% |
| Class F | -0.11% | -0.04% | -0.03% | 99% | 106% |
| TGM | -0.16% | -0.19% | -0.09% | 100% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.56% | 0.81% | 1.88% | 28% | 63% |
| Class C | 0.02% | -0.41% | -0.02% | 99% | 99% |
| Class E | 0.03% | 0.88% | -0.20% | 99% | 101% |
| Overall | 0.25% | 0.42% | 0.73% | 58% | 83% |
| Class D | -0.01% | 0.11% | -0.39% | 100% | 103% |
| Class F | -0.10% | -0.40% | 0.66% | 99% | 100% |
| TGM | -0.28% | -0.35% | -0.29% | 100% | 98% |

Table 8. Method 2.1 under CTC low QPs

To be updated

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0-lowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.02% | 99% | 100% |
| Class A2 | 0.03% | -0.01% | -0.02% | 100% | 99% |
| Class B | 0.00% | -0.02% | -0.06% | 99% | 100% |
| Class C | 0.08% | 0.06% | 0.03% | 97% | 95% |
| Class E | -0.21% | -0.38% | -0.48% | 97% | 100% |
| Overall | -0.01% | -0.06% | -0.10% | 99% | 99% |
| Class D | -0.05% | -0.09% | -0.07% | 100% | 101% |
| Class F | -1.40% | -0.81% | -0.79% | 98% | 100% |
| TGM | -1.46% | -1.28% | -1.28% | 100% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0-lowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | 0% | 9% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | 0% | 5% |
| Class B | -0.05% | -0.10% | -0.14% | 99% | 100% |
| Class C | -0.03% | -0.04% | -0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | 5% | 34% |
| Class D | -0.07% | -0.04% | -0.13% | 100% | 107% |
| Class F | -1.78% | -0.62% | -0.79% | 100% | 104% |
| TGM | -2.14% | -1.33% | -1.33% | 100% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0-lowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.09% | -0.09% | 99% | 100% |
| Class C | -0.03% | -0.04% | -0.05% | 99% | 100% |
| Class E | -0.08% | -0.03% | -0.08% | 99% | 100% |
| Overall | -0.06% | -0.06% | -0.07% | 99% | 100% |
| Class D | -0.08% | -0.09% | -0.20% | 99% | 103% |
| Class F | -0.64% | -0.54% | -0.71% | 100% | 100% |
| TGM | -1.18% | -0.84% | -0.86% | 100% | 102% |

Table 9. Method 2.1 under lossless CTC

|  |  | All Intra |  |  | Random Access |  |  | Low delay B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  |  | ratio |  |  | ratio |  |  |
|  | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings |
| Class A1 | 2.2 | 2.3 | -4.16% | 2.2 | 2.3 | -4.09% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.00% | 1.7 | 1.8 | -5.86% |  |  |  |
| Class B | 2.2 | 2.3 | -3.75% | 2.3 | 2.4 | -1.92% | 2.3 | 2.4 | -1.93% |
| Class C | 1.9 | 2.0 | -4.84% | 2.4 | 2.5 | -2.60% | 2.4 | 2.5 | -2.48% |
| Class D | 1.9 | 2.0 | -5.45% | 2.8 | 2.8 | -2.74% | 2.7 | 2.8 | -2.61% |
| Class E | 2.8 | 2.9 | -4.50% |  |  |  | 3.1 | 3.2 | -3.43% |
| Class F | 5.3 | 5.5 | -4.13% | 33.7 | 34.9 | -3.14% | 50.7 | 52.3 | -2.91% |
| TGM | 11.8 | 12.4 | -4.69% | 107.1 | 116.7 | -6.63% | 124.9 | 137.1 | -6.75% |
| Overall with Classes D, F, TGM excluded | 2.1 | 2.2 | -4.89% | 2.2 | 2.3 | -3.32% | 2.6 | 2.6 | -2.49% |
| Enc Time[%] |  | 99% |  |  | 100% |  |  | 100% |  |
| Dec Time[%] |  | 101% |  |  | 103% |  |  | 105% |  |

Table 10. Method 2.2 under CTC standard QPs

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.11% | 0.11% | 99% | 102% |
| Class A2 | 0.02% | -0.03% | -0.01% | 101% | 103% |
| Class B | 0.02% | 0.02% | -0.05% | 101% | 103% |
| Class C | -0.01% | -0.02% | -0.04% | 100% | 100% |
| Class E | 0.00% | -0.04% | 0.02% | 100% | 101% |
| Overall | 0.01% | -0.03% | 0.00% | 100% | 102% |
| Class D | -0.01% | -0.02% | -0.01% | 101% | 108% |
| Class F | -0.14% | -0.17% | -0.11% | 99% | 100% |
| TGM | -0.38% | -0.27% | -0.32% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.09% | 0.02% | 100% | 100% |
| Class A2 | -0.02% | 0.17% | 0.06% | 99% | 106% |
| Class B | 0.01% | 0.12% | 0.07% | 100% | 101% |
| Class C | -0.02% | 0.13% | 0.05% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.09% | 0.05% | 100% | 102% |
| Class D | -0.01% | 0.03% | 0.00% | 99% | 102% |
| Class F | -0.15% | -0.03% | 0.08% | 99% | 102% |
| TGM | -0.10% | -0.15% | -0.13% | 100% | 103% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.48% | 0.23% | 100% | 99% |
| Class C | 0.01% | -0.23% | -0.13% | 99% | 101% |
| Class E | 0.03% | 0.88% | -0.20% | 99% | 100% |
| Overall | 0.01% | -0.06% | 0.00% | 99% | 100% |
| Class D | -0.03% | 0.39% | -0.50% | 101% | 104% |
| Class F | -0.14% | 0.12% | 0.36% | 100% | 101% |
| TGM | -0.11% | -0.24% | -0.30% | 100% | 98% |

Table 11. Method 2.2 under CTC low QPs

To be updated

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0-lowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.02% | 99% | 100% |
| Class A2 | 0.04% | -0.01% | -0.01% | 100% | 100% |
| Class B | -0.01% | -0.03% | -0.05% | 100% | 103% |
| Class C | 0.05% | 0.06% | 0.04% | 96% | 98% |
| Class E | -0.21% | -0.40% | -0.47% | 98% | 105% |
| Overall | -0.02% | -0.07% | -0.09% | 99% | 101% |
| Class D | -0.06% | -0.10% | -0.08% | 99% | 100% |
| Class F | -1.47% | -0.85% | -0.83% | 98% | 100% |
| TGM | -1.69% | -1.50% | -1.50% | 100% | 99% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0-lowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.08% | 2.47% | 3.09% | 3% | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | 0% | 5% |
| Class B | -0.06% | -0.08% | -0.12% | 99% | 99% |
| Class C | -0.03% | -0.04% | -0.07% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | 10% | #NUM! |
| Class D | -0.10% | 0.00% | -0.15% | 100% | 104% |
| Class F | -1.93% | -0.76% | -0.91% | 100% | 100% |
| TGM | -2.33% | -1.36% | -1.37% | 100% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0-lowQP |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.10% | -0.10% | 99% | 100% |
| Class C | -0.03% | 0.01% | -0.01% | 99% | 100% |
| Class E | -0.07% | -0.02% | -0.06% | 99% | 100% |
| Overall | -0.06% | -0.05% | -0.06% | 99% | 100% |
| Class D | -0.08% | -0.06% | -0.25% | 100% | 100% |
| Class F | -0.64% | -0.52% | -0.63% | 100% | 103% |
| TGM | -1.27% | -1.01% | -1.02% | 101% | 101% |

Table 12. Method 2.2 under lossless CTC

|  |  | All Intra |  |  | Random Access |  |  | Low delay B |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  |  | ratio |  |  | ratio |  |  |
|  | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings | VTM7 | Test | bit-rate savings |
| Class A1 | 2.2 | 2.3 | -4.54% | 2.2 | 2.3 | -4.46% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.79% | 1.7 | 1.8 | -6.49% |  |  |  |
| Class B | 2.2 | 2.3 | -3.92% | 2.3 | 2.4 | -2.05% | 2.3 | 2.4 | -2.06% |
| Class C | 1.9 | 2.0 | -5.06% | 2.4 | 2.5 | -2.65% | 2.4 | 2.5 | -2.53% |
| Class D | 1.9 | 2.0 | -5.71% | 2.8 | 2.8 | -2.82% | 2.7 | 2.8 | -2.67% |
| Class E | 2.8 | 2.9 | -4.53% |  |  |  | 3.1 | 3.2 | -3.40% |
| Class F | 5.3 | 5.6 | -4.25% | 33.7 | 35.0 | -3.22% | 50.7 | 52.4 | -2.99% |
| TGM | 11.8 | 12.5 | -5.08% | 107.1 | 117.1 | -6.89% | 124.9 | 137.4 | -6.94% |
| Overall with Classes D, F, TGM excluded | 2.1 | 2.2 | -5.19% | 2.2 | 2.3 | -3.58% | 2.6 | 2.6 | -2.55% |
| Enc Time[%] |  | 101% |  |  | 102% |  |  | 101% |  |
| Dec Time[%] |  | 99% |  |  | 100% |  |  | 103% |  |

# Conclusions

In this contribution, four methods to improve TSRC by using rice parameter other than 1 are proposed. Method 1.1 and Method 1.2 improve the coding efficiency of TSRC by reusing the rice parameter table in RRC. Method 2.1 and Method 2.2 improve the coding efficiency of TSRC without using any rice parameter table, and the rice parameter table required by RRC is removed. Compression performance improvements were shown.

# References

- VTM7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- Y.-W. Chen, X. Xiu, T.-C. Ma, H.-J. Jhu, X. Wang, “Non-CE8: Transform skip residual coding simplification,” Document of Joint Video Experts Team, JVET-P0562.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 7),” Document of Joint Video Experts Team, JVET-P2001.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).