| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0188-v1 |
| --- | --- |

| Title: | CE4-related: Simplification on geometric partitioning mode by replacing motion index calculation with subsampled weight information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Chun-Chia Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yuling.hsiao, chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two methods to remove motion index calculation in geometric partitioning mode (GEO) by subsampling the luma blending weight information with simple calculations. In method 1, motion indexes are subsampled from luma blending weights. In method 2, motion indexes are subsampled from luma blending weight indexes. Results are summarized as follows.

Proposed method 1
VTM7.0-RA: {Y-BD-rate=-0.28%, EncT=103%, DecT=99%}
VTM7.0-LB: {Y-BD-rate=-0.69%, EncT=103%, DecT=98%}

Proposed method 2
VTM7.0-RA: {Y-BD-rate=-0.28%, EncT=103%, DecT=98%}
VTM7.0-LB: {Y-BD-rate=-0.66%, EncT=104%, DecT=98%}

GEO common base
VTM7.0-RA: {Y-BD-rate=-0.28%, EncT=104%, DecT=98%}
VTM7.0-LB: {Y-BD-rate=-0.70%, EncT=105%, DecT=103%}

# Introduction

Geometric partitioning mode (GEO) applies the same blending process to generate predictors as triangular prediction mode (TPM), where the extended blending masks are used. In JVET-P0884 [1], simplified GEO was proposed. It quantizes GEO angles to arc tangent of power-of-2 to replace per-sample multiplications with shifting and enables storing pre-calculated blending masks and motion storage masks with minimum total storage requirement. This version serves as GEO common base for evaluation in CE2 [2].

Among GEO operations, it contains two parts, predictor blending and motion storage for reference. For the predictor blending, the blending weight indexes are firstly calculated, and then a lookup table is used to derive the blending weights. The blending weights are used to blend two predictors to generate the final predictor. On the other hand, for the motion storage for reference, the motion indexes are firstly calculated and used to determine the to-be-stored motion for each 4x4 subblock, such as either one of the motion from the two partitions or the combined motions from both. In the GEO common base, the equations for deriving the blending weight indexes and the motion indexes are different, which means it is necessary to derive them separately.

In this contribution, it is proposed to remove motion index calculation to further simplify GEO. Similar to generation of chroma blending weights using subsampling of the luma ones, it is proposed to subsample either luma blending weights or luma blending weight indexes to determine which motion should be stored. With the proposed methods, the calculation of motion indexes could be removed. Details are described in the following sections.

# Proposed method

## Method 1: Subsampling luma blending weights

Motion index of subblock (i,j) (where i is horizontal index and j is vertical index) is set equal to luma blending weight at coordinate (4i+2,4j+2). When the motion index is in the range of 1 to 7, the combined motion from the two uni-predicted partitions is stored. When the motion index is equal to 0 or 8, only one of the two uni-prediction motions is stored.

## Method 2: Subsampling luma blending weight indexes

Motion index of subblock (i,j) is firstly set equal to luma blending weight index at coordinate (4i+2,4j+2). Then, it follows the original equation in the GEO common base to determine which motion should be stored according to the input motion index. That is, when the absolute value of motion index is smaller than a threshold, the combined motion is stored. Otherwise, one of the two uni-prediction motions is stored depending on the sign of the motion index.

# Experiment results

The proposed methods have been implemented on top of GEO common base. The common test conditions [3] are used for evaluation. Tables 1 and 2 summarize the results of the method 1 and method 2, respectively.

Table 1. Results of the proposed method 1 (subsampling luma blending weights)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.17% | -0.18% | 103% | 99% |
| Class A2 | -0.22% | -0.10% | -0.03% | 103% | 105% |
| Class B | -0.15% | -0.06% | -0.17% | 103% | 98% |
| Class C | -0.61% | -0.91% | -0.92% | 102% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.32% | -0.34% | 103% | 99% |
| Class D | -0.48% | -0.70% | -0.61% | 103% | 99% |
| Class F | -0.31% | -0.38% | -0.30% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.38% | -0.58% | -0.43% | 105% | 96% |
| Class C | -0.79% | -0.94% | -0.81% | 104% | 98% |
| Class E | -1.08% | -0.11% | -0.76% | 101% | 98% |
| Overall | -0.69% | -0.58% | -0.64% | 103% | 98% |
| Class D | -0.70% | -0.83% | -1.36% | 103% | 100% |
| Class F | -0.47% | -1.30% | -0.35% | 102% | 99% |

Table 2. Results of the proposed method 2 (subsampling luma blending weight indexes)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.26% | -0.19% | 103% | 97% |
| Class A2 | -0.21% | -0.19% | -0.07% | 103% | 101% |
| Class B | -0.16% | -0.16% | -0.18% | 103% | 97% |
| Class C | -0.59% | -0.96% | -0.92% | 103% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.40% | -0.36% | 103% | 98% |
| Class D | -0.41% | -0.69% | -0.57% | 103% | 98% |
| Class F | -0.29% | -0.37% | -0.29% | 101% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | -0.53% | -0.37% | 105% | 97% |
| Class C | -0.78% | -1.14% | -0.71% | 105% | 100% |
| Class E | -1.01% | 0.12% | -0.62% | 101% | 98% |
| Overall | -0.66% | -0.57% | -0.55% | 104% | 98% |
| Class D | -0.66% | -0.88% | -1.35% | 102% | 101% |
| Class F | -0.55% | -0.87% | -0.14% | 103% | 99% |

# Conclusion

In this contribution, two methods are proposed to remove motion index calculation by subsampling luma weight information. With the unified derivation, not only specification text can be simplified but resource sharing could be possible for some software and hardware implementations. Compared with GEO common base, the BD-rate change is minor.

# References

[1] H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Blaser, and J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304),” Document of Joint Video Experts Team, JVET-P0884.

[2] C.-C. Chen, R.-L. Liao, X. Xiu, and H. Yang, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning,” Document of Joint Video Experts Team, JVET-P2024.

[3] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).