| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0189-v1 |
| --- | --- |

| Title: | CE4-related: Reducing GEO modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Yu-Ling Hsiao, Ching-Yeh Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {olena.chubach, yuling.hsiao, chingyeh.chen, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

A geometric merge mode (GEO) is currently studied in the CE4. In particular, in CE4-1, a possibility of reducing 82 modes down to 64 modes is checked. In this proposal, it is suggested to reduce the number of modes from 82 to 54. With this simplification, the majority of the coding gain of the original GEO design is preserved, while reducing computational complexity of GEO even more. The BD-rates over the CE4 common base GEO software are as follows(Y/Cb/Cr):

RA: 0.01% / 0.05% / 0.02%
LB: 0.02% / 0.12% / 0.03%

# Introduction

GEO has been proposed in [1]. There, the same predictors blending concept as TPM is used, and different blending masks are supported for 140 different modes (32 angles and 5 offsets).

As described in [1], the GEO split modes are described by angle index φi and distance index ρi. In CE4-1 [2], the split modes of GEO are proposed to be reduced down to 64, still preserving most of the coding gain. For supporting 64 modes, angle index is quantized with fixed step 11.25 degrees around 360 degrees. In addition, the angle indexes indicating horizontal and near-horizontal spilts with all distances are removed. Indexes 4, 12, 20, and 28, which indicate 45 or 135 degrees with all distances are also removed. Distance index is quantized into 4 steps, in addition, the fourth distance index for horizontal and vertical angles (0, 8, 16, 24) are removed.

In total, 4 angles with 2 distances, 8 angles with 3 distances, and 8 angles with 4 distances are preserved, which results in 64 split modes. Table 1 contains initial 82 modes with removed accordingly modes as described above (marked in yellow).

Table 1 – Specification of the angleIdx and distanceIdx values based on the geo_partition_idx value, supporting 64 modes.

| geo_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| angleIdx | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| distanceIdx | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 |
| geo_partition_idx | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| angleIdx | 4 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | 8 | 8 | 8 | 8 | 9 | 9 |
| distanceIdx | 3 | 0 | 1 | 2 | 3 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 |
| geo_partition_idx | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| angleIdx | 9 | 9 | 10 | 10 | 10 | 10 | 11 | 11 | 11 | 11 | 12 | 12 | 12 | 13 | 13 | 13 | 14 | 14 |
| distanceIdx | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 |
| geo_partition_idx | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 |
| angleIdx | 14 | 15 | 15 | 15 | 16 | 16 | 16 | 17 | 17 | 17 | 18 | 18 | 18 | 19 | 19 | 19 | 20 | 20 |
| distanceIdx | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 |
| geo_partition_idx | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 | 81 |  |  |  |  |  |  |  |  |
| angleIdx | 20 | 21 | 21 | 21 | 22 | 22 | 22 | 23 | 23 | 23 |  |  |  |  |  |  |  |  |
| distanceIdx | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 |  |  |  |  |  |  |  |  |

- Proposed method

In this contribution it is proposed to reduce the number of supported modes from 82 to 54, by removing angles which have slope equal to 4 or 1/4, with all related distances, as shown in Table 2. Consequently, the look-up table Dis for derivation of geo partitioning distance can be updated as it is shown in Table 3, and binarization of the merge_geo_partition_idx[ ][ ] is updated as it is shown in Table 4.

Table 2 – Modified specification of the angleIdx and distanceIdx values based on the geo_partition_idx value, supporting 54 modes

| geo_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| angleIdx | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| distanceIdx | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 |
| geo_partition_idx | 14 | 19 | 20 | 21 | 22 | 15 | 16 | 17 | 26 | 27 | 28 | 29 | 18 | 19 | 20 | 21 | 22 | 23 |
| angleIdx | 4 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | 8 | 8 | 8 | 8 | 9 | 9 |
| distanceIdx | 3 | 0 | 1 | 2 | 3 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 |
| geo_partition_idx | 24 | 25 | 26 | 27 | 28 | 29 | 42 | 43 | 44 | 45 | 30 | 31 | 32 | 49 | 50 | 51 | 33 | 34 |
| angleIdx | 9 | 9 | 10 | 10 | 10 | 10 | 11 | 11 | 11 | 11 | 12 | 12 | 12 | 13 | 13 | 13 | 14 | 14 |
| distanceIdx | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 |
| geo_partition_idx | 33 | 36 | 37 | 38 | 39 | 40 | 41 | 61 | 62 | 63 | 42 | 43 | 44 | 67 | 68 | 69 | 45 | 46 |
| angleIdx | 14 | 15 | 15 | 15 | 16 | 16 | 16 | 17 | 17 | 17 | 18 | 18 | 18 | 19 | 19 | 19 | 20 | 20 |
| distanceIdx | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 |
| geo_partition_idx | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 79 | 80 | 81 |  |  |  |  |  |  |  |  |
| angleIdx | 20 | 21 | 21 | 21 | 22 | 22 | 22 | 23 | 23 | 23 |  |  |  |  |  |  |  |  |
| distanceIdx | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 |  |  |  |  |  |  |  |  |

Table 3 – Modified look-up table Dis for derivation of geoc partitioning distance

| idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Dis[idx] | 8 | 8 | 8 | 4 | 4 | 2 | 0 | -2 | −4 | −4 | −8 | −8 |
| idx | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| Dis[idx] | −8 | −8 | −8 | −4 | −4 | -2 | 0 | 2 | 4 | 4 | 8 | 8 |

Table 4 – Syntax elements and associated binarizations

| merge_geo_partition_idx[ ][ ] | TB | cMax = 6453 |
| --- | --- | --- |

- Experimental results

The proposed method was implemented on top of VTM7.0 [3]. The simulations were performed following the JVET common test conditions (CTC) [4]. Table 5 contains results of GEO with support of 54 modes instead of 82 modes, Table 6 contains results of CE4-1.

Table 5.  Results of proposed method (over CE4 common base GEO software, inaccurate runtime data)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.03% | -0.04% | 138% | 100% |
| Class A2 | 0.02% | 0.17% | 0.01% | 139% | 96% |
| Class B | -0.01% | -0.04% | 0.03% | 141% | 101% |
| Class C | 0.03% | 0.12% | 0.05% | 139% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | 0.02% | 140% | 100% |
| Class D | -0.03% | -0.02% | 0.09% | 139% | 100% |
| Class F | 0.00% | 0.01% | 0.00% | 138% | 102% |
|  |  | Low delay B Main10 |  |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.06% | 0.19% | 137% | 99% |
| Class C | -0.02% | 0.20% | -0.16% | 133% | 93% |
| Class E | 0.10% | 0.13% | 0.01% | 131% | 97% |
| Overall | 0.02% | 0.12% | 0.03% | 134% | 96% |
| Class D | -0.05% | 0.63% | -0.11% | 133% | 99% |
| Class F | 0.08% | -0.12% | -0.14% | 130% | 97% |

Table 6.  Results of CE4-1 (over CE4 common base GEO software)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4-CommonBase |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.11% | 0.00% | 99% | 99% |
| Class A2 | 0.02% | 0.07% | 0.00% | 99% | 100% |
| Class B | -0.01% | -0.01% | 0.08% | 99% | 100% |
| Class C | -0.02% | 0.01% | 0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.03% | 99% | 100% |
| Class D | 0.02% | 0.02% | 0.09% | 100% | 100% |
| Class F (optional) | -0.01% | 0.04% | -0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE4-CommonBase |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.10% | -0.06% | 99% | 99% |
| Class C | 0.01% | 0.12% | 0.04% | 99% | 100% |
| Class E | -0.04% | 0.08% | 0.45% | 99% | 99% |
| Overall | 0.00% | 0.10% | 0.10% | 99% | 99% |
| Class D | 0.00% | -0.34% | 0.47% | 99% | 100% |
| Class F | -0.08% | 0.25% | 0.40% | 100% | 99% |

Conclusion

In this contribution, a method of reducing number of modes in GEO from 82 to 54 is proposed, which allows to reduce even further computational complexity of GEO, while preserving the majority of the coding gain of the original GEO design. In RA the BD-rate results are as follows (over the CE4 common base GEO software, Y/Cb/Cr): 0.01% / 0.05% / 0.02%; in LB results are: 0.02% / 0.12% / 0.03%.

- References

- S. Esenlik, H. Gao, A. Filippov, V. Rufitskiy, A. M. Kotra, B. Wang, E. Alshina, M. Bläser, and J. Sauer, “Non-CE4: Geometrical partitioning for inter blocks,” Document of Joint Video Experts Team, JVET-O0489.

- C.-C. Chen, R.-L. Liao, X. Xiu, and H. Yang, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning,” Document of Joint Video Experts Team, JVET-P2024.

- VTM7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).