| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0210_r1 |
| --- | --- |

| Title: | [AHG9]: Miscellaneous HLS clean-ups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seethal Paluri 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | dr.hendry@lge.com seethal.paluri@lge.com |
| Source: | LG Electronics USA |  |  |

_____________________________

# Abstract

This contribution recommends the following changes to the general HLS of VVC specification:

- Prohibit the corner case such that two rectangular slices in a tile belong to different subpictures.

- When subpicture signalling is present but there is only one subpicture, omit the signalling of syntax elements subpic_ctu_top_left_x[ 0 ], subpic_ctu_top_left_y[ 0 ], subpic_width_minus1[ 0 ], subpic_height_minus1[ 0 ], subpic_treated_as_pic_flag[ i ], and loop_filter_across_subpic_enabled_flag[ i ].

- Specify a constraint such that when subpicture signalling is present, there shall be at least one subpicture that is an independently coded subpicture.

- Specify a constraint such that when subpicture id is signalled in PPS or picture header, all subpictures are independently coded subpictures.

- Remove the constraint on the single_slice_per_subpic_flag such that when subpics_present_flag is equal to 0, single_slice_per_subpic_flag shall be equal to 0.

- Specify a constraint such that when RPR is enabled, the signalling of virtual boundary position, if present, shall be in picture header.

- Specify a constraint such that when subpicture signalling is present, the signalling of virtual boundary position, if present, shall be in SPS.

- Remove the use of uek(v).

- Change the coding of bit_depth_minus8 and min_qp_prime_ts_minus4 from ue(v) to u(3) and u(6), respectively.

In the first revision of this contribution, experiment results for proposed aspect #8 is added. It shows that the removal of uek(v) in the ALF syntax table have 0.02% and 0.01% for each components for AI and RA configurations, respectively.

# Proposal 1: on definition of subpicture

In subclause 6.3.1 Partitioning of pictures into subpictures, slices, and tiles, the following is specified:

“A subpicture contains one or more slices that collectively cover a rectangular region of a picture.”

Considering rectangular slice can contain only subset of a tile, the above specification fails to prevent the case where two rectangular slices are within the same tile but belong to different subpicture as illustrated in the figure below.

Figure 1 – Example of corner case of subpicture, slice, and tile layout.

We consider the above case is a corner case and suggest that it is prohibited for simplicy of the VVC picture partitioning scheme unless there a compelling use-case that requires such configuration.

Proposal: We propose to that the following constraint is specified for subpicture and tile.

When a picture is partitioned onto two or more subpicture, one or both of the following conditions shall be fulfilled for each subpicture and tile:

- All CTUs in a tile belong to the same subpicture.

- All CTUs in a subpicture belong to the same tile.

# Proposal 2: on subpicture signalling when there is only one subpicture

In the current subpicture signalling, when subpics_present_flag is equal to 1, it is possible that the number of subpicture in each picture referring to the SPS is equal to 1 (i.e., sps_num_subpics_minus1 is equal to 0). Such decision was made for supporting subpicture extraction use-case in which an independently coded subpicture can be extracted from a bitstream to form another bitstream without having to change anything lower than parameter sets.

Consequently, when subpics_present_flag is equal to 1 and sps_num_subpics_minus1 is equal to 0, the syntax elements subpic_ctu_top_left_x[ 0 ], subpic_ctu_top_left_y[ 0 ], subpic_width_minus1[ 0 ], , subpic_height_minus1[ 0 ], subpic_treated_as_pic_flag[ i ], and loop_filter_across_subpic_enabled_flag[ i ] are still present. The presence of these syntax elements in such situation is not only redundant but also possibly makes the behaviour of decoder unpredicatable if incorrect values are signalled for those syntax element. For example, when subpics_present_flag is equal to 1 and sps_num_subpics_minus1 is equal to 0, it means there is only one subpicture, which is the picture itself, the value of subpic_treated_as_pic_flag[ 0 ] is supposed to be equal to 1. If that value is signalled to be equal to 0, it would create problem for decoding process.

Proposal: We propose to condition the presence of syntax elements subpic_ctu_top_left_x[ 0 ], subpic_ctu_top_left_y[ 0 ], subpic_width_minus1[ 0 ], subpic_height_minus1[ 0 ], subpic_treated_as_pic_flag[ i ], and loop_filter_across_subpic_enabled_flag[ i ] on the value of sps_num_sub_pics_minus1. When the value of sps_num_sub_pics_minus1 is equal to 0, those syntax elements are not present and their values are inferred as follows:

- subpic_ctu_top_left_x[ 0 ] is inferred to be equal to 0

- subpic_ctu_top_left_y[ 0 ] is inferred to be equal to 0

- subpic_width_minus1[ 0 ] is inferred to be equal to Ceil( pic_width_max_in_luma_samples ÷ CtbSizeY ) )

- subpic_height_minus1[ 0 ] is inferred to be equal to Ceil( pic_height_max_in_luma_samples ÷ CtbSizeY ) )

- subpic_treated_as_pic_flag[ 0 ] is inferred to be equal to 1

- loop_filter_across_subpic_enabled_flag[ 0 ] is inferred to be equal to 0

# Proposal 3: on number of independently coded subpicture

In the current specification, it is possible that subpicture(s) are present but none of them are independently coded subpicture. In our opinion, there is no reason to use subpicture for picture partitioning if none of the subpicture is an independently coded subpicture. For such situation, tile would be enough.

Proposal: We propose to specify the following constraint:

It is a bistream conformance constraint that when subpics_present_flag is equal to 1, there shall be at least one value of subpic_treated_as_pic_flag[ i ], for i in the range of 0 to sps_num_subpics_minus1, inclusive, be equal to 1.

# Proposal 4: on the signalling of subpicture id location

We believe that only independently coded subpicture may be extracted and further, if desired, be merged with other subpicture(s). Thus, in the bitstream resulting from bitstream extraction process and / or bitstream merging process, subpictures in such bitstreams are independently coded subpictures. Otherwise, when merging subpicture that is not an independently coded subpicture, it is possible that blocks in that subpicture is coded with reference to block outside the subpicture which would cause problem after when decoding the merged subpictures.

Related to the above, when subpicture id signalling is present, it may present in either SPS, PPS, or PH (picture header). The signalling of subpicture id in location other than in SPS is for supporting the case that the bitstream is the result of subpicture extraction and / or merging process. Consequently, when subpicture id signalling is not in SPS, then all subpictures are independently coded subpicture.

Proposal: We propose to specify a constraint such that when subpicture id is signalled in PPS or PS, all subpictures are independently coded subpictures. Such constraint may be expressed as follow:

It is a bistream conformance constraint that when sps_subpic_id_present_flag is equal to 1 and sps_subpic_id_signalling_present_flag is equal to 0, the value of subpic_treated_as_pic_flag[ i ] for all subpictures shall be equal to 1.

Furthermore, a note may be added to describe that subpictures in picture resulted from bitstream extraction or bitstream merging are all independently coded subpictures.

# Proposal 5: on the constraint for single_slice_per_subpic_flag

For the current tile and rectangular slice signalling, the flag single_slice_per_subpic_flag specifies whether each subpicture of the picture contains exactly one slice or not. For this flag, it is also constrained that when the value of subpics_present_flag is equal to 0, the value of single_slice_per_subpic_flag shall be equal to 0.

We think that the constraint is not necessary. In fact, without such constraint, when subpics_present_flag is equal to 0 and single_slice_per_subpic_flag is equal to 1, the flag can specify that there is only one slice in the picture.

Proposal: We propose to remove the constraint that when the value of subpics_present_flag is equal to 0, the value of single_slice_per_subpic_flag shall be equal to 0. To make the signalling concept works, the following changes are proposed for the mapping of CtbAddrInSlice[ ][ ] specified in subclause 6.5.1:

For rectangular slices, the list NumCtuInSlice[ i ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, specifying the number of CTU in the i-th slice, and the matrix CtbAddrInSlice[ i ][ j ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, and j ranging from 0 to NumCtuInSlice[ i ] − 1, inclusive, specifying the picture raster scan address of the j-th CTB within the i-th slice, are derived as follows:

if( subpics_present_flag  &&  single_slice_per_subpic_flag ) {
	for( i = 0; i  <=   sps_num_subpics_minus1; i++ )
		NumCtuInSlice[ i ] = 0
	for( i = 0; i < PicSizeInCtbsY; i ++ ) {
		sliceIdx = subpics_present_flag ? 0 : CtbToSubPicIdx[ i ]
		CtbAddrInSlice[ sliceIdx ][ NumCtuInSlice[ sliceIdx ] ] = i
		NumCtuInSlice[ sliceIdx ]++
	}
} else {
	…
}

# Proposal 6: on RPR and virtual boundary signalling

In the current VVC draft, there are editor’s notes in the semantics of sps_virtual_boundaries_pos_x[ i ] and sps_virtual_boundaries_pos_y[ i ] as follows:

sps_virtual_boundaries_pos_x[ i ] is used to compute the value of VirtualBoundariesPosX[ i ], which specifies the location of the i-th vertical virtual boundary in units of luma samples. The value of sps_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_in_luma_samples ÷ 8 ) − 1, inclusive. [Ed. (VD): pic_width_in_luma_samples is in the PPS, not in the SPS.]

sps_virtual_boundaries_pos_y[ i ] is used to compute the value of VirtualBoundariesPosY[ i ], which specifies the location of the i-th horizontal virtual boundary in units of luma samples. The value of sps_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_in_luma_samples ÷ 8 ) − 1, inclusive. [Ed. (VD): pic_height_in_luma_samples is in the PPS, not in the SPS.]

When both RPR and loop filter disabling at virtual boundary feature are both enabled for pictures in the CLVS, in our opinion, it does not make sense for the signalling of virtual boundary position to be present in the SPS since pictures in the CLVS may have different spatial resolutions.

Proposal: We propose that when RPR for pictures in the CLVS, virtual boundaries, if present, shall be signalled in picture header.

If the above proposal is agreed, the semantics of sps_virtual_boundaries_pos_x[ i ] and sps_virtual_boundaries_pos_y[ i ] can be updated as follows

sps_virtual_boundaries_pos_x[ i ] is used to compute the value of VirtualBoundariesPosX[ i ], which specifies the location of the i-th vertical virtual boundary in units of luma samples. The value of sps_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_max_in_luma_samples ÷ 8 ) − 1, inclusive.

sps_virtual_boundaries_pos_y[ i ] is used to compute the value of VirtualBoundariesPosY[ i ], which specifies the location of the i-th horizontal virtual boundary in units of luma samples. The value of sps_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_max_in_luma_samples ÷ 8 ) − 1, inclusive.

The semantics of sps_loop_filter_across_virtual_boundaries_disabled_present_flag is updated as follows:

sps_loop_filter_across_virtual_boundaries_disabled_present_flag equal to 1 specifies that the in-loop filtering operations are disabled across the virtual boundaries in pictures referring to the SPS. sps_loop_filter_across_virtual_boundaries_disabled_present_flag equal to 0 specifies that no such disabling of in-loop filtering operations is applied in pictures referring to the SPS. In-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When the value of ref_pic_resampling_enabled_flag is equal to 1, the value of sps_loop_filter_across_virtual_boundaries_disabled_present_flag shall be equal to 0.

# Proposal 7: on subpicture and virtual boundary signalling

When subpicture signalling is present and virtual boundary position is present in picture header, we have concern that for subpicture extraction and subpicture merging scenario, picture headers need to be rewritten to make sure the signalling of the virtual boundary position is correct. This is against the design objective of subpicture extraction and merging in which no bitstream rewritten is needed for NAL unit lower than parameter sets.

Proposal: We propose that when subpicture signalling is present, the signalling of virtual boundary position, if present, shall not be in picture header.

# Proposal 8: on removal of uek(v)

In the current VVC working draft, the k-th order of exponential golomb code with k = 3 is used to signal the absolute values of the the luma and chroma ALF coefficients, i.e., alf_luma_coeff_abs and alf_chroma_coeff_abs. We do not find any other syntax element in the current VVC specification coded by using such method. As the k-th order exponential golomb coding may computational overhead and complexity but it is only used in these two instances throughout the VVC Specification text, we suggest that the JVET group to consider removing the use of uek(v) for simplification.

Proposal: We propose to replace the coding of syntax element alf_luma_coeff_abs[ sfIdx ][ j ] and alf_chroma_coeff_abs[ altIdx ][ j ] from uek(v) to u(v). The number of bits for signalling these syntax elements can be signalling explicitly.

The spec text for this proposal is as follows:

| alf_data( ) { | Descriptor |
| --- | --- |
| … |  |
| alf_luma_coeff_abs_len_minus1 | ue(v) |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| for( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_abs[ sfIdx ][ j ] | u(v) |
| if( alf_luma_coeff_abs[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| if( alf_luma_clip_flag ) |  |
| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) |  |
| for( j = 0; j < 12; j++ ) |  |
| alf_luma_clip_idx[ sfIdx ][ j ] | u(2) |
| } |  |
| if( alf_chroma_filter_signal_flag ) { |  |
| alf_chroma_num_alt_filters_minus1 | ue(v) |
| alf_chroma_coeff_abs_len_minus1 | ue(v) |
| for( altIdx = 0; altIdx <= alf_chroma_num_alt_filters_minus1; altIdx++ ) { |  |
| alf_chroma_clip_flag[ altIdx ] | u(1) |
| for( j = 0; j < 6; j++ ) { |  |
| alf_chroma_coeff_abs[ altIdx ][ j ] | u(v) |
| if( alf_chroma_coeff_abs[ altIdx ][ j ] > 0 ) |  |
| alf_chroma_coeff_sign[ altIdx ][ j ] | u(1) |
| } |  |
| … |  |
| } |  |

alf_luma_coeff_abs_len_minus1 plus 1 specifies the number of bits used to represent the syntax element alf_luma_coeff_abs[ sfIdx ][ j ]. The value of alf_luma_abs_len_minus1 shall be in the range of 0 to 15, inclusive.

alf_luma_coeff_abs[ sfIdx ][ j ] specifies the absolute value of the j-th coefficient of the signalled luma filter indicated by sfIdx. When alf_luma_coeff_abs[ sfIdx ][ j ] is not present, it is inferred to be equal 0.

The order k of the exp-Golomb binarization uek(v) is set equal to 3.

alf_chroma_coeff_abs_len_minus1 plus 1 specifies the number of bits used to represent the syntax element alf_chroma_coeff_abs[ altIdx ][ j ]. The value of alf_chroma_abs_len_minus1 shall be in the range of 0 to 15, inclusive.

alf_chroma_coeff_abs[ altIdx ][ j ] specifies the absolute value of the j-th chroma filter coefficient for the alternative chroma filter with index altIdx. When alf_chroma_coeff_abs[ altIdx ][ j ] is not present, it is inferred to be equal 0.

The order k of the exp-Golomb binarization uek(v) is set equal to 3.

A summary of the results for AI and RA for VTM 7.1 under CTC conditions is shown below:

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.01% | 100% | 99% |
| Class A2 | 0.01% | 0.01% | 0.01% | 100% | 99% |
| Class B | 0.01% | 0.02% | 0.02% | 100% | 99% |
| Class C | 0.04% | 0.03% | 0.03% | 100% | 97% |
| Class E | 0.04% | 0.04% | 0.04% | 101% | 99% |
| Overall | 0.02% | 0.02% | 0.02% | 100% | 99% |
| Class D | 0.03% | 0.03% | 0.03% | 101% | 100% |
| Class F | 0.04% | 0.04% | 0.04% | 99% | 98% |
| Class SCC | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class A2 | 0.01% | 0.01% | 0.01% | 100% | 100% |
| Class B | 0.02% | 0.02% | 0.02% | 100% | 99% |
| Class C | 0.02% | 0.02% | 0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.01% | 100% | 99% |
| Class D | 0.01% | 0.01% | 0.01% | 100% | 101% |
| Class F | 0.03% | 0.03% | 0.03% | 100% | 101% |
| Class SCC | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# Proposal 9: on signalling of bit_depth_minus8 and min_qp_prime_ts_minus4

In the current VVC working draft, both bit_depth_minus8 and min_qp_prime_ts_minus4 are coded using ue(v). In our opinion, it would be simpler to code them using fixed length code such as u(3) for bit_depth_minus8 and u(6) for min_qp_prime_ts_minus4.

Proposal: We propose to change the signalling of bit_depth_minus8 and min_qp_prime_ts_minus4 from ue(v) to u(3) and u(6), respectively.

The spec text for this proposal is as follows:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| bit_depth_minus8 | u(3) |
| min_qp_prime_ts_minus4 | u(6) |
| … |  |
| } |  |

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).