| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0224-v2 |
| --- | --- |

| Title: | AHG9/AHG12: On raster scan slice within a picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Chun-Chia Chen, Chih-Wei Hsu, Yu-Ling Hsiao, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, chunchia.chen, cw.hsu, yuling.hsiao, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to allow and support the raster scan slice in units of CTU within a picture. It is alleged the proposed support enables the HEVC-like raster scan slice and the coding efficiency benefits from a larger tile size partition in a picture.

- the proposed raster scan slice related text change

- the proposed text change on single_slice_per_subpic_flag

  - removing constraint on single_slice_per_subpic_flag (When subpics_present_flag is equal to 0, single_slice_per_subpic_flag shall be equal to 0.)

  - extending to the picture with inference for value of num_slices_in_pic_minus1 (When single_slice_per_subpic_flag is equal to 1 and sps_num_subpics_minus1 is equal to 0, num_slices_in_pic_minus1 is inferred to be equal to 0) and saving the rest of singalling for one subpicture case.

- some other semantics improvement or clarification

V2 with the update on the abstract

Introduction

In the VVC Draft 7 [1], the picture partition and slice layout information are specified in the PPS. When no_pic_partition_flag is equal to 1, it specifies that no picture partitioning is applied to each picture referring to the PPS. It may imply a single tile or no tile partition in the picture although the tile partition parameters are not present.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| … |  |
| output_flag_present_flag | u(1) |
| mixed_nalu_types_in_pic_flag | u(1) |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| … |  |
| picture_header_extension_present_flag | u(1) |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

When no_pic_partition_flag is equal to 0, it specifies each picture referring to the PPS may be partitioned into more than one tile or slice. Slice is specified as: An integer number of complete tiles or an integer number of consecutive complete CTU rows within a tile of a picture that are exclusively contained in a single NAL unit. Simply, the tile in slice case (namely the tile-in-slice case) and the rectangular slice in tile (the rect-slice-in-tile case). Basically, the tile partitioning is required for specifying slices within a picture. For the case similar to that in HEVC that slice is spcecified directly in CTUs (the ctu-slice-in-pic case) is not supported. It is observed that some coding tools may not be allowed to use across the tile partition boundaries, e.g., spatial merge candidates for regular merge mode, affine merge mode reference, and the intra-prediction reference etc., the coding efficiency to some extent may be decreased. Therefore it may be practically desired for some applications to use a reasonable number of tile partitions or no tile partitions.

Proposal

This contribution proposes that the tile partition for the raster scan slice within a picture is optional and the slice partition is specified in units of CTU, i.e., the ctu-slice-in-pic case as illustrated below.

The proposed syntax and semantics for pic_parameter_set_rbsp( ) and slice_header( ) are as follows and highlighted in green.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| … |  |
| output_flag_present_flag | u(1) |
| mixed_nalu_types_in_pic_flag | u(1) |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| else |  |
| { |  |
| raster_scan_slice_in_pic_enabled_flag | u(1) |
| if( raster_scan_slice_in_pic_enabled_flag ) |  |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| … |  |
| picture_header_extension_present_flag | u(1) |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( raster_scan_slice_in_pic_enabled_flag ¦ ¦ !rect_slice_flag && NumTilesInPic = 1) |  |
| slices_in_pic_present_flag | u(1) |
| if( slices_in_pic_present_flag ) { |  |
| slice_address | u(v) |
| num_ctus_in_slice_minus1 | ue(v) |
| } |  |
| else if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| … |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

subpics_present_flag equal to 1 specifies that one or more subpictures are present in each coded picture in the CLVS and  subpicture parameters are present in in the SPS RBSP syntax. subpics_present_flag equal to 0 specifies that no subpictures are present in each coded picture in the CLVS and subpicture parameters are not present in the SPS RBSP syntax.

NOTE 2 – When a bitstream is the result of a sub-bitstream extraction process and contains only a subset of the subpictures of the input bitstream to the sub-bitstream extraction process, it might be required to set the value of subpics_present_flag equal to 1 in the RBSP of the SPSs. When subpics_present_flag equal to 0, each coded picture in the CLVS may be considered as a subpicture in a bitstream extraction and merging process.

sps_num_subpics_minus1 plus 1 specifies the number of subpictures. sps_num_subpics_minus1 shall be in the range of 0 to 254. When not present, the value of sps_num_subpics_minus1 is inferred to be equal to 0. sps_num_subpics_minus1 equal to 0 indicates that the each coded picture in the CLVS is the subpicture.

rect_slice_flag equal to 0 specifies that tiles or CTUs within each slice are in raster scan order and the slice information is not signalled in PPS. rect_slice_flag equal to 1 specifies that tiles or CTUs within each slice cover a rectangular region of the picture and the slice information is signalled in the PPS. When not present, rect_slice_flag is inferred to be equal to 1. When subpics_present_flag is equal to 1, the value of rect_slice_flag shall be equal to 1. When raster_scan_slice_in_pic_enabled_flag is equal to 1, the value of rect_slice_flag is inferred to be equal to 0.

no_pic_partition_flag equal to 1 specifies that no picture tile partitioning applied to each picture referring to the PPS. no_pic_partition_flag equal to 0 specifies each picture referring to the PPS may be partitioned into more than one tile or slice.

raster_scan_slice_in_pic_enabled_flag equal to 0 specifies that no raster scan slice partitioning applied to each picture referring to the PPS. raster_scan_slice_in_pic_enabled_flag equal to 1 specifies that, when no_pic_partition_flag is equal to 1, each picture referring to the PPS may be partitioned into more then one raster scan slice. When not present, the value of raster_scan_slice_in_pic_enabled_flag is inferred to be equal to 0.

single_slice_per_subpic_flag equal to 1 specifies that each subpicture consists of one and only one rectangular slice. single_slice_per_subpic_flag equal to 0 specifies that each subpicture may consist one or more rectangular slices. When subpics_present_flag is equal to 0, single_slice_per_subpic_flag shall be equal to 0. When single_slice_per_subpic_flag is equal to 1, num_slices_in_pic_minus1 is inferred to be equal to sps_num_subpics_minus1. When single_slice_per_subpic_flag is equal to 1 and sps_num_subpics_minus1 is equal to 0, num_slices_in_pic_minus1 is inferred to be equal to 0 (i.e. the current picture is the subpicure and consists only one rectangular slice).

slices_in_pic_present_flag equal to 1 specifies that mulitple slices are present in the current picture. slices_in_pic_present_flag equal to 0 specifyies no multiple slices is present in the current picture. When not present, slices_in_pic_present_flag is inferred to be equal to 0.

num_ctus_in_slice_minus1 plus 1 specifies the number of CTUs in the current slice. When slices_in_pic_present_flag is equal to 1, the value of num_ctus_in_slice_minus1 shall be in the range of 0 to PicSizeInCtbsY − 1, inclusive. When not present, num_ctus_in_slice_minus1 is inferred to be equal to PicSizeInCtbsY − 1.

slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0.

If slices_in_pic_present_flag is equal to 1, the following applies:

- The slice address is the first CTB in the slice, in CTB raster scan of a picture.

- The length of slice_address is Ceil( Log2 ( PicSizeInCtbsY ) ) bits.

- The value of slice_address shall be in the range of 0 to PicSizeInCtbsY − 1, inclusive, and the value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

Otherwise, iIf rect_slice_flag is equal to 0, the following applies:

- The slice address is the raster scan tile index.

- The length of slice_address is Ceil( Log2 ( NumTilesInPic ) ) bits.

- The value of slice_address shall be in the range of 0 to NumTilesInPic − 1, inclusive.

Otherwise (rect_slice_flag is equal to 1), the following applies:

- The slice address is the slice index of the slice within the SubPicIdx-th subpicture.

- The length of slice_address is Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ) bits.

- The value of slice_address shall be in the range of 0 to NumSlicesInSubpic[ SubPicIdx ] − 1, inclusive.

It is a requirement of bitstream conformance that the following constraints apply:

- If rect_slice_flag is equal to 0 or subpics_present_flag is equal to 0, the value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

- Otherwise, the pair of slice_subpic_id and slice_address values shall not be equal to the pair of slice_subpic_id and slice_address values of any other coded slice NAL unit of the same coded picture.

- When rect_slice_flag is equal to 0, the slices of a picture shall be in increasing order of their slice_address values.

- The shapes of the slices of a picture shall be such that each CTU, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded CTU(s).

num_tiles_in_slice_minus1 plus 1, when present, specifies the number of tiles in the slice. The value of num_tiles_in_slice_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive.

The variable NumCtuInCurrSlice, which specifies the number of CTUs in the current slice, and the list CtbAddrInCurrSlice[ i ], for i ranging from 0 to NumCtuInCurrSlice − 1, inclusive, specifying the picture raster scan address of the i-th CTB within the slice, are derived as follows:

if (slices_in_pic_present_flag)

NumCtuInCurrSlice = num_ctus_in_slice_minus1 + 1

else if( rect_slice_flag ) {
	picLevelSliceIdx = SliceSubpicToPicIdx[ SubPicIdx ][ slice_address ]
	NumCtuInCurrSlice = NumCtuInSlice[ picLevelSliceIdx ]
	for( i = 0; i < NumCtuInCurrSlice; i++ )
		CtbAddrInCurrSlice[ i ] = CtbAddrInSlice[ picLevelSliceIdx ][ i ]	(7-80)
} else {
	NumCtuInCurrSlice = 0
	for( tileIdx = slice_address; tileIdx  <=  slice_address + num_tiles_in_slice_minus1[ i ]; tileIdx++ ) {
		tileX = tileIdx % NumTileColumns
		tileY = tileIdx / NumTileColumns
		for( ctbY = tileRowBd[ tileY ]; ctbY < tileRowBd[ tileY + 1 ]; ctbY++ ) {
			for( ctbX = tileColBd[ tileX ]; ctbX < tileColBd[ tileX + 1 ]; ctbX++ ) {
				CtbAddrInCurrSlice[ NumCtuInCurrSlice ] = ctbY * PicWidthInCtb + ctbX
				NumCtuInCurrSlice++
			}
		}
	}
}

Test results

The experiments were conducted on top of VTM7.0. The common test conditions [2] are used for evaluation. Tables 2 to Table 7 summarize the results of the 1, 2, and 4 CTUs per tile with in-loop filtering not across or across tile boundaries, respectively. The detailed configuration setting is shown in Table 1.

Table 1. Tile partition configuration setting

| Tiles setting | Table 2 | Table 3 | Table 4 | Table 5 | Table 6 | Table 7 |
| --- | --- | --- | --- | --- | --- | --- |
| TileUniformSpacing | 1 | 1 | 1 | 1 | 1 | 1 |
| UniformTileColsWidthMinus1 | 0 | 0 | 1 | 1 | 3 | 3 |
| UniformTileRowHeightMinus1 | 0 | 0 | 0 | 0 | 0 | 0 |
| LFCrossTileBoundaryFlag | 0 | 1 | 0 | 1 | 0 | 1 |

Table 2. Results of the 1 CTU per tile

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 5.45% | 3.56% | 4.99% | 130% | 96% |
| Class A2 | 3.57% | 1.77% | 3.44% | 112% | 89% |
| Class B | 2.21% | 1.63% | 1.48% | 108% | 95% |
| Class C | 1.24% | 1.13% | 1.30% | 103% | 99% |
| Class E | 2.13% | 2.17% | 2.56% | 100% | 91% |
| Overall | 2.75% | 1.96% | 2.53% | 109% | 94% |
| Class D | 0.86% | 0.65% | 0.89% | 96% | 98% |
| Class F | 5.63% | 5.44% | 5.34% | 86% | 90% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 24.87% | 24.14% | 27.58% | 109% | 89% |
| Class A2 | 20.36% | 18.77% | 21.06% | 108% | 94% |
| Class B | 14.56% | 13.91% | 14.33% | 106% | 93% |
| Class C | 6.93% | 6.95% | 7.52% | 101% | 96% |
| Class E |  |  |  |  |  |
| Overall | 15.74% | 15.07% | 16.51% | 106% | 93% |
| Class D | 4.97% | 4.64% | 5.58% | 100% | 95% |
| Class F | 13.66% | 13.60% | 13.88% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 13.54% | 12.23% | 10.86% | 108% | 88% |
| Class C | 6.18% | 5.87% | 4.41% | 106% | 91% |
| Class E | 32.39% | 30.18% | 30.98% | 102% | 86% |
| Overall | 15.80% | 14.60% | 13.74% | 106% | 88% |
| Class D | 5.21% | 3.72% | 4.09% | 104% | 94% |
| Class F | 21.32% | 21.17% | 20.76% | 104% | 93% |

Table 3. Results of the 1 CTU per tile with in-loop filtering across tile boundary

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 4.84% | 1.96% | 2.86% | 127% | 93% |
| Class A2 | 3.04% | 0.91% | 2.67% | 114% | 91% |
| Class B | 1.90% | 0.85% | 0.51% | 107% | 96% |
| Class C | 1.05% | 0.60% | 0.61% | 99% | 95% |
| Class E | 1.70% | 1.28% | 1.31% | 104% | 96% |
| Overall | 2.36% | 1.06% | 1.42% | 109% | 94% |
| Class D | 0.76% | 0.28% | 0.48% | 97% | 98% |
| Class F | 5.42% | 4.92% | 4.87% | 87% | 94% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 23.31% | 22.58% | 25.86% | 107% | 94% |
| Class A2 | 19.27% | 17.88% | 20.38% | 107% | 95% |
| Class B | 13.91% | 12.94% | 13.41% | 106% | 94% |
| Class C | 6.60% | 6.56% | 7.06% | 102% | 96% |
| Class E |  |  |  |  |  |
| Overall | 14.91% | 14.16% | 15.60% | 105% | 95% |
| Class D | 4.81% | 4.40% | 5.30% | 100% | 96% |
| Class F | 13.41% | 13.28% | 13.49% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 13.09% | 11.38% | 10.78% | 107% | 90% |
| Class C | 5.91% | 5.30% | 3.88% | 105% | 90% |
| Class E | 32.06% | 28.74% | 28.98% | 101% | 89% |
| Overall | 15.44% | 13.70% | 13.03% | 105% | 90% |
| Class D | 5.06% | 3.86% | 4.07% | 103% | 93% |
| Class F | 21.10% | 20.39% | 20.53% | 103% | 94% |

Table 4. Results of the 2 CTUs per tile

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 3.76% | 1.92% | 3.10% | 125% | 90% |
| Class A2 | 2.42% | 0.74% | 2.28% | 112% | 91% |
| Class B | 1.45% | 0.85% | 0.62% | 105% | 92% |
| Class C | 0.82% | 0.65% | 0.88% | 98% | 93% |
| Class E | 1.29% | 0.93% | 1.20% | 101% | 93% |
| Overall | 1.83% | 0.98% | 1.46% | 107% | 92% |
| Class D | 0.48% | 0.18% | 0.62% | 96% | 96% |
| Class F | 3.12% | 2.84% | 2.97% | 93% | 90% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 15.86% | 15.38% | 18.19% | 105% | 96% |
| Class A2 | 12.36% | 11.05% | 12.94% | 106% | 94% |
| Class B | 9.49% | 8.62% | 9.26% | 104% | 94% |
| Class C | 4.76% | 4.63% | 5.23% | 101% | 94% |
| Class E |  |  |  |  |  |
| Overall | 10.08% | 9.39% | 10.71% | 104% | 94% |
| Class D | 2.62% | 2.31% | 2.42% | 101% | 96% |
| Class F | 8.03% | 8.13% | 8.26% | 102% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 8.76% | 7.35% | 6.64% | 110% | 91% |
| Class C | 4.15% | 3.98% | 2.49% | 107% | 90% |
| Class E | 17.55% | 16.62% | 16.08% | 106% | 93% |
| Overall | 9.42% | 8.54% | 7.62% | 108% | 91% |
| Class D | 2.75% | 2.06% | 1.63% | 105% | 94% |
| Class F | 12.11% | 11.77% | 11.80% | 107% | 96% |

Table 5. Results of the 2 CTUs per tile with in-loop filtering across tile boundary

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 3.34% | 0.79% | 1.62% | 128% | 94% |
| Class A2 | 2.03% | 0.14% | 1.72% | 115% | 92% |
| Class B | 1.24% | 0.30% | -0.05% | 107% | 96% |
| Class C | 0.69% | 0.28% | 0.40% | 100% | 96% |
| Class E | 0.96% | 0.50% | 0.42% | 101% | 93% |
| Overall | 1.55% | 0.38% | 0.70% | 109% | 94% |
| Class D | 0.43% | -0.03% | 0.39% | 99% | 101% |
| Class F | 2.95% | 2.49% | 2.64% | 94% | 94% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 14.74% | 14.22% | 16.82% | 106% | 88% |
| Class A2 | 11.62% | 10.31% | 12.38% | 106% | 91% |
| Class B | 9.02% | 7.98% | 8.64% | 105% | 92% |
| Class C | 4.53% | 4.39% | 5.08% | 101% | 95% |
| Class E |  |  |  |  |  |
| Overall | 9.48% | 8.74% | 10.07% | 104% | 92% |
| Class D | 2.52% | 2.21% | 2.48% | 100% | 96% |
| Class F | 7.89% | 7.87% | 8.02% | 102% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 8.29% | 6.76% | 6.11% | 107% | 91% |
| Class C | 3.96% | 3.62% | 2.61% | 107% | 93% |
| Class E | 17.13% | 15.27% | 15.03% | 104% | 92% |
| Overall | 9.06% | 7.84% | 7.17% | 107% | 92% |
| Class D | 2.59% | 1.91% | 1.24% | 104% | 95% |
| Class F | 11.93% | 11.33% | 11.27% | 106% | 95% |

Table 6. Results of the 4 CTUs per tile

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.71% | 1.05% | 2.18% | 125% | 94% |
| Class A2 | 1.70% | 0.31% | 1.63% | 112% | 91% |
| Class B | 0.92% | 0.37% | 0.16% | 106% | 94% |
| Class C | 0.52% | 0.37% | 0.49% | 99% | 96% |
| Class E | 0.94% | 0.64% | 0.73% | 102% | 95% |
| Overall | 1.26% | 0.52% | 0.91% | 108% | 94% |
| Class D | 0.23% | 0.04% | 0.32% | 97% | 98% |
| Class F | 1.33% | 1.10% | 1.15% | 98% | 94% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 11.13% | 10.56% | 13.15% | 106% | 92% |
| Class A2 | 8.39% | 7.12% | 8.94% | 105% | 88% |
| Class B | 6.28% | 5.49% | 6.15% | 104% | 92% |
| Class C | 2.96% | 2.89% | 2.95% | 102% | 97% |
| Class E |  |  |  |  |  |
| Overall | 6.79% | 6.14% | 7.25% | 104% | 92% |
| Class D | 1.24% | 1.06% | 1.26% | 101% | 98% |
| Class F | 4.65% | 4.70% | 4.80% | 104% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 5.76% | 4.53% | 5.31% | 108% | 90% |
| Class C | 2.71% | 2.62% | 1.37% | 107% | 93% |
| Class E | 11.61% | 10.24% | 10.81% | 103% | 88% |
| Overall | 6.21% | 5.32% | 5.37% | 106% | 91% |
| Class D | 1.40% | 1.22% | 0.67% | 104% | 95% |
| Class F | 7.51% | 7.83% | 7.71% | 108% | 95% |

Table 7. Results of the 4 CTUs per tile with in-loop filtering across tile boundary

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.38% | 0.14% | 0.99% | 125% | 94% |
| Class A2 | 1.38% | -0.17% | 1.16% | 111% | 92% |
| Class B | 0.77% | -0.02% | -0.31% | 106% | 96% |
| Class C | 0.42% | 0.12% | 0.12% | 99% | 95% |
| Class E | 0.65% | 0.15% | 0.12% | 102% | 95% |
| Overall | 1.04% | 0.04% | 0.32% | 108% | 94% |
| Class D | 0.21% | -0.08% | 0.17% | 98% | 99% |
| Class F | 1.21% | 0.82% | 0.87% | 98% | 92% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 10.23% | 9.74% | 12.15% | 105% | 92% |
| Class A2 | 7.81% | 6.62% | 8.53% | 105% | 94% |
| Class B | 5.93% | 4.98% | 5.61% | 105% | 96% |
| Class C | 2.77% | 2.66% | 2.71% | 103% | 95% |
| Class E |  |  |  |  |  |
| Overall | 6.32% | 5.64% | 6.73% | 104% | 95% |
| Class D | 1.20% | 1.03% | 1.18% | 101% | 95% |
| Class F | 4.54% | 4.57% | 4.61% | 104% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 5.40% | 3.89% | 4.65% | 107% | 90% |
| Class C | 2.54% | 2.35% | 1.00% | 107% | 93% |
| Class E | 11.21% | 9.66% | 9.99% | 104% | 91% |
| Overall | 5.90% | 4.82% | 4.77% | 106% | 91% |
| Class D | 1.29% | 0.99% | 0.40% | 104% | 95% |
| Class F | 7.30% | 7.47% | 8.58% | 106% | 95% |

Conclusion

This contribution proposes to allow the HEVC-like raster scan slice in units of CTU within a picture without the tile partition. The test results show the coding efficiency benefits from a larger tile size partition in a picture. The proponents suggest the group adopt the support of the above raster scan slice in units of CTU within a picture.

- References

- “Versatile Video Coding (Draft 7),” JVET-P2001-vE, October 2019, Geneva, Switzerland.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).