| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0260 |
| --- | --- |

| Title: | AHG9: Intended display resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Tel: Email: | +46 721 88 21 69 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution contains a proposal for syntax elements to be added to the Decoding Parameter Set (DPS). The syntax elements indicate the intended display resolution of the pictures encoded in the layer to which the DPS applies. It is asserted that it is of high importance for receivers and decoders to determine the resolution that the decoded pictures are intended to be displayed at. It is asserted that dynamic encoding techniques used for example in Adaptive Bit Rate (ABR) streaming applications and Reference Picture Resampling (RPR) makes it difficult for receivers and decoders to determine the intended display resolution. The second version of this document adds an alternative solution in which cropping information is provided in the SPS.

# Introduction

In the current draft of VVC, the resolution of coded pictures is indicated in the PPS and it can change at any point in a bitstream, also within a CVS since the introduction of RPR. The resolution of the pictures can be reduced as a technique to improve the quality of the compressed video rather than increasing quantization which can lead to objectionable compression artifacts. This technique is used both in Adaptive Streaming Applications and video conferencing. Regardless of if the resolution changes occur at IDR pictures (e.g. at scene changes within a DASH segment) or within a CVS (e.g. in response to bandwidth fluctuations in a video conferencing call) it is assumed the reduced resolution will be upsampled by the receiver to the full resolution in order to provide a good user experience when the video is displayed. It is asserted that it is desirable for a receiver/decoder to know the intended display resolution (typically the original resolution of the video) even when the coded video starts with reduced resolution. It is asserted that knowing the intended display resolution is of high importance in order to properly set up memory allocations, rescaling and cropping configuration and display areas at the start of decoding a bitstream.

It can be argued that the pic_width_max_in_luma_samples and pic_height_max_in_luma_samples in the SPS may be used to determine intended display resolution. However, since no cropping information is present in the SPS, the actual information about output resolution cannot be determined from the SPS. Furthermore, since a new SPS can be signaled and activated at an IDR picture at any point in the bitstream, the output resolution cannot be determined even if a PPS with pic_width_in_luma_samples equal to pic_width_max_in_luma_samples and pic_height_in_luma_samples equal to pic_height_max_in_luma_samples is activated in the first access unit of the bitstream.

# Example bitstreams

Figure 1 -3 shows examples of valid VVC bitstreams according to the latest VVC draft. Without any additional information about intended display resolution it is extremely difficult for a receiver/decoder to make correct assumptions of how the decoded pictures should be displayed.

Figure 1. Valid bitsteam A

Figure 2. Valid bitsteam B

Figure 3. Valid bitsteam C

# Proposal

## Option A

It is proposed to add intended display width and intended display height as syntax elements in the DPS. Since the DPS contains information that applies to an entire bitstream, the receiver/decoder can use this information to make appropriate allocation and configuration for correct display of the decoded video.

Figure 4. Bitsteam A with addition of intended display resolution in DPS

| decoding_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| dps_decoding_parameter_set_id | u(4) |
| dps_max_sublayers_minus1 | u(3) |
| dps_reserved_zero_5bits | u(5) |
| dps_num_ptls_minus1 | u(4) |
| for( i = 0; i <= dps_num_ptls_minus1; i++ ) |  |
| profile_tier_level( 1, dps_max_sublayers_minus1 ) |  |
| intended_display_width_in_luma_samples | u(16) |
| intended_display_height_in_luma_samples | u(16) |
| dps_extension_flag | u(1) |
| if( dps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

intended_display_width_in_luma_samples specifies the intended display width, in units of luma samples, of each cropped decoded picture referring to the DPS.

intended_display_height_in_luma_samples specifies the intended display height, in units of luma samples, of each cropped decoded picture referring to the DPS.

NOTE – The intended display width and intended display height provide guidance for the width, height and aspect ratio to which the cropped decoded pictures are inteded to be mapped. When the intended display width and height is different from the coded display width and height, a resampling process may be applied to the cropped decoded picture but no specific resampling method is mandated or suggested. The intended display width and intended display height does not impose any constraints on the coded picture width and coded picture heights as expressed by pic_width_in_luma_samples and pic_height_in_luma_samples, respectively.

## Option B

An alternative method is proposed to address the problem.

By signaling a cropping window in SPS it will be clear to a receiver/decoder if cropping will be applied when the coded resolution is equal to the maximum coded resolution.

The following is proposed:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| gdr_enabled_flag | u(1) |
| sps_seq_parameter_set_id | u(4) |
| chroma_format_idc | u(2) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| ref_pic_resampling_enabled_flag | u(1) |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| sps_conformance_window_flag | u(1) |
| if( sps_conformance_window_flag ) { |  |
| sps_conf_win_left_offset | ue(v) |
| sps_conf_win_right_offset | ue(v) |
| sps_conf_win_top_offset | ue(v) |
| sps_conf_win_bottom_offset | ue(v) |
| } |  |
| … |  |
| } |  |

sps_conformance_window_flag equal to 1 indicates that the conformance cropping window offset parameters follow next in the SPS. conformance_window_flag equal to 0 indicates that the conformance cropping window offset parameters are not present.

sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset specify the cropping window that is applied to pictures with pic_width_in_luma_samples equal to pic_width_max_in_luma_samples and pic_height_in_luma_samples  equal to pic_height_max_in_luma_samples. When conformance_window_flag is equal to 0, the values of sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset are inferred to be equal to 0.

Additionally, the following editorial change is proposed to the PPS to avoid that syntax elements are substrings of other syntax elements:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| pps_conformance_window_flag | u(1) |
| if(pps_conformance_window_flag ) { |  |
| pps_conf_win_left_offset | ue(v) |
| pps_conf_win_right_offset | ue(v) |
| pps_conf_win_top_offset | ue(v) |
| pps_conf_win_bottom_offset | ue(v) |
| } |  |
| … |  |

pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset specify the samples of the pictures in the CLVS that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output. When conformance_window_flag is equal to 0, the values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset are inferred to be equal to 0.

The conformance cropping window contains the luma samples with horizontal picture coordinates from SubWidthC * pps_conf_win_left_offset to pic_width_in_luma_samples − ( SubWidthC * pps_conf_win_right_offset + 1 ) and vertical picture coordinates from SubHeightC * pps_conf_win_top_offset to pic_height_in_luma_samples − ( SubHeightC * pps_conf_win_bottom_offset + 1 ), inclusive.

The value of SubWidthC * ( pps_conf_win_left_offset + pps_conf_win_right_offset ) shall be less than pic_width_in_luma_samples, and the value of SubHeightC * ( pps_conf_win_top_offset + pps_conf_win_bottom_offset ) shall be less than pic_height_in_luma_samples.

When ChromaArrayType is not equal to 0, the corresponding specified samples of the two chroma arrays are the samples having picture coordinates ( x / SubWidthC, y / SubHeightC ), where ( x, y ) are the picture coordinates of the specified luma samples.

NOTE – The conformance cropping window offset parameters are only applied at the output. All internal decoding processes are applied to the uncropped picture size.

Let ppsA and ppsB be any two PPSs referring to the same SPS. It is a requirement of bitstream conformance that, when ppsA and ppsB have the same the values of pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, ppsA and ppsB shall have the same values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset, respectively.

Additionally, the following is proposed in the semantics of the PPS:

When pic_width_in_luma_samples is equal to pic_width_max_in_luma_samples and pic_height_in_luma_samples  is equal to pic_height_max_in_luma_samples, it is a requirement of bitstream conformance that, pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset, are equal to sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset, respectively.

# Summary

This contribution contains a proposal for syntax elements to be added to the Decoding Parameter Set (DPS) indicating the intended display resolution. It is proposed to add intended_display_width_in_luma_samples and intended_display_height_in_luma_samples in the next draft of the VVC. The second version of this document adds an alternative solution in which cropping information is provided in the SPS.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).