| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0264 |
| --- | --- |

| Title: | CE3-related: Computation reduction in the derivation of Rice parameter for abs_remainder[ ] in regular residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheung Auyeung, Xiang Li, Xin Zhao, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 {cauyeung, xlxiangli, xinzzhao, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes to reduce the computation in the derivation of Rice parameter for abs_remainder[ ] in residual coding without transform skip by replacing the calculation of locSumAbs in VVC Draft 7. Three variants of the proposal (A,B,C) with similar performance are presented.

For proposal A with CTC-QP AI/RA/LB configurations, the BDR is 0.03%/0.03%/-0.01%, the encoding time is 100%/100%/100%, and the decoding time is 99%/99%/101%, respectively. For proposal A low-QP AI/RA/LB configurations, the BDR is 0.17/xxxx/0.02%, the encoding time is 99%/xxxx/99%, and the decoding time is 97%/xxx/98%, respectively.

For proposal B with CTC-QP AI/RA/LB configurations, the BDR is 0.00%/0.00%/-0.02%, the encoding time is 100%/100%/99%, and the decoding time is 99%/99%/100%, respectively. For proposal B low-QP AI/RA/LB configurations, the BDR is -0.02/xxxx/0.00%, the encoding time is 100%/xxxx/100%, and the decoding time is 96%/xxx/97%, respectively.

For proposal C with CTC-QP AI/RA/LB configurations, the BDR is 0.00%/0.00%/-0.02%, the encoding time is 99%/99%/99%, and the decoding time is 97%/97%/98%, respectively. For proposal C low-QP AI/RA/LB configurations, the BDR is -0.02/xxxx/0.01%, the encoding time is 99%/xxxx/99%, and the decoding time is 96%/xxx/95%, respectively.

More accurate decoding timing, by decoding the bitstream multiple times in a single computer, shall be updated in a later revision of this document.

# Introduction

In VVC Draft 7 [1], the derivation of Rice parameter for abs_remainder[ ] and dec_abs_level[ ] for the residual coding of transform domain coefficients, i.e. without transform skip, required to compute locSumAbs from AbsLevel[ x ][ y ]:

locSumAbs = 0
if( xC < (1 << log2TbWidth) − 1 ) {
	locSumAbs += AbsLevel[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 )
		locSumAbs += AbsLevel[ xC + 2 ][ yC ]
	if( yC < (1 << log2TbHeight) − 1 )
		locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ]	(1494)
}
if( yC < (1 << log2TbHeight) − 1 ) {
	locSumAbs += AbsLevel[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) − 2 )
		locSumAbs += AbsLevel[ xC ][ yC + 2 ]
} 
locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 )

The computation of locSumAbs is implemented in VTM-7.0 [2] with multiple branches and additions. This contribution propose to replace this calculation for abs_remainder[ ] by simpler calculation without branches.

# Proposed simplification to the derivation of Rice parameter for abs_remainder[ ]

This contribution proposes to reduce the computation in the derivation of the Rice parameter for transform domain abs_remainder[ n ] in a subblock by means of Markov Decision Process (MDP).

The MDP has two parts. One part is to determine the Rice paramter (ricePar) according to the current state by an action function, action(state). The second part is to determine the next state according to the curent state and the decoded remainder (rem) by a state transition function, nextSate(state, rem). The proposed decoding process with MDP can be described by the following pseudo code:

ricePar = action(state)

rem = decodeRem(ricePar)

state = nextState(state, rem) = (state + rem)>>1

The initial state is set to 0 at the beginning of a subblock.

## Proposed A

In proposal A, the action function is defined as follows:

ricePar = 0

The state and the nextState() function are not needed.

## Proposal B

In proposal B, the action function is defined as follows:

ricePar = action(state) = ((state>0) + (state>2)

## Proposed C

In proposal C, the action function is defined as follows:

ricePar = action(state) = ((state>0) + (state>2) + (state>7))

# Comparison of worst computational complexity

The following table compares the worst case number of operations and lookup table size for deriving the Rice parameter for abs_remainder[ ]. In the following table, “if”, “min” and “max” operations are considered as branches as in x86 ISA [4].

For VTM-7, the number of listed operations are determined from the function templateAbsSum() in VTM-7.0 [2]. For proposal A, B, C, the number of listed operations is determined from the action function and the state transition function.

|  | branches | shift | Addition /subtraction | Multiplication | Logical comparison | Table lookup | Table size |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VTM-7 | 7 | 1 | 14 | 2 | 0 | 1 | 32 |
| Proposal A | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Proposal B | 0 | 1 | 2 | 0 | 2 | 0 | 0 |
| Proposal C | 0 | 1 | 3 | 0 | 3 | 0 | 0 |

# Results

The testing condition of the proposed follows the CE3 description [3] testing conditions for standard and low QP. Since lossless coding in CE3 does not use regular residual coding, the lossless results are not reported here.

## Summary of overall luma BD rate changes

|  | Standard QP |  |  | Low QP |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | AI | RA | LDB | AI | RA | LDB |
| Proposal A | 0.03% | 0.02% | -0.01% | 0.17% |  | 0.02% |
| Proposal B | 0.00% | 0.00% | -0.02% | -0.02% |  | 0.00% |
| Proposal C | 0.00% | 0.00% | -0.02% | -0.02% |  | 0.01% |

## Proposal A standard QP results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.05% | 0.20% | 100% | 100% |
| Class A2 | 0.06% | 0.07% | -0.01% | 100% | 100% |
| Class B | 0.02% | 0.03% | -0.10% | 99% | 97% |
| Class C | 0.01% | 0.03% | 0.01% | 100% | 101% |
| Class E | 0.03% | -0.01% | 0.05% | 99% | 98% |
| Overall | 0.03% | 0.02% | 0.01% | 100% | 99% |
| Class D | 0.00% | 0.00% | -0.04% | 100% | 99% |
| Class F | 0.01% | -0.04% | -0.03% | 100% | 100% |
| Class TGM | 0.01% | 0.02% | 0.01% | 99% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.11% | 0.05% | 101% | 100% |
| Class A2 | -0.01% | 0.05% | 0.11% | 100% | 101% |
| Class B | 0.01% | 0.12% | 0.21% | 99% | 98% |
| Class C | 0.01% | 0.04% | 0.12% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.04% | 0.14% | 100% | 99% |
| Class D | 0.00% | -0.06% | -0.02% | 100% | 101% |
| Class F | 0.00% | 0.05% | 0.08% | 100% | 100% |
| Class TGM | -0.02% | -0.05% | -0.03% | 99% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.22% | -0.38% | 101% | 102% |
| Class C | 0.01% | -0.06% | 0.08% | 100% | 100% |
| Class E | -0.02% | 0.47% | -0.11% | 99% | 100% |
| Overall | -0.01% | 0.00% | -0.16% | 100% | 101% |
| Class D | -0.02% | -0.42% | -0.27% | 100% | 98% |
| Class F | 0.10% | -0.34% | 0.42% | 98% | 97% |
| Class TGM | -0.03% | -0.06% | -0.09% | 99% | 98% |

## Proposal A low QP results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.13% | 0.10% | 100% | 97% |
| Class A2 | 0.52% | 0.63% | 0.63% | 100% | 99% |
| Class B | 0.08% | 0.12% | 0.11% | 98% | 95% |
| Class C | 0.21% | 0.20% | 0.24% | 99% | 100% |
| Class E | 0.06% | 0.18% | 0.11% | 100% | 97% |
| Overall | 0.17% | 0.23% | 0.22% | 99% | 97% |
| Class D | 0.24% | 0.24% | 0.27% | 99% | 101% |
| Class F | 0.10% | 0.09% | 0.13% | 99% | 100% |
| Class TGM | 0.26% | 0.23% | 0.22% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.04% | 100% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.02% | 0.06% | 0.05% | 99% | 96% |
| Class C | 0.05% | 0.04% | 0.00% | 99% | 95% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.07% | 0.10% | -0.01% | 99% | 97% |
| Class F | 0.08% | 0.16% | 0.12% | 99% | 99% |
| Class TGM | 0.17% | 0.37% | 0.36% | 99% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.04% | 0.05% | 100% | 100% |
| Class C | 0.02% | 0.03% | 0.06% | 99% | 96% |
| Class E | -0.01% | 0.00% | 0.02% | 99% | 97% |
| Overall | 0.02% | 0.03% | 0.04% | 99% | 98% |
| Class D | 0.01% | 0.06% | -0.01% | 99% | 97% |
| Class F | 0.02% | 0.12% | 0.01% | 99% | 99% |
| Class TGM | 0.16% | 0.34% | 0.37% | 99% | 96% |

## Proposal B standard QP results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | -0.10% | 0.05% | 100% | 100% |
| Class A2 | 0.00% | -0.02% | -0.04% | 100% | 99% |
| Class B | 0.01% | 0.02% | 0.00% | 100% | 97% |
| Class C | 0.00% | 0.02% | -0.02% | 100% | 100% |
| Class E | -0.01% | 0.00% | 0.04% | 100% | 99% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 99% |
| Class D | -0.01% | 0.11% | 0.03% | 99% | 99% |
| Class F | 0.03% | -0.03% | -0.03% | 100% | 98% |
| Class TGM | -0.01% | -0.02% | -0.02% | 101% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.04% | -0.01% | 99% | 100% |
| Class A2 | -0.01% | 0.00% | 0.00% | 99% | 100% |
| Class B | -0.01% | 0.06% | 0.08% | 99% | 98% |
| Class C | 0.01% | 0.00% | 0.07% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.04% | 100% | 99% |
| Class D | 0.01% | -0.23% | -0.15% | 99% | 100% |
| Class F | -0.04% | 0.10% | 0.18% | 100% | 100% |
| Class TGM | -0.03% | -0.07% | -0.01% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.29% | 0.32% | 100% | 102% |
| Class C | 0.00% | -0.24% | -0.26% | 99% | 99% |
| Class E | -0.07% | 0.77% | -0.90% | 100% | 98% |
| Overall | -0.02% | -0.01% | -0.18% | 99% | 100% |
| Class D | -0.07% | -0.12% | -0.63% | 100% | 97% |
| Class F | -0.05% | -0.55% | 0.37% | 99% | 100% |
| Class TGM | -0.03% | -0.18% | -0.10% | 100% | 98% |

## Proposal B low QP results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.02% | 100% | 97% |
| Class A2 | 0.08% | 0.09% | 0.09% | 100% | 97% |
| Class B | -0.01% | 0.00% | -0.02% | 99% | 94% |
| Class C | -0.10% | -0.08% | -0.06% | 100% | 98% |
| Class E | -0.02% | 0.01% | -0.04% | 100% | 96% |
| Overall | -0.02% | 0.00% | -0.01% | 100% | 96% |
| Class D | -0.12% | -0.11% | -0.11% | 100% | 99% |
| Class F | -0.03% | -0.05% | -0.04% | 100% | 100% |
| Class TGM | -0.08% | -0.11% | -0.08% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | -0.01% | 101% | 100% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.00% | -0.02% | 100% | 95% |
| Class C | -0.01% | -0.03% | -0.01% | 99% | 95% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | 0.04% | -0.08% | 99% | 96% |
| Class F | -0.13% | 0.02% | 0.04% | 100% | 98% |
| Class TGM | 0.22% | 0.08% | 0.11% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.01% | 0.03% | 100% | 99% |
| Class C | 0.00% | 0.01% | -0.01% | 101% | 95% |
| Class E | -0.01% | -0.01% | -0.04% | 99% | 96% |
| Overall | 0.00% | 0.01% | 0.00% | 100% | 97% |
| Class D | 0.01% | 0.05% | -0.06% | 100% | 96% |
| Class F | 0.04% | 0.05% | -0.04% | 100% | 99% |
| Class TGM | 0.01% | -0.05% | -0.02% | 99% | 96% |

## Proposal C standard QP results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.20% | 0.08% | 99% | 99% |
| Class A2 | 0.01% | -0.02% | -0.06% | 99% | 97% |
| Class B | 0.01% | 0.03% | 0.00% | 99% | 94% |
| Class C | 0.00% | 0.01% | -0.02% | 100% | 97% |
| Class E | -0.01% | -0.01% | 0.04% | 99% | 97% |
| Overall | 0.00% | -0.03% | 0.01% | 99% | 97% |
| Class D | -0.01% | 0.09% | 0.02% | 99% | 96% |
| Class F | 0.02% | -0.05% | -0.09% | 100% | 97% |
| Class TGM | -0.01% | -0.02% | -0.02% | 100% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.02% | 100% | 100% |
| Class A2 | -0.02% | 0.08% | 0.05% | 100% | 100% |
| Class B | -0.01% | 0.08% | 0.15% | 99% | 95% |
| Class C | 0.02% | 0.00% | 0.00% | 99% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.05% | 0.06% | 99% | 97% |
| Class D | 0.01% | -0.22% | -0.20% | 99% | 99% |
| Class F | -0.04% | 0.07% | 0.14% | 99% | 97% |
| Class TGM | -0.06% | -0.07% | -0.02% | 99% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.12% | 0.00% | 100% | 102% |
| Class C | 0.00% | -0.23% | -0.26% | 98% | 96% |
| Class E | -0.08% | 0.74% | -0.76% | 98% | 96% |
| Overall | -0.02% | 0.06% | -0.28% | 99% | 98% |
| Class D | -0.07% | -0.11% | -0.63% | 99% | 92% |
| Class F | -0.13% | -0.42% | 0.27% | 98% | 96% |
| Class TGM | 0.02% | -0.07% | 0.00% | 99% | 96% |

## Proposal C low QP results

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.01% | 99% | 96% |
| Class A2 | 0.05% | 0.04% | 0.04% | 99% | 97% |
| Class B | -0.01% | -0.01% | -0.02% | 99% | 94% |
| Class C | -0.11% | -0.08% | -0.07% | 99% | 97% |
| Class E | -0.03% | 0.00% | -0.03% | 100% | 95% |
| Overall | -0.02% | -0.01% | -0.02% | 99% | 96% |
| Class D | -0.12% | -0.17% | -0.16% | 100% | 98% |
| Class F | -0.04% | -0.10% | -0.03% | 99% | 98% |
| Class TGM | -0.11% | -0.13% | -0.13% | 100% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.00% | 100% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.01% | 0.02% | 99% | 95% |
| Class C | -0.01% | -0.03% | -0.02% | 99% | 94% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | 0.08% | -0.18% | 99% | 96% |
| Class F | 0.00% | -0.05% | -0.06% | 100% | 98% |
| Class TGM | 0.27% | -0.01% | -0.04% | 99% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.02% | 0.02% | 99% | 97% |
| Class C | 0.01% | -0.02% | -0.02% | 98% | 93% |
| Class E | 0.01% | -0.05% | -0.04% | 99% | 95% |
| Overall | 0.01% | -0.01% | -0.01% | 99% | 95% |
| Class D | -0.04% | 0.01% | -0.15% | 99% | 94% |
| Class F | 0.01% | -0.04% | -0.10% | 100% | 98% |
| Class TGM | -0.09% | -0.12% | -0.11% | 100% | 97% |

# Proposed specification text

The proposed changes are highlighted in yellow, and the proposed text deletion are highlighted in blue. There is no syntax change for proposal A.

7.3.9.11		Residual coding syntax for proposal B and C

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { |  |
| --- | --- |
| … |  |
| remState = 0 |  |
| for( n = firstPosMode0; n > firstPosMode1; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( abs_level_gtx_flag[ n ][ 1 ] ) { |  |
| abs_remainder[ n ] | ae(v) |
| remState = (remState + abs_remainder[ n ])>>1 |  |
| } |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] +2 * abs_remainder[ n ] |  |
| } |  |
| … |  |

##### 9.3.3.2   	Rice parameter derivation process for abs_remainder[ ] and dec_abs_level[ ]

Inputs to this process are the base level baseLevel, the colour component index cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left sample of the current picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the Rice parameter cRiceParam.

Given the array AbsLevel[ x ][ y ] for the transform block with component index cIdx and the top-left luma location ( x0, y0 ), the variable locSumAbs is derived as specified by the following pseudo code:

locSumAbs = 0
if( xC < (1 << log2TbWidth) − 1 ) {
	locSumAbs += AbsLevel[ xC + 1 ][ yC ]
	if( xC < (1 << log2TbWidth) − 2 )
		locSumAbs += AbsLevel[ xC + 2 ][ yC ]
	if( yC < (1 << log2TbHeight) − 1 )
		locSumAbs += AbsLevel[ xC + 1 ][ yC + 1 ]	(1494)
}
if( yC < (1 << log2TbHeight) − 1 ) {
	locSumAbs += AbsLevel[ xC ][ yC + 1 ]
	if( yC < (1 << log2TbHeight) − 2 )
		locSumAbs += AbsLevel[ xC ][ yC + 2 ]
} 
locSumAbs = Clip3( 0, 31, locSumAbs − baseLevel * 5 )

Given the variable locSumAbs, the Rice parameter cRiceParam is derived as specified in Table 124.

When baseLevel is equal to 0, the The variable ZeroPos[ n ] is derived as follows:

ZeroPos[ n ] = ( QState < 2 ?  1 : 2 )  <<  cRiceParam	(1495)

Table 124 – Specification of cRiceParam based on locSumAbs

| locSumAbs | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| cRiceParam | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 2 | 2 |
| locSumAbs | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| cRiceParam | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 |

#### 9.3.3.11 Binarization process for abs_remainder[ ] for proposal A

…

The rice parameter cRiceParam is derived as follows:

- If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1, the Rice parameter cRiceParam is set equal to 1.

- Otherwise, the rice parameter cRiceParam is set equal to 0. derived by invoking the rice parameter derivation process for abs_remainder[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 4, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

The variable cMax is derived from cRiceParam as:

cMax = 6  <<  cRiceParam		(1503)

…

#### 9.3.3.11 Binarization process for abs_remainder[ ] for proposal B

…

The rice parameter cRiceParam is derived as follows:

- If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1, the Rice parameter cRiceParam is set equal to 1.

- Otherwise, the rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 4, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.as follows:

cRiceParam = (remState>0) + (remState>2)

The variable cMax is derived from cRiceParam as:

cMax = 6  <<  cRiceParam		(1503)

…

#### 9.3.3.11 Binarization process for abs_remainder[ ] for proposal C

…

The rice parameter cRiceParam is derived as follows:

- If transform_skip_flag[ x0 ][ y0 ][ cIdx ] is equal to 1, the Rice parameter cRiceParam is set equal to 1.

- Otherwise, the rice parameter cRiceParam is derived by invoking the rice parameter derivation process for abs_remainder[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 4, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.as follows:

cRiceParam = (remState>0) + (remState>2) + (remState>7)

The variable cMax is derived from cRiceParam as:

cMax = 6  <<  cRiceParam		(1503)

…

9.3.3.12 	Binarization process for dec_abs_level[ ]

Input to this process is a request for a binarization of the syntax element dec_abs_level[ n ], the colour component cIdx, the luma location ( x0, y0 ) specifying the top-left sample of the current transform block relative to the top-left luma sample of the picture, the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight.

Output of this process is the binarization of the syntax element.

The rice parameter cRiceParam is derived by invoking the rice parameter derivation process for dec_abs_level[] as specified in clause 9.3.3.2 with the variable baseLevel set equal to 0, the colour component index cIdx, the luma location ( x0, y0 ), the current coefficient scan location ( xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as inputs.

The variable cMax is derived from cRiceParam as:

cMax = 6  <<  cRiceParam		(1506)

# References

- B. Bross, J. Chen, S. Liu, Y.K. Wang, “Versatile Video Coding (Draft 7),” JVET-P2001-vE, 16th Meeting: Geneva, CH, 1–11 Oct. 2019.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

- T. Nguyen, T.C. Ma, A. Nalci, “Description of Core Experiment 3 (CE3): Lossless Coding,” JVET-P2023-v3, 16th Meeting: Geneva, CH, 1–11 Oct. 2019.

- https://en.wikibooks.org/wiki/X86_Disassembly/Branches

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).