| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0318 |
| --- | --- |

| Title: | AHG9: Constraints on RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kai Zhang, Li Zhang, Ye-Kui Wang, Hongbin Liu, Jizheng Xu, Zhipin Deng 8910 University Center Lane San Diego, CA, U.S.A. 92122 | Tel: Email: | +1-858-380-8522 zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In VVC, scaling ratios of reference pictures in RPR are controlled by scaling windows. In this contribution, the following three sets of constraints are proposed to clean up the design of RPR:

- The scaling ratio determined by the scaling windows of a reference picture and the current picture cannot be too large or too small.

- It is not allowed that a reference picture is larger (or smaller) than the current picture while the scaling window of the reference picture is smaller (or larger) than the current picture.

- The memory bandwidth requirement in the interpolation process for RPR is constrained.

# Introduction

In VVC, scaling ratios of reference pictures in RPR are controlled by scaling windows. There is no constrains on dimensions of a scaling window. Therefore, the scaling ratio of a reference picture can be arbitrary which may cause problems. For example, a reference picture is larger (or smaller) than the current picture, but the scaling window of the reference picture may be smaller (or larger) than the current picture. The inconsistence of sizes may cause confusion and unnecessary computational complexity. In another example, if all pictures in a sequence have the same width and height, but the scaling window of a reference picture is with a twice width and a twice height as the scaling window of the current picture, then the interpolation process of the current picture will require a higher bandwidth than the process when RPR is not applied.

# Proposed Constraints

To solve the problems mentioned above, the following three sets of constraints on RPR are proposed:

- The scaling ratio determined by the scaling windows of a reference picture and the current picture cannot be too large or too small.

- It is not allowed that a reference picture is larger (or smaller) than the current picture while the scaling window of the reference picture is smaller (or larger) than the current picture.

- The memory bandwidth requirement in the interpolation process for RPR is constrained.

## Memory bandwidth analysis

In the following, the numbers of required integer luma samples in the interpolation filter are analyzed in an ideal model to mimic the bandwidth requirements.

Suppose that the width and height of the current picture are denoted as PW and PH, respectively; the width and height of scaling window of the current picture are denoted as SW and SH, respectively; the width and height of scaling window of the reference picture are denoted as SW’ and SH’, respectively; the width and height of the allowed maximum picture are denoted as Wmax and Hmax. For convenience, let

rw=SW'SW, Rw=WmaxPW, rh=SH'SH and Rh=HmaxPH. 						(1)

Suppose a picture is partitioned into blocks with a same size as Bw×Bh.

The total number of required luma samples in the interpolation process for the current picture is derived as

Nc=(rw×Bw+7)×(rh×Bh+7)×PW×PHBw×Bh=(rw×Bw+7)×PWBw×(rh×Bh+7)×PHBh. (2)

Similarly, the number of required luma samples in the interpolation process for an allowed maximum picture without RPR is derived as

Nmax=(Bw+7)×(Bh+7)×Wmax×HmaxBw×Bh=(Bw+7)×WmaxBw×(Bh+7)×HmaxBh.		(3)

It is required that Nc <= Nmax. As a stricter constrain, it is required that

(rw×Bw+7)×PWBw≤(Bw+7)×WmaxBw (rh×Bh+7)×PHBh ≤(Bh+7)×HmaxBh.							(4)

The horizontal part can be rearranged as

(rw×Bw+7)≤(Bw+7)×Rw,										(5)

which can be further rewritten as

rw≤Rw+7×(Rw-1)Bw.													(6)

Since Rw >=1, the boundary can be reached when Bw is equal to its maximum value: 128.

rw≤Rw+7×(Rw-1)128.													(7)

In the same way, the boundary of rh can be derived to be:

rh≤Rh+7×(Rh-1)128.													(8)

The two inequations (7) and (8) can also be rewritten in another way as:

135×Wmax×SW- 128×SW'+7×SW×PW≥0135×Hmax×SH-(128×SH'+7×SH)×PH≥0.					(9)

## Spec modifications

The spec texts for the proposed constraints are provided below, where the three sets of constraints are highlighted in red, yellow and turquoise, respectively.

For each reference picture of the current picture, the following applies:

- Let refPicWidthInLumaSamples and refPicHeightInLumaSamples be the pic_width_in_luma_samples and pic_height_in_luma_samples, respectively, of a reference picture of a current picture referring to this PPS. Let refPicOutputWidthL and refPicOutputHeightL be the PicOutputWidthL and PicOutputHeightL, respectively, of the reference picture. It is a requirement of bitstream conformance that all the following conditions are satisfied for each reference picture of the current picture:

–	The value of PicOutputWidthL * 2 shall be greater than or equal to refPicOutputWidthL.

–	The value of PicOutputHeightL * 2 shall be greater than or equal to refPicOutputHeightL.

–	The value of PicOutputWidthL shall be less than or equal to refPicOutputWidthL * 8.

–	The value of PicOutputHeightL shall be less than or equal to refPicOutputHeightL * 8.

–	The value of ( PicOutputWidthL − refPicOutputWidthL ) * ( PicWidthInLumaSamples − refPicWidthInLumaSamples ) shall be greater than or equal to 0.

–	The value of ( PicOutputHeightL − refPicOutputHeightL ) * ( PicHeightInLumaSamples − refPicHeightInLumaSamples ) shall be greater than or equal to 0.

–	The value of 135 * pic_width_max_in_luma_samples * PicOutputWidthL − (128* refPicOutputWidthL + 7 * PicOutputWidthL ) * PicWidthInLumaSamples shall be greater than or equal to 0.

–	The value of 135 * pic_height_max_in_luma_samples * PicOutputHeightL − (128* refPicOutputHeightL + 7 * PicOutputHeightL ) * PicHeightInLumaSamples shall be greater than or equal to 0.

# Conclusions

Since the proposed method constrains the memory bandwidth required by RPR, it is suggested adopting this proposal to VVC.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form.