| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0320-v1 |
| --- | --- |

| Title: | AHG13: Performance of VVC field coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Haiwei Sun Han Boon Teo Chong Soon Lim | Tel: Email: | +65-6550-5417 haiwei.sun@sg.panasonic.com hanboon.teo@sg.panasonic.com chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

According to VTM ticket #503, field coding was broken since VTM6.1. This contribution presents the work on fixing and evaluating field coding on VTM7.0. The updated source code and four RA and LDB configurations for field coding have been shared in VTM branch with merge request #1147. There is no change of VVC’s specification.

The summary of simulation results are as follows.

Compared to HM16.21 field coding, VTM7.0 field coding shows -25.65% / -38.00% / -32.09% BD-rates in RA configuration, and -21.07% / -18.33% / -18.05% BD-rates in LDB configuration for interlaced sequences.

Compared to VTM7.0 frame coding, VTM7.0 field coding shows -34.94% / -17.36% / -4.78% BD-rates in RA configuration, and -25.94% / -8.15% / -0.22% BD-rates in LDB configuration for interlaced sequences.

# Introduction

According to VTM ticket #503 [1], field coding was broken since VTM6.1. Based on analysis, below three problems were found in VTM7.0.

- Reference pictures for the second field are not initialized (Figure 1).

- PSNR and bitrate calculation for interlaced frames are not correct.

- Wrong picture height is used for skipping frames.

Figure 1 - Example showing GOP structure of field coding

Those problems have been solved in the merge request #1147 [2].

Additionally, two RA and two LDB configurations were added for supporting field coding in VTM7.0.

- VTM_encoder_randomaccess_field_GOP16.cfg

- VTM_encoder_randomaccess_field_GOP32.cfg

- VTM_encoder_lowdelay_field_GOP8.cfg

- VTM_encoder_lowdelay_field_GOP16.cfg

# Test sequences

A total of 5 sequences from Sveriges Television AB (SVT) [3] are used for this simulation. The video information of the sequences used is listed in Table 1. The thumbnails and brief descriptions of the sequences are listed in Table 2.

Table 1 - Video information of the sequences

| Resolution | 1920 x 1080i |
| --- | --- |
| Chroma format | YCbCr 4:2:0 |
| Frame number | 250 (or 500 fields) |
| Frame rate | 25 (or 50i) |
| Bit depth | 8-bit |

Table 2 - Thumbnails and brief description of the sequences

| Thumbnail | Brief description |
| --- | --- |
| CrowdRun | Start of a crowd running. (camera pan) |
| DucksTakeOff | Ducks are on water and take off. (fixed camera) |
| InToTree | Helicopter flight approaching an old castle building. (aerial filming and zoom) |
| OldTownCross | Sky view of Stockholm. (aerial filming) |
| ParkJoy | People are running in front of trees during a left to right camera movement. (camera dolly) |

# Test settings

Three sets of comparisons are conducted. They are VTM7.0 field coding compared to HM16.21 field coding (Comparison 1), VTM7.0 field coding compared to VTM7.0 frame coding (Comparison 2), and HM16.21 field coding compared to HM16.21 frame coding (Comparison 3). Comparison 3 is used as a reference. The GOP size and intra period for these three sets of comparisons are shown in Table 3, Table 4, and Table 5.

Table 3 - GOP size and intra period for Comparison 1

| Comparison 1 | HM16.21 field coding |  | VTM7.0 field coding |  |
| --- | --- | --- | --- | --- |
|  | GOP size | Intra Period | GOP size | Intra Period |
| RA | 16 fields | 32 fields | 16 fields | 32 fields |
| LDB | 8 fields | -1 | 8 fields | -1 |

Table 4 - GOP size and intra period for Comparison 2

| Comparison 2 | VTM7.0 frame coding |  | VTM7.0 field coding |  |
| --- | --- | --- | --- | --- |
|  | GOP size | Intra Period | GOP size | Intra Period |
| RA | 16 frames | 32 frames | 32 fields | 64 fields |
| LDB | 8 frames | -1 | 16 fields | -1 |

Table 5 - GOP size and intra period for Comparison 3

| Comparison 3 | HM16.21 frame coding |  | HM16.21 field coding |  |
| --- | --- | --- | --- | --- |
|  | GOP size | Intra Period | GOP size | Intra Period |
| RA | 8 frames | 16 frames | 16 fields | 32 fields |
| LDB | 4 frames | -1 | 8 fields | -1 |

For fair comparison, same GOP structure is used between VTM7.0 and HM16.21 in Comparison 1. Simulations were conducted at QP 22, 27, 32 and 37 in RA and LDB configurations. The HM configuration files used were attached to this contribution.

# Test results

The test results of the three sets of comparisons are shown in Table 6, Table 7, and Table 8. PSNRs of (interlaced) frames are used for calculating BR-rates. The encoding and decoding timings are not reliable as the simulations were carried out on different PC clusters.

Table 6 - Results of Comparison 1: HM field vs VTM field

| Comparison 1 |  | Y | U | V |
| --- | --- | --- | --- | --- |
| RA | CrowdRun | -21.02% | -23.38% | -19.41% |
|  | DucksTakeOff | -23.94% | -21.23% | -44.15% |
|  | InToTree | -29.26% | -77.80% | -42.67% |
|  | OldTownCross | -33.35% | -52.92% | -37.91% |
|  | ParkJoy | -20.69% | -14.66% | -16.32% |
|  | Overall | -25.65% | -38.00% | -32.09% |
| LDB | CrowdRun | -17.29% | -9.43% | -7.38% |
|  | DucksTakeOff | -20.55% | -12.97% | -30.14% |
|  | InToTree | -20.99% | -25.42% | -20.21% |
|  | OldTownCross | -30.02% | -37.90% | -31.26% |
|  | ParkJoy | -16.50% | -5.93% | -1.25% |
|  | Overall | -21.07% | -18.33% | -18.05% |

Table 7 - Results of Comparison 2: VTM frame vs VTM field

| Comparison 2 |  | Y | U | V |
| --- | --- | --- | --- | --- |
| RA | CrowdRun | -39.55% | -34.71% | -34.88% |
|  | DucksTakeOff | -3.24% | 43.96% | 81.88% |
|  | InToTree | -51.04% | -55.86% | -36.51% |
|  | OldTownCross | -44.28% | -8.98% | -9.65% |
|  | ParkJoy | -36.60% | -31.20% | -24.74% |
|  | Overall | -34.94% | -17.36% | -4.78% |
| LDB | CrowdRun | -31.04% | -23.58% | -24.37% |
|  | DucksTakeOff | 0.56% | 20.96% | 61.67% |
|  | InToTree | -39.38% | -18.68% | -19.25% |
|  | OldTownCross | -32.41% | 3.06% | -1.68% |
|  | ParkJoy | -27.41% | -22.50% | -17.49% |
|  | Overall | -25.94% | -8.15% | -0.22% |

Table 8 - Results of Comparison 3: HM frame vs HM field

| Comparison 3 |  | Y | U | V |
| --- | --- | --- | --- | --- |
| RA | CrowdRun | -38.12% | -33.21% | -33.96% |
|  | DucksTakeOff | -0.90% | 18.83% | 51.14% |
|  | InToTree | -47.69% | -33.30% | -33.13% |
|  | OldTownCross | -41.75% | -13.28% | -13.25% |
|  | ParkJoy | -33.43% | -26.42% | -24.37% |
|  | Overall | -32.38% | -17.48% | -10.71% |
| LDB | CrowdRun | -35.24% | -30.74% | -31.44% |
|  | DucksTakeOff | 1.00% | 19.76% | 49.70% |
|  | InToTree | -39.65% | -21.53% | -18.46% |
|  | OldTownCross | -32.29% | 8.61% | 9.58% |
|  | ParkJoy | -29.66% | -25.06% | -20.92% |
|  | Overall | -27.17% | -9.79% | -2.31% |

# Analysis of results

The simulation results show that CrowdRun, InToTree, OldTownCross, and ParkJoy have high camera motion and action and favour field coding. DucksTakeOff has no camera motion and favours frame coding.

# Reference

[1] https://jvet.hhi.fraunhofer.de/trac/vvc/ticket/503

[2] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/merge_requests/1147

[3] C. Rosewarne, A. Tourapis, G. Barroux, M. Naccari, “Draft verification test plan for interlaced video and format range extensions”, JCTVC-S1003, 19th Meeting: Strasbourg, FR, 17 Oct. –24 Oct. 2014.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).