| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0348 |
| --- | --- |

| Title: | CE4-Related: Displacement Restriction on Geometric Inter Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Yu-Chiao Yang Chih-Yu Teng | Tel: Email: | yuciaoyang@fginnov.com zachary.deng@fginnov.com |
| Source: | Foxconn |  |  |

_____________________________

- Abstract

In geometric inter prediction (GEO) proposed by JVET-P0884/JVET-P0885 [1], the blending weights and motion field masks for combine two predictions are cropped from pre-defined mask sets for each partition in the CE4 common base [2] software. In addition, the memory storage for blending weight and motion field mask is up to 176.86 KB. Therefore, this contribution proposes a method only allowing the first 3 of the 4 displacements when the block width or block height is equal to the maximum GEO block size, otherwise, 4 displacements are all allowed. As a result, the memory storage of blending weight and motion field mask can be reduced around 26.53% in this contribution implemented on CE4 common base, CE4-2.1, CE4-2.2. Compared to CE4 common base, the overall results are reported as follows:

Test 1 (Proposed + Common base): 	RA 0.00%/0.01%/-0.02% with 100%/100% coding time.
	LB 0.04%/0.02%/0.19% with 100%/100% coding time.

Test 2 (Proposed + CE4-2.1):	RA -0.02%/0.00%/0.01% with 100%/100% coding time
	LB 0.08%/0.02%/0.18% with 100%/100% coding time

Test 3 (Proposed + CE4-2.2):	RA 0.02%/0.03%/-0.01% with 100%/100% coding time
	LB 0.28%/0.33%/0.15% with 99%/100% coding time

Introduction

GEO may partition a rectangular block into two sub-blocks each having a triangle shape or a non-triangle shape. A partition index is signalled to indicate one of 82 GEO partitions, wherein the GEO partitions are generated based on 24 angles and 4 displacements in the CE4 common base. In addition, for each partition, the blending weights and motion field masks for combine two predictions are cropped from pre-defined mask sets in the CE4 common base software. Furthermore, the memory storage for blending weight and motion field mask is up to (24.5+0.77) KB of an angle.

To reduce the memory storage and complexity, one pre-defined mask can be removed from CE4-1, thus the number of pre-defined mask can be reduced from 7 to 6, and different block size restrictions are tested in CE4-2.1 and CE4-2.1. Compared to the common base, the memory storage requirements and the percentage of memory that can be reduced are listed in Table 1.

Table 1. Summary of memory requirements

| Test # | Blending Weights | Motion Field Mask | Overall | Reduced (%) |
| --- | --- | --- | --- | --- |
| Common base | 171.5 KB | 5.36 KB | 176.86 KB | - |
| CE4-1 | 147 KB | 4.59 KB | 151.59 KB | 14.29% |
| CE4-2.1 | 42.86 KB | 1.34 KB | 44.21 KB | 75.00% |
| CE4-2.2 | 10.72 KB | 0.33 KB | 11.05 KB | 93.75% |

The memory requirement of blending weights and motion field mask are calculated as follows:

BlendingSize(BS)=S×Dmax_indexD-1≫Log2D≪1≪1+S

BlendingStorage(Bits)=BS×BS×4×AngMaskNum

MotionFieldStorageBits=BlendingStorage≫5

Variable S is maximum block size supported in GEO

Variable Dmax_index is the maximum displacement index in GEO

Variable D is number of displacements supported in GEO

Variable AngMaskNum is pre-defined mask number in GEO

Proposed

To reduce the memory storage of blending weight and motion field mask, this contribution proposes to only allow the first 3 of the 4 displacements when the block width or block height is equal to the maximum GEO block size, otherwise, 4 displacements are all allowed. Consequently, the size of pre-defined mask can be reduced, and the memory requirements are listed in Table 2.

Table 2. Summary of memory requirements on CE4 common base, CE4-2.1 and CE4-2.2 with proposed method

| Test # | Blending Weights | Motion Field Mask | Overall | Reduced (%) |
| --- | --- | --- | --- | --- |
| Common base | 171.5 KB | 5.36 KB | 176.86 KB | - |
| Common base + Proposed | 126 KB | 3.94 KB | 129.94 KB | 26.53% |
| CE4-2.1+ Proposed | 31.5 KB | 0.98 KB | 32.48 KB | 81.63% |
| CE4-2.2+ Proposed | 7.88 KB | 0.25 KB | 8.12 KB | 95.41% |

Compared with original methods of CE4 common base, CE4-2.1 and CE4-2.2, the memory storage can be reduced around 26.5% and shows in Table 3. Moreover, the partition number is reduced from 82 to 58 when the block width or block height is equal to maximum GEO block size in the proposed method. Therefore, another partition index is signalled to indicate one of 58 partitions.

Table 3. Comparison of memory requirements with/without proposed method on CE4 common base, CE4-2.1 and CE4-2.2

| Test # | Without Proposed | With Proposed | Reduced (%) |
| --- | --- | --- | --- |
| Common base | 176.86 KB | 129.94 KB | 26.53% |
| CE4-2.1 | 44.21 KB | 32.48 KB | 26.53% |
| CE4-2.2 | 11.05 KB | 8.12 KB | 26.53% |

Experimental results

The test condition described in JVET-N1010 [3] is used in the simulation. The proposed method is applied to CE4 Common base, CE4-2.1 and CE4-2.2. Table 4 and Table 7 show the simulation results of Test 1. Table 5 and Table 8 show the simulation results of Test 2. Table 6 and Table 9 show the simulation results of Test 3. Table 4, Table 5 and Table 6 are compared with VTM-7.0. Table 7, Table 8 and Table 9 are compared with CE4 common base.

Test 1: Proposed method on Common base.

Test 2: Proposed method on CE4-2.1.

Test 3: Proposed method on CE4-2.2

Table 4. Summary of experimental results (Test 1 over VTM-7.0)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.13% | -0.25% | 103% | 101% |
| Class A2 | -0.20% | -0.15% | -0.06% | 103% | 101% |
| Class B | -0.15% | -0.09% | -0.18% | 103% | 100% |
| Class C | -0.59% | -0.96% | -0.89% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.27% | -0.34% | -0.36% | 103% | 100% |
| Class D | -0.43% | -0.67% | -0.62% | 103% | 100% |
| Class F | -0.30% | -0.37% | -0.33% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34% | -0.57% | -0.38% | 104% | 99% |
| Class C | -0.78% | -1.19% | -0.92% | 104% | 98% |
| Class E | -1.03% | -0.10% | -0.45% | 102% | 99% |
| Overall | -0.66% | -0.66% | -0.58% | 103% | 99% |
| Class D | -0.67% | -0.58% | -1.09% | 103% | 99% |
| Class F | -0.53% | -0.90% | -0.18% | 102% | 99% |

Table 5. Summary of experimental results (Test 2 over VTM-7.0)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.26% | -0.08% | 103% | 101% |
| Class A2 | -0.23% | -0.10% | -0.10% | 103% | 101% |
| Class B | -0.16% | -0.09% | -0.17% | 103% | 100% |
| Class C | -0.64% | -0.93% | -0.85% | 104% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.29% | -0.35% | -0.32% | 103% | 100% |
| Class D | -0.45% | -0.74% | -0.70% | 104% | 100% |
| Class F | -0.31% | -0.41% | -0.31% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | -0.41% | -0.41% | 104% | 99% |
| Class C | -0.80% | -1.26% | -0.89% | 105% | 97% |
| Class E | -0.90% | -0.24% | -0.46% | 101% | 99% |
| Overall | -0.63% | -0.65% | -0.59% | 104% | 99% |
| Class D | -0.73% | -0.80% | -1.03% | 104% | 98% |
| Class F | -0.58% | -1.24% | -0.07% | 102% | 99% |

Table 6. Summary of experimental results (Test 3 over VTM-7.0)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.10% | -0.18% | 101% | 101% |
| Class A2 | -0.17% | -0.13% | -0.01% | 102% | 101% |
| Class B | -0.16% | -0.12% | -0.16% | 102% | 100% |
| Class C | -0.57% | -0.89% | -0.97% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | -0.32% | -0.35% | 102% | 100% |
| Class D | -0.41% | -0.73% | -0.67% | 104% | 100% |
| Class F | -0.26% | -0.37% | -0.36% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.19% | -0.38% | -0.45% | 103% | 99% |
| Class C | -0.70% | -0.94% | -0.86% | 105% | 97% |
| Class E | -0.45% | 0.49% | -0.57% | 100% | 99% |
| Overall | -0.42% | -0.35% | -0.62% | 103% | 99% |
| Class D | -0.62% | -0.78% | -0.58% | 105% | 98% |
| Class F | -0.39% | -0.92% | -0.46% | 101% | 99% |

Table 7. Summary of experimental results (Test 1 over CE4 common base)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | -0.07% | 100% | 100% |
| Class A2 | 0.02% | 0.03% | -0.04% | 100% | 100% |
| Class B | -0.01% | 0.01% | -0.01% | 100% | 100% |
| Class C | 0.00% | 0.03% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.02% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.01% | 0.01% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE4 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.06% | 0.21% | 100% | 100% |
| Class C | 0.01% | 0.05% | -0.03% | 100% | 100% |
| Class E | 0.14% | 0.09% | 0.43% | 100% | 100% |
| Overall | 0.04% | 0.02% | 0.19% | 100% | 100% |
| Class D | 0.01% | -0.13% | -0.04% | 100% | 100% |
| Class F | -0.02% | 0.01% | 0.12% | 100% | 100% |

Table 8. Summary of experimental results (Test 2 over CE4 common base)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.17% | 0.10% | 100% | 100% |
| Class A2 | -0.01% | 0.09% | -0.09% | 100% | 100% |
| Class B | -0.01% | 0.01% | 0.00% | 100% | 100% |
| Class C | -0.05% | 0.05% | 0.04% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | 0.01% | 100% | 100% |
| Class D | -0.03% | -0.08% | -0.08% | 101% | 100% |
| Class F | 0.01% | -0.03% | 0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE4 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.09% | 0.18% | 100% | 100% |
| Class C | -0.01% | -0.01% | -0.01% | 101% | 100% |
| Class E | 0.27% | -0.05% | 0.43% | 99% | 100% |
| Overall | 0.08% | 0.02% | 0.18% | 100% | 100% |
| Class D | -0.05% | -0.35% | 0.01% | 101% | 100% |
| Class F | -0.07% | -0.33% | 0.23% | 100% | 100% |

Table 9. Summary of experimental results (Test 3 over CE4 common base)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.01% | 0.00% | 99% | 100% |
| Class A2 | 0.04% | 0.05% | 0.01% | 99% | 100% |
| Class B | -0.02% | -0.02% | 0.02% | 99% | 100% |
| Class C | 0.02% | 0.09% | -0.08% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | -0.01% | 100% | 100% |
| Class D | 0.02% | -0.06% | -0.05% | 101% | 100% |
| Class F | 0.05% | 0.02% | -0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over CE4 common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.17% | 0.13% | 0.15% | 98% | 100% |
| Class C | 0.09% | 0.30% | 0.03% | 100% | 100% |
| Class E | 0.73% | 0.68% | 0.32% | 98% | 101% |
| Overall | 0.28% | 0.33% | 0.15% | 99% | 100% |
| Class D | 0.06% | -0.34% | 0.47% | 101% | 100% |
| Class F | 0.12% | 0.00% | -0.15% | 99% | 100% |

Conclusion

This contribution proposes a method only allowing the first 3 of the 4 displacements when the block width or block height is equal to the maximum GEO block size, otherwise, 4 displacements are all allowed. As a result, the memory storage of blending weight and motion field mask can be reduced around 26.53%. It is suggested to adopt this proposal into the next VVC working draft if GEO has been adopted.

References

H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Blaser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304)”, JVET-P0884, Geneva, CH, October 2019.

C.-C. Chen, R.-L. Liao, X. Xiu, H. Yang, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning”, JVET-P2024 , Geneva, CH, October 2019.

F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March 2019.

Patent rights declaration(s)

Foxconn may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Proposed GEO WD changes

7.3.9.7	Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( !ciip_flag[ x0 ][ y0 ] && MaxNumGeoMergeCand > 1 ) { |  |
| if(cbWidth == 128 ¦¦ cbHeight ==128) |  |
| merge_geo_non_last_dist_partition_idx[ x0 ][ y0 ] | ae(v) |
| else |  |
| merge_geo _partition_idx[ x0 ][ y0 ] | ae(v) |
| merge_geo_idx0[ x0 ][ y0 ] | ae(v) |
| if( MaxNumGeoMergeCand > 2 ) |  |
| merge_geo_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

merge_geo_non_last_dist_partition_idx [ x0 ][ y0 ] specifies the geometric splitting direction of the merge geometric mode. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When merge_geo_non_last_dist_partition_idx[ x0 ][ y0 ] is not present, it is inferred to be equal to 0.

8.5.7	Decoding process for geo inter blocks

8.5.7.1	General

- If cbWidth or cbHeight is equal to 128, the merge_geo_partition_idx[ xCb ][ yCb] is set according to the value of merge_geo_non_last_dist_partition_idx[ xCb ][ yCb ] as specified in Table 36.

- The partition ange and distance of merge geo mode variable angleIdx and distanceIdx are set according to the value of merge_geo_parition_idx[ xCb ][ yCb ] as specified in Table 37.

Table 36 – Specification of the merge_geo_partition_idx based on the merge_geo_non_last_dist_partition_idx.

| merge_geo_non_last_dist_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| merge_geo_partition_idx | 0 | 1 | 3 | 4 | 5 | 7 | 8 | 9 | 11 | 12 | 13 | 15 | 16 | 17 | 19 | 20 | 21 |
| merge_geo_non_last_dist_partition_idx | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| merge_geo_partition_idx | 23 | 24 | 26 | 27 | 28 | 30 | 31 | 32 | 34 | 35 | 36 | 38 | 39 | 40 | 42 | 43 | 44 |
| merge_geo_non_last_dist_partition_idx | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 |
| merge_geo_partition_idx | 46 | 47 | 49 | 50 | 52 | 53 | 55 | 56 | 58 | 59 | 61 | 62 | 64 | 65 | 67 | 68 | 70 |
| merge_geo_non_last_dist_partition_idx | 51 | 52 | 53 | 54 | 55 | 56 | 57 |
| merge_geo_partition_idx | 71 | 73 | 74 | 76 | 77 | 79 | 80 |

Table 37 – Specification of the angleIdx and distanceIdx values based on the merge_geo_partition_idx value.

| merge_geo_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| angleIdx | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| distanceIdx | 1 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 |
| merge_geo_partition_idx | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| angleIdx | 4 | 6 | 6 | 8 | 8 | 8 | 8 | 9 | 9 | 9 | 9 | 10 | 10 | 10 | 10 | 11 | 11 |
| distanceIdx | 3 | 1 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 |
| merge_geo_partition_idx | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 |
| angleIdx | 11 | 11 | 12 | 12 | 13 | 13 | 13 | 14 | 14 | 14 | 15 | 15 | 15 | 16 | 16 | 16 | 18 |
| distanceIdx | 2 | 3 | 1 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 |
| merge_geo_partition_idx | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| angleIdx | 18 | 20 | 20 | 20 | 21 | 21 | 21 | 22 | 22 | 22 | 23 | 23 | 23 |
| distanceIdx | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 |

9.3.3	Binarization process

9.3.3.1	General

Table 123 – Syntax elements and associated binarizations

| Syntax structure | Syntax element | Binarization |  |
| --- | --- | --- | --- |
|  |  | Process | Input parameters |
| merge_data( ) | merge_geo_partition_idx[ ][ ] | TB | cMax = 82 |
|  | merge_geo_non_last_dist_partition_idx[ ][ ] | TB | cMax = 58 |
|  | merge_geo_idx0[ ][ ] | TR | cMax = MaxNumGeoMergeCand − 1, cRiceParam = 0 |
|  | merge_geo_idx1[ ][ ] | TR | cMax = MaxNumGeoMergeCand − 2, cRiceParam = 0 |

9.3.4.2	Derivation process for ctxTable, ctxIdx and bypassFlag

9.3.4.2.1	General

Table 128 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| merge_geo_partition_idx[ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| merge_geo_non_last_dist_partition_idx[ ][ ] | bypass | bypass | bypass | bypass | bypass | bypass |
| merge_geo_idx0[ ][ ] | 0 | bypass | bypass | bypass | bypass | na |
| merge_geo_idx1[ ][ ] | 0 | bypass | bypass | bypass | na | na |