| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0364-v2 |
| --- | --- |

| Title: | CE4.4-related: Combination of GEO or TPM and Weighted-Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P.Bordes, T. Poirier, F. Le Léannec | Email: | philippe.bordes@interdigital.com tangi.poirier@interdigital.com fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In the current VTM-7.0 and CE4 software, the weighted prediction (WP) is not enabled with TPM and GEO. In the last JVET meeting (Geneva, Oct-2019), it has been proposed to either cascade WP and TPM or to re-scale the blending weights with WP parameters prior the TPM blending stage. However, these methods have a latency issue since they add an additional stage and/or need an additional multiplier for the blending.

In the CE4.4, it is studied turning off the blending filter for GEO in case of screen content.

In this proposal, when the weighted prediction (WP) is enabled for at least one GEO partition, it is proposed to apply the regular WP weights on the two GEO prediction partitions and to turn off the blending filter. It is stated that the proposed method avoids cascading two weighting stages (WP weighting and GEO blending filters).

It is reported that:

- compared to CE4.4 with WP enabled (CTC fade sequences), the proposed method provides BD-rate gains of 0.07%(Y),-0.08%(U),-0.01%(V) / 0.06%(Y),-0.48%(U),0.04%(V) in RA / LDB configurations with fade-to-black sequences, and -0.08%(Y),-0.07%(U),-0.26%(V) / -0.18%(Y),-0.13%(U),-0.47%(V) in RA / LDB configurations with fade-to-white sequences.

- compared to CE4.4 with WP enabled (TGM fade sequences), the proposed method provides BD-rate gains of -1.88%(Y),-1.48%(U),-1.37%(V) / -2.25%(Y),-1.79%(U),-1.62%(V) in RA / LDB configurations with fade-to-black sequences, and -1.96%(Y),-1.46%(U),-1.42%(V) / -2.38%(Y),-1.88%(U),-1.77%(V) in RA / LDB configurations with fade-to-white sequences.

The encoder and decoder complexity are unchanged compared to anchors.

# Introduction / Problem Statement

In the current VTM-7.0 and CE4 software, the weighted prediction (WP) is not enabled with TPM and GEO. In the last JVET meeting (Geneva, Oct-2019), it has been proposed to either cascade WP and TPM [1] or to re-scale the blending weights with WP parameters prior the TPM blending stage [2].

However, these methods have a latency issue since they add an additional stage and/or need an additional multiplier for the blending. Additionally, some participants commented that we should not add this extra complexity for a benefit that is only found on special content such as fading content.

# Proposal

In the CE4.4, it is studied turning off the blending filter for GEO in case of screen content.

One proposes to extend this method when the weighted prediction is enabled for at least one of the reference refIdxA, or refIdxB (WP enabled at slice level and at least one WP weights associated with refIdxA or refIdxB is not default). In this case, it is proposed to apply the regular explicit uni-directional WP weights on each GEO prediction partitions and to turn off the blending filter.

In case the weighted prediction is disabled for both the reference refIdxA, and refIdxB (WP disabled at slice level or WP weights associated with refIdxA and refIdxB are defaults) then the regular GEO with blending filter applies (Figure 1).

Figure 1 – The regular GEO process with blending filter (left) or GEO process without blending filter (right) is selected depending on whether WP is disabled for both reference partitions or not.

# Experimental results

Since weighted prediction is not used on CTC, the experiments by using weighted prediction and fade-sequences have been conducted in this contribution.

The fade sequences are generated with tool provided in [3]. In our experiments, a linear fade is applied to the about first 2 seconds of the regular sequence set, and the experiments are conducted with them. A linear fade-out (sequence to black or white) is applied in the 1st half and a linear fade-in (respectively black or white to sequence) in the 2nd half. The fade strength is limited to the range [0.25, 0.75] instead of [0, 1] to avoid extreme PSNR numbers at the frame level that cause abnormal rate-distortion graphs.

The anchors are VTM-7.0 (CE4.4) with WP enabled (incl. ibc bug-fix) and blendingCtrl=0 (blending filter turned on). The encoder method-0 is used for estimating WP parameters.

Table 1: summary of the proposed GEO mode combined with WP (fade-to-black and fade-to-white sequences).

|  | Proposal difference with VTM-7.0 (Fade to black) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.37% | -0.10% | 0.11% | 100% | 100% |  |  |  |  |  |
| Class A2 | -0.05% | 0.09% | -0.13% | 99% | 99% |  |  |  |  |  |
| Class B | 0.00% | -0.29% | 0.01% | 99% | 98% | 0.17% | -0.29% | -0.09% | 101% | 101% |
| Class C | 0.04% | 0.05% | -0.04% | 100% | 101% | -0.17% | -0.52% | 0.39% | 99% | 100% |
| Class E |  |  |  |  |  | 0.18% | -0.74% | -0.22% | 100% | 99% |
| Overall | 0.07% | -0.08% | -0.01% | 99% | 99% | 0.06% | -0.48% | 0.04% | 100% | 100% |
| Class D | -0.42% | -1.08% | -0.51% | 97% | 98% | -0.44% | -0.77% | 0.76% | 98% | 100% |
| Class F | -0.15% | -0.11% | -0.18% | 99% | 98% | -0.24% | -0.16% | -0.31% | 99% | 98% |
| TGM | -1.88% | -1.48% | -1.37% | 99% | 103% | -2.25% | -1.79% | -1.62% | 98% | 100% |

|  | Proposal difference with VTM-7.0 (Fade to white) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | -0.15% | 99% | 101% |  |  |  |  |  |
| Class A2 | -0.13% | -0.04% | -0.14% | 100% | 98% |  |  |  |  |  |
| Class B | -0.10% | 0.13% | -0.24% | 101% | 100% | -0.03% | 0.16% | 0.25% | 101% | 102% |
| Class C | -0.06% | -0.38% | -0.45% | 99% | 100% | -0.07% | -0.08% | -0.54% | 99% | 102% |
| Class E |  |  |  |  |  | -0.59% | -0.66% | -1.56% | 100% | 98% |
| Overall | -0.08% | -0.07% | -0.26% | 100% | 100% | -0.18% | -0.13% | -0.47% | 100% | 101% |
| Class D | -0.32% | -0.52% | -0.54% | 97% | 97% | -0.32% | -0.38% | -1.74% | 101% | 101% |
| Class F | -0.24% | 0.00% | -0.07% | 99% | 99% | -0.56% | -0.23% | -0.60% | 97% | 97% |
| TGM | -1.96% | -1.46% | -1.42% | 98% | 98% | -2.38% | -1.88% | -1.77% | 98% | 99% |

# Syntax modifications

The modification of the CE4.4 syntax for TPM is highlighted by yellow:

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesLA and predSamplesLB,

- the prediction list flags predListFlagA and predListFlagB,

- the reference indices refIdxA and refIdxB,

- a variable triangleDir specifying the partition direction,

- a variable cIdx specifying colour component index.

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

The variable nCbWY, nCbHY and nCbR is derived as follows:

nCbWY = ( cIdx = = 0 )  ?  nCbW  :  nCbW * SubWidthC	(1027)

nCbHY = ( cIdx = = 0 )  ?  nCbH  :  nCbH * SubHeightC	(1028)

nCbR = ( nCbWY > nCbHY )  ?  ( nCbWY / nCbHY )  :  ( nCbHY / nCbWY )	(1029)

Variables shift1 and offset1 are derived as follows:

–	The variable shift1 is set equal to Max( 5, 17 − BitDepth).

–	The variable offset1 is set equal to 1  <<  ( shift1 − 1 ).

The variable blending_off_flag is derived as follows:

- blending_off_flag is set equal to 1.

- If slice_ blending_off_flag is equal to 1, then blending_off_flag is set equal to 1,

- Otherwise, if cIdx is equal to 0 and (predListFlagX is equal to 0 and luma_weight_l0_flag[ refIdxX ] is equal to 1) or (predListFlagX is equal to 1 and luma_weight_l1_flag[ refIdxX ] is equal to 1), then blending_off_flag is set equal to 0,

- Otherwise, if cIdx is not equal to 0 and (predListFlagX is equal to 0 and chroma_weight_l0_flag[ refIdxX ] is equal to 1) or (predListFlagX is equal to 1 and chroma_weight_l1_flag[ refIdxX ] is equal to 1), then blending_off_flag is set equal to 0,

Depending on the values of triangleDir and cIdx, the prediction samples pbSamples[ x ][ y ] with x = 0..nCbW − 1 and y = 0..nCbH − 1 are derived as follows:

–	The variable wIdx is derived as follows:

- If cIdx is equal to 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( x / nCbR − y ) + 4 ) )	(8-842)
	: ( Clip3( 0, 8, ( x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is equal to 0 and triangleDir is equal to 1, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 8, ( nCbH − 1 − x / nCbR − y ) + 4 ) ) 	(8-843)
		( Clip3( 0, 8, ( nCbW − 1 − x − y / nCbR ) + 4 ) )

- Otherwise, if cIdx is greater than 0 and triangleDir is equal to 0, the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( x / nCbR − y ) + 2 ) )	(8-844)
	: ( Clip3( 0, 4, ( x − y / nCbR ) + 2 ) )

- Otherwise (if cIdx is greater than 0 and triangleDir is equal to 1), the following applies:

wIdx = ( nCbW > nCbH ) ? ( Clip3( 0, 4, ( nCbH − 1 − x / nCbR − y ) + 2 ) ) 	(8-845)
		( Clip3( 0, 4, ( nCbW − 1 − x − y / nCbR ) + 2 ) )

–	if slice_blending_off_flag is equal to 0, the variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows:

wValue = ( cIdx = = 0 )  ?  Clip3( 0, 8, wIdx )  :  Clip3( 0, 8, wIdx * 2 )	(8-846)

–	otherwise (slice_blending_off_flag is equal to 1), the variable wValue specifying the weight of the prediction sample is derived using wIdx and cIdx as follows:

if y < (nCbH >>1) :

wValue = ( cIdx = = 0 )  ? (wIdx <= 4 ? 0 : 8) : (wIdx * 2 <= 4 ? 0 : 8)

otherwise

wValue = ( cIdx = = 0 )  ? (wIdx < 4 ? 0 : 8) : (wIdx * 2 < 4 ? 0 : 8)

–	The prediction sample values are derived as follows:

pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  BitDepth ) − 1, ( predSamplesLA[ x ][ y ] * wValue + 	(1032)
		predSamplesLB[ x ][ y ] * ( 8 − wValue ) + offset1 ) >> shift1 )

# Subjective tests

One provides in Figure 2 one example extracted from the same frame in anchor and proposal (RA, FadeWhite, D_RaceHorses, poc=22, qp=37). It illustrates the coding artefacts that may be caused by using GEO with reference samples which are not compensated with WP weights (CE4.4 anchor, left) and how this is corrected with proposal (right).

Figure 2 – On the left, GEO mode applied without WP weights (CE4.4, anchor) causes luminance artefacts, while on the right the proposed method has no luminance artefact.

# Conclusion

It is proposed to enable the weighted prediction with GEO. It is stated that the proposed method avoids cascading two weighting stages (WP weighting and GEO blending filters). The encoder and decoder complexity are unchanged compared to anchors.

# References

- T.Chujoh, E.Sasaki, T.Ikai, “Non-CE4: On a simplification for triangle merge mode,” 16th Meeting: Geneva, CH, 1–11 October 2019.

- A.Filippov, V.Rufitskiy, “Non-CE4: On TPM merge mode in the presence of weighted prediction,” 16th Meeting: Geneva, CH, 1–11 October 2019.

- P. Bordes, E. François and F. L. Léannec, AHG13 - Weighted Prediction vs Generalized Bi-prediction with Fade sequences, Joint Video Exploration Team Document, JVET-L0201, Macao, Oct. 2018.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).