| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0368-v1 |
| --- | --- |

| Title: | On context model for mvp_flag and ref_idx |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen, Haitao Yang | Tel: Email: | chenhuanbang@huawei.com haitao.yang@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

- Abstract

This contribution reports a mismatch between VTM-7.0 and VVC working draft 7 related to the coding of mvp_l0_flag, mvp_l1_flag, ref_idx_l0, and ref_idx_l1. In VTM-7.0, mvp_l0_flag and mvp_l1_flag are coded with a single context. While in WD7, these two syntaxes are coded with separate contexts. Same issue is detected for ref_idx_l0 and ref_idx_l1. This contribution investigated two methods for solving this issue: 1) software is aligned to spec and 2) spec is aligned to software.

Simulation results for method 1 reportedly show that the luma BD-rate 0.00% and 0.XX% in RA and LB cases, respectively, when compared to VTM-7.0.

Introduction

A mismatch between VTM-7.0 and VVC WD 7 is founded. In the VTM-7.0, mvp_l0_flag and mvp_l1_flag are coded with a single context. But in WD7, these two syntaxes are coded with separate contexts. Same issue is detected for ref_idx_l0 and ref_idx_l1. The current text shows as follows:

| Table 48 – Association of ctxIdx and syntax elements for each initializationType in the initialization process |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax structure | Syntax element | ctxTable | initType |  |  |  |
|  |  |  | 0 | 1 | 2 |  |
| coding_unit( ) | ref_idx_l0[ ][ ] | Table 81 |  | 0..1 | 2..3 |  |
|  | mvp_l0_flag[ ][ ] | Table 82 | 0..1 | 2..3 | 4..5 |  |
|  | ref_idx_l1[ ][ ] | Table 83 |  | 0..1 | 2..3 |  |
|  | mvp_l1_flag[ ][ ] | Table 84 |  | 0..1 | 2..3 |  |

Proposed method

SW is aligned to text

To be aligned with text, 1 contexts is added for mvp_l1_flag and 2 contexts are added for ref_idx_l1.

Text is aligned to SW

In this case, there is no change of the coding performance. The proposed specification is as follows:

| Table 48 – Association of ctxIdx and syntax elements for each initializationType in the initialization process |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax structure | Syntax element | ctxTable | initType |  |  |  |
|  |  |  | 0 | 1 | 2 |  |
| coding_unit( ) | ref_idx_l0[ ][ ], ref_idx_l1[ ][ ] | Table 81 |  | 0..1 | 2..3 |  |
|  | mvp_l0_flag[ ][ ], mvp_l1_flag[ ][ ] | Table 82 | 0..1 | 2..3 | 4..5 |  |
|  | ref_idx_l1[ ][ ] | Table 83 |  | 0..1 | 2..3 |  |
|  | mvp_l1_flag[ ][ ] | Table 84 |  | 0..1 | 2..3 |  |

Table 81 – Specification of initValue and shiftIdx for ctxIdx of ref_idx_l0 and ref_idx_l1

| Initialization variable | ctxIdx of ref_idx_l0 and ref_idx_l1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| initValue | EP | EP | EP | EP |
| shiftIdx | 0 | 0 | 0 | 0 |

Table 82 – Specification of initValue and shiftIdx for ctxIdx of mvp_l0_flag and mvp_l1_flag

| Initialization variable | ctxIdx of mvp_l0_flag and mvp_l1_flag |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 |
| initValue | EP | EP | EP | EP | EP | EP |
| shiftIdx | 0 | 0 | 0 | 0 | 0 | 0 |

Table 83 – Specification of initValue and shiftIdx for ctxIdx of ref_idx_l1

| Initialization variable | ctxIdx of ref_idx_l1 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| initValue | EP | EP | EP | EP |
| shiftIdx | 0 | 0 | 0 | 0 |

Table 84 – Specification of initValue and shiftIdx for ctxIdx of mvp_l1_flag

| Initialization variable | ctxIdx of mvp_l1_flag |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| initValue | EP | EP | EP | EP |
| shiftIdx | 0 | 0 | 0 | 0 |

Simulation results

The proposed method 1 is implemented on top of VTM-7.0. The simulation results under CTC are shown in Table 1.

Table 1. Results of method 1 relative to VTM-7.0

|  | Random Access Main |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.09% | -0.02% | 99% | 99% |
| Class A2 | -0.03% | -0.04% | 0.00% | 99% | 98% |
| Class B | 0.00% | 0.07% | -0.03% | 100% | 99% |
| Class C | 0.01% | -0.06% | 0.01% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.01% | 100% | 99% |
| Class D | 0.00% | 0.02% | -0.10% | 100% | 100% |
| Class F | -0.02% | -0.03% | -0.04% | 100% | 99% |
|  | Low delay B Main |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.02% | -0.38% | -0.14% | 100% | 99% |
| Class E | -0.11% | 0.02% | -0.70% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.15% | -0.06% | 0.06% | 100% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusion

To solve the mismatch between text and software, two methods are proposed. It is recommend to adopt one of the methods into VTM or VVC.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).