| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0370-v1 |
| --- | --- |

| Title: | On context model for merge indices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen, Haitao Yang | Tel: Email: | chenhuanbang@huawei.com haitao.yang@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

- Abstract

This contribution reports a mismatch between VTM-7.0 and VVC working draft 7 related to the coding of merge indices for regular merge mode and triangle merge mode. In the VTM-7.0, the first bin of merge_idx, the first bin of merge_triangle_idx0, and the first bin of merge_triangle_idx1 are coded with a single context model. While in WD7, these bins are coded with separate context models.

Firstly, the specification aligned with software is provided in this proposal. Following the spirt of sharing context for the merge indices of different merge modes using the regular merge candidate list, it is proposed to use the same context for the coding mmvd_cand_flag, the first bin of merge_idx, the first bin of merge_triangle_idx0, and the first bin of merge_triangle_idx1. Simulation results for this method reportedly show that the luma BD-rate 0.00 % and -0.01% in RA and LB cases, respectively, when compared to VTM-7.0.

In addition, the method for aligning the software to spec is tested. Simulation results for this method reportedly show that the luma BD-rate 0.00% and 0.XX% in RA and LB cases, respectively, when compared to VTM-7.0.

Introduction

A mismatch between VTM-7.0 and VVC WD 7 and is founded. The first bin of merge_idx, the first bin of merge_triangle_idx0, and the first bin of merge_triangle_idx1 are coded with a joint context model in the software. But in WD7, separate context models are used. The current text shows as follows:

| Table 48 – Association of ctxIdx and syntax elements for each initializationType in the initialization process |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax structure | Syntax element | ctxTable | initType |  |  |  |
|  |  |  | 0 | 1 | 2 |  |
| merge_data( ) | mmvd_cand_flag[ ][ ] | Table 100 |  | 0 | 1 |  |
|  | merge_triangle_idx0[ ][ ] | Table 105 |  | 1 | 2 |  |
|  | merge_triangle_idx1[ ][ ] | Table 105 |  | 1 | 2 |  |
|  | merge_idx[ ][ ] | Table 105 | 0 | 1 | 2 |  |

Table 105 – Specification of initValue and shiftIdx for ctxInc of merge_idx, merge_triangle_idx0, and merge_triangle_idx1.

| Initialization variable | ctxIdx of merge_idx, merge_triangle_idx0, merge_triangle_idx1 |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | EP | EP | EP |
| shiftIdx | 0 | 0 | 0 |

Proposed methods

Text is aligned to SW

In this case, there is no change of the coding performance. The proposed specification is as follows:

| Table 48 – Association of ctxIdx and syntax elements for each initializationType in the initialization process |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax structure | Syntax element | ctxTable | initType |  |  |  |
|  |  |  | 0 | 1 | 2 |  |
| merge_data( ) | mmvd_cand_flag[ ][ ] | Table 100 |  | 0 | 1 |  |
|  | merge_triangle_idx0[ ][ ] | Table 105 |  | 1 | 2 |  |
|  | merge_triangle_idx1[ ][ ] | Table 105 |  | 1 | 2 |  |
|  | merge_idx[ ][ ] merge_triangle_idx0[ ][ ] merge_triangle_idx1[ ][ ] | Table 105 | 0 | 1 | 2 |  |

Share context for all modes using the regular merge candidate list

Since the same merge candidate list is used for regular merge mode, MMVD mode and triangle merge mode, the second method is proposed to share the context model for the first bin of merge_idx, the first bin of merge_triangle_idx0, the first bin of merge_triangle_idx1 and mmvd_cand_flag. The proposed specification is as follows:

| Table 48 – Association of ctxIdx and syntax elements for each initializationType in the initialization process |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Syntax structure | Syntax element | ctxTable | initType |  |  |  |
|  |  |  | 0 | 1 | 2 |  |
| merge_data( ) | mmvd_cand_flag[ ][ ] | Table 100 |  | 0 | 1 |  |
|  | merge_triangle_idx0[ ][ ] | Table 105 |  | 1 | 2 |  |
|  | merge_triangle_idx1[ ][ ] | Table 105 |  | 1 | 2 |  |
|  | merge_idx[ ][ ] merge_triangle_idx0[ ][ ] merge_triangle_idx1[ ][ ] mmvd_cand_flag[ ][ ] | Table 105 | 0 | 1 | 2 |  |

Table 100 – Specification of initValue and shiftIdx for ctxInc of mmvd_cand_flag

| Initialization variable | ctxIdx of mmvd_cand_flag |  |
| --- | --- | --- |
|  | 0 | 1 |
| initValue | EP | EP |
| shiftIdx | 0 | 0 |

Table 105 – Specification of initValue and shiftIdx for ctxInc of merge_idx, merge_triangle_idx0, and merge_triangle_idx1, and mmvd_cand_flag.

| Initialization variable | ctxIdx of merge_idx, merge_triangle_idx0, merge_triangle_idx1, mmvd_cand_flag |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| initValue | EP | EP | EP |
| shiftIdx | 0 | 0 | 0 |

SW is aligned to text

To be aligned with text, 2 contexts are added to the software for merge_triangle_idx0 and merge_triangle_idx1. The initialization value of these two contexts are same as merge_idx.

Simulation results

The proposed methods are implemented on top of VTM-7.0. The simulation results under CTC are shown in Table 1 and Table 2.

Table 1. Results of method 2 relative to VTM-7.0

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | -0.02% | 98% | 98% |
| Class A2 | -0.01% | 0.01% | 0.02% | 98% | 99% |
| Class B | 0.00% | 0.05% | 0.13% | 98% | 95% |
| Class C | 0.00% | 0.00% | -0.04% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.03% | 99% | 97% |
| Class D | -0.03% | -0.11% | -0.15% | 100% | 99% |
| Class F | -0.02% | 0.00% | 0.03% | 99% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.20% | -0.08% | 99% | 99% |
| Class C | 0.02% | -0.20% | -0.21% | 99% | 95% |
| Class E | -0.03% | 0.12% | -0.31% | 99% | 97% |
| Overall | -0.01% | -0.12% | -0.18% | 99% | 97% |
| Class D | -0.05% | 0.33% | -0.01% | 100% | 100% |
| Class F | -0.02% | -0.38% | 0.11% | 99% | 98% |

Table 2. Results of method 3 relative to VTM-7.0

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | -0.02% | 98% | 98% |
| Class A2 | -0.02% | 0.04% | 0.02% | 98% | 98% |
| Class B | 0.02% | 0.08% | 0.08% | 99% | 96% |
| Class C | -0.02% | 0.02% | -0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.03% | 0.02% | 99% | 97% |
| Class D | 0.01% | -0.04% | -0.07% | 100% | 100% |
| Class F | -0.01% | 0.00% | -0.01% | 100% | 99% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | -0.27% | 0.10% | 99% | 95% |
| Class E | 0.04% | 0.07% | -0.33% | 99% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.01% | -0.03% | -0.33% | 100% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusion

To solve the mismatch between text and software for the coding of merge indices, three methods are proposed. It is recommend to adopt one of the methods into VVC.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).