| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0376_v2 |
| --- | --- |

| Title: | AhG9: Picture header refinement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Laroche Nael Ouedraogo Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 guillaume.laroche@crf.canon.fr |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution proposes a modification of the picture header signaling to avoid extra signaling of some picture header parameters not needed when the whole picture contains only one slice type (I, P, B). In particular, the parameters related to delta QP signaling for Inter and Intra are merged into single parameters. The override flag for partitioning parameters is changed into 2 override flags: one for inter and one for intra slices. Moreover, an override flag is added for motion information parameters in inter slices.

It is reported that the first proposed modifications obtains the average YUV BDR results of -0.06% -0.33% for respectively RA and LD configurations compared to VTM-7.1 for the common test conditions. Additional results with higher QP settings are reported. Average YUV BDR results of -0.48% -2.13% for this additional configuration are obtained for respectively RA and LD configurations.

# Problem Statement

In the current Picture header syntax, some parameters are dedicated to Intra or Inter slices only. These parameters are transmitted even if they are not needed and never used in the slice data. This creates additional bitrate, which is not needed.

The syntax elements related to intra slices are:

- pic_log2_diff_min_qt_min_cb_intra_slice_luma

- pic_max_mtt_hierarchy_depth_intra_slice_luma

- pic_log2_diff_max_bt_min_qt_intra_slice_luma

- pic_log2_diff_max_tt_min_qt_intra_slice_luma

- pic_log2_diff_min_qt_min_cb_intra_slice_chroma

- pic_max_mtt_hierarchy_depth_intra_slice_chroma

- pic_log2_diff_max_bt_min_qt_intra_slice_chroma

- pic_log2_diff_max_tt_min_qt_intra_slice_chroma

- pic_cu_qp_delta_subdiv_intra_slice

- pic_cu_chroma_qp_offset_subdiv_intra_slice

The syntax elements related to inter slices are:

- pic_log2_diff_min_qt_min_cb_inter_slice

- pic_max_mtt_hierarchy_depth_inter_slice

- pic_log2_diff_max_bt_min_qt_inter_slice

- pic_log2_diff_max_tt_min_qt_inter_slice

- pic_cu_qp_delta_subdiv_inter_slice

- pic_cu_chroma_qp_offset_subdiv_inter_slice

- pic_temporal_mvp_enabled_flag

- mvd_l1_zero_flag

- pic_six_minus_max_num_merge_cand

- pic_five_minus_max_num_subblock_merge_cand

- pic_fpel_mmvd_enabled_flag

- pic_disable_bdof_flag

- pic_disable_dmvr_flag

- pic_disable_prof_flag

- pic_max_num_merge_cand_minus_max_num_triangle_cand

# Propose modifications

All modifications were inserted on top of JVET-Q0041-v1_DraftText and attached to this contribution.

## Merge Delta QP parameters

To avoid duplicate signaling of delta QP parameters when only one type is used in the current picture, the parameters pic_cu_qp_delta_subdiv_intra_slice and pic_cu_qp_delta_subdiv_inter_slice are merged into one single parameter pic_cu_qp_delta_subdiv. In the same way, pic_cu_chroma_qp_offset_subdiv_intra_slice and pic_cu_chroma_qp_offset_subdiv_inter_slice are merged into one single parameter pic_cu_chroma_qp_offset_subdiv.

With this modification, when only intra or only inter slices are present in the current picture, 2 unused parameters are not transmitted if the cu delta QP is enabled for Luma and Chroma.

When both intra and inter slices are used in the current picture, only one value is available for Luma and one for Chroma. Yet, this has low impact. Indeed, these parameters gives the maximum block size where the CU QP delta is signaled. The impact of these parameters is manly for intra slices where the amount of residual is larger than inter slices and the sizes of the selected blocks are smaller. For inter slices, the sizes of the selected blocks are larger and the inter slices contain generally less residual so less delta QP. Moreover, the inter residual is often on small block sizes. Consequently the impact of these parameters values is small for Inter and having the same parameters for both inter and intra slices in the same picture is not critical in term of coding efficiency.

| if( cu_qp_delta_enabled_flag ) { |  |
| --- | --- |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| } |  |

## Intra/Inter override flags for partitioning parameters

To avoid the extra signaling of partitioning parameters, the partition_constraints_override_flag is split into 2 override flags, one for inter partitioning parameters "partition_constraints_override_flag_inter" and one for intra partitioning parameters "partition_constraints_override_flag_intre".

Thanks to this override flag for intra, the intra partitionning parameters are not transmitted for all pictures containing only inter slices as with the current CTCs.

| if( partition_constraints_override_enabled_flag ) { |  |
| --- | --- |
| partition_constraints_override_flag_inter |  |
| if( partition_constraints_override_flag_inter ) { |  |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| } |  |
| partition_constraints_override_flag_intra | u(1) |
| if( partition_constraints_override_flag_intra ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |

## Override flag for motion information

To avoid the extra signaling of motion information for pictures containing only Intra slices, an override flag is added before all motion information dedicated to Inter only slices.

| motion_parameters _override_flag | u(1) |
| --- | --- |
| if( motion_parameters _override_flag ) { |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| pic_temporal_mvp_enabled_flag | u(1) |
| if( !pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| pic_five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| pic_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| pic_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| pic_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| pic_disable_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } |  |
| if ( sps_ibc_enabled_flag ) |  |
| pic_six_minus_max_num_ibc_merge_cand | ue(v) |

# Experimental Results

# The three proposed modifications are implemented on top of VTM-7.1which includes the picture header signaling. Only the modifications 2 and 3 have an impact on the common test conditions. Table 1 gives the results for the four configurations. The 2 proposed modifications gives, in average YUV, 0.0%, -0.06%, -0.33%, -0.32% for respectively, AI, RA, LB and LP configurations. The average gain is not negligible for both low delay configurations. The gain reaches -1.14% for the sequence Johnny (Class E).

Table 1  Results of proposed Changes compared to VTM-7.1 (CTCs)

|  |  |  | All Intra Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.1_ANCHOR_VTM |  |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 103% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 108% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 110% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 104% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 121% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 105% |
|  | Random access Main10 |  |  |  |  |  |  |
|  | Over VTM-7.1_ANCHOR_VTM |  |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.02% | -0.02% | -0.02% | -0.02% | 101% | 97% |
| Class A2 | -0.02% | -0.02% | -0.02% | -0.02% | 101% | 99% |
| Class B | -0.06% | -0.06% | -0.06% | -0.06% | 99% | 102% |
| Class C | -0.13% | -0.13% | -0.13% | -0.13% | 100% | 109% |
| Class E |  |  |  |  |  |  |
| Overall | -0.06% | -0.06% | -0.06% | -0.06% | 100% | 102% |
| Class D | -0.35% | -0.35% | -0.35% | -0.35% | 99% | 123% |
| Class F | -0.14% | -0.14% | -0.14% | -0.14% | 100% | 115% |
|  | Low delay B Main10 |  |  |  |  |  |  |
|  | Over VTM-7.1_ANCHOR_VTM |  |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.07% | -0.07% | -0.07% | -0.07% | 100% | 100% |
| Class C | -0.16% | -0.15% | -0.15% | -0.16% | 101% | 100% |
| Class E | -0.78% | -0.74% | -0.73% | -0.77% | 100% | 101% |
| Overall | -0.28% | -0.26% | -0.26% | -0.33% | 100% | 101% |
| Class D | -0.46% | -0.44% | -0.44% | -0.45% | 99% | 106% |
| Class F | -0.39% | -0.36% | -0.36% | -0.38% | 100% | 100% |
|  | Low delay P Main10 |  |  |  |  |  |  |
|  | Over VTM-7.1_ANCHOR_VTM |  |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.07% | -0.07% | -0.07% | -0.07% | 100% | 101% |
| Class C | -0.15% | -0.15% | -0.15% | -0.15% | 101% | 99% |
| Class E | -0.76% | -0.72% | -0.72% | -0.75% | 98% | 99% |
| Overall | -0.27% | -0.26% | -0.26% | -0.32% | 100% | 100% |
| Class D | -0.44% | -0.42% | -0.43% | -0.44% | 104% | 101% |
| Class F | -0.38% | -0.35% | -0.35% | -0.38% | 108% | 102% |

Table 2 gives the results of the proposed modifications for higher QP than those used in the CTCs: 37, 42, 47, 52.

The average gains largely increase for respectively, AI, RA, LB and LP configurations.

Table 2 Results of proposed Changes compared to VTM-7.1 QPs: 37, 42, 47, 52.

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.1_ANCHOR_VTM |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 96% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 98% | 103% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 97% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 98% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 98% | 110% |
| Class F | -0.01% | -0.01% | -0.01% | -0.01% | 100% | 95% |
|  | Random access Main10 |  |  |  |  |  |
|  | Over VTM-7.1_ANCHOR_VTM |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -0.15% | -0.16% | -0.16% | -0.15% | 100% | 100% |
| Class A2 | -0.16% | -0.17% | -0.17% | -0.16% | 100% | 100% |
| Class B | -0.50% | -0.55% | -0.55% | -0.51% | 95% | 101% |
| Class C | -1.07% | -1.20% | -1.18% | -1.09% | 98% | 104% |
| Class E |  |  |  |  |  |  |
| Overall | -0.51% | -0.57% | -0.56% | -0.48% | 98% | 102% |
| Class D | -2.50% | -2.76% | -2.76% | -2.55% | 98% | 156% |
| Class F | -0.69% | -0.74% | -0.74% | -0.70% | 100% | 104% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | Over VTM-7.1_ANCHOR_VTM |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.77% | -0.83% | -0.87% | -0.79% | 97% | 101% |
| Class C | -1.54% | -1.70% | -1.70% | -1.56% | 99% | 96% |
| Class E | -4.00% | -4.30% | -4.29% | -4.05% | 95% | 95% |
| Overall | -1.83% | -1.99% | -2.00% | -2.13% | 97% | 98% |
| Class D | -3.98% | -4.41% | -4.50% | -4.06% | 98% | 100% |
| Class F | -1.57% | -1.69% | -1.63% | -1.59% | 100% | 99% |
|  | Low delay P Main10 |  |  |  |  |  |
|  | Over VTM-7.1_ANCHOR_VTM |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.76% | -0.83% | -0.82% | -0.77% | 97% | 101% |
| Class C | -1.51% | -1.67% | -1.67% | -1.53% | 99% | 100% |
| Class E | -3.93% | -4.23% | -4.22% | -3.98% | 95% | 100% |
| Overall | -1.80% | -1.96% | -1.95% | -2.10% | 97% | 100% |
| Class D | -3.93% | -4.25% | -4.37% | -4.00% | 100% | 108% |
| Class F | -1.56% | -1.64% | -1.60% | -1.57% | 103% | 103% |

# Conclusion

In this contribution, a modification of the picture header is proposed in order to avoid extra signaling of the syntax elements not used in the slice data when only inter and intra slices are used in the picture. The proposed modification adds 2 overrides flags and removes two syntax elements. These modifications offer almost similar flexibility compared to the current design, yet it increases the coding efficiency.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).