| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0400 |
| --- | --- |

| Title: | AHG9: On extra picture header bits in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karsten Suehring Yago Sanchez Robert Skupin Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to add reserved extra bits to the picture header similar to extra slice header bits in HEVC. The proposal, however, differs from the HEVC design in that the presence of the flags in the picture header is controlled by the sequence parameter set.

# Introduction

AVC and HEVC have had some syntax elements in the slice_header() or even for AVC in the NAL unit header (nal_ref_idc) that indicate some properties of a slice that do not affect to the decoding process but are useful for systems, non-mainstream use-cases or later codec extensions. Such a signalling may include properties such as:

- Non-reference picture (within a layer)

- Discardable: non-referenced at all, not even from other layers

- Cross-layer BLA (HEVC)

The current VVC draft contains currently only a single flag in the picture header of this type that indicates that the current picture is not used as a reference picture. It does not differentiate between layers or sub-layers, i.e. it is a global indication. In previous meetings there were tedious discussions about the usefulness of a sub layer specific indication versus the current global indication and vice-versa.

In fact, there are numerous scenarios, each with the specifics signalling needs and therefore the authors consider an extensible mechanism as that of HEVC desirable.

Some signalling that could be considered useful to be signalled with these flags is listed in the following:

- sub_layer_non_reference: as in HEVC

- layer_non_reference: not referenced within a layer

- global_non_reference: not referenced at all as in the current VVC draft

More options may become relevant with future meeting decisions and developments in systems standards, like for instance (but currently not proposed):

- refreshed_region: picture contains data of a refreshed region in a GDR period

- clean_area_used_for_tmvp_only: non-refreshed areas in a GDR period not used for TMVP

- indications that are currently specified by external means only

- etc.

# Proposal

The proposal in this document is not to add all possible described flags in the previous section but to add a mechanism that allows for extensibility where some bits can be added to the picture header. Also, it is asserted that adding the discussed flags at a certain position in the picture header (first come first serve as in HEVC) irrespective of whether they have a meaning for the application in use does not seem to be a proper solution. Therefore, the proposal is to indicate the length of the extra bits and their meaning in the SPS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| […] |  |
| num_extra_picture_header_bits_bytes | u(4) |
| extra_picture_header_bits_struct() |  |
| […] |  |
| rbsp_trailing_bits( ) |  |
| } |  |

The meaning of the extra bits is defined in the following struct, which indicates the presence of the flags within the extra bits.

| extra_picture_header_bits_struct ( ) { | Descriptor |
| --- | --- |
| global_non_reference_flag_present_flag | u(1) |
| layer_non_reference_flag_present_flag | u(1) |
| sub_layer_non_reference_flag_present_flag | u(1) |
| for (i = 3; i < (num_extra_picture_header_bits_bytes * 8) ; i++) |  |
| reserved_extra_zero_bit[ i ] | u(1) |
| } |  |

The variable numExtraPHBits is derived as follows:

i = 0
if (global._non_reference_flag_present_flag = =1)
	i++
if (layer_non_reference_flag_present_flag = =1)
	i++
if (sub_layer_non_reference_flag_present_flag = =1) 	(X-X)
	i++
for (i = 3; i < ( num_extra_picture_header_bits_bytes * 8) ; i++ )
	if (reserved_extra_zero_bit[ i ] = = 1)
		i++
numExtraPHBits = i

| picture_header( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < numExtraPHBits; i++ ) |  |
| extra_bit[ i ] | u(1) |
| gdr_pic_flag | u(1) |
| […] |  |

The variables globalNonReferenceFlag, layerNonReferenceFlag and subLayerNonReferenceFlag are derived as follows:

i = 0
if (global_non_reference_flag_present_flag = =1)
	gloablNonReferenceFlag = extra_bit[i++]
if (layer_non_reference_flag_present_flag = =1)	 (X-X)
	layerNonReferenceFlag = extra_bit [i++] 
if (sub_layer_non_reference_flag_present_flag = =1)
	subLayerNonReferenceFlag = extra_bit [i++]

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)